| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0193_r1 |
| --- | --- |

| Title: | AHG5: Unification of processing order of RDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai, Shan Liu, and Shawmin Lei MediaTek USA 2860 Junction Ave San Jose, CA 95134, USA | Tel: Email: | polin.lai@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Currently in the Range Extension reference software, lossy RDPCM methods for Intra and Inter blocks have different processing orders with respect to transform-skip and residue rotation. Furthermore, the Range Extension text specification only captures one of the processing orders, namely the one for lossy-RDPCM Intra. This document presents methods to unify the processing order of lossy RDPCM in Intra and Inter cases, in particular when residue rotation is involved. Simulation results and modifications to the text specification are provided.

# Introduction

In the 14th JCTVC meeting, several coding tools were adopted together for Range Extension of HEVC. In particular, the tool residue differential pulse-code modulation (RDPCM) [1,2,3] and residue rotation (RR) [4] both work under the scenario when transforms are bypassed, such as for lossless transform-quantization bypass blocks or for lossy transform-skip (TS) blocks. Currently in the Range Extension reference software HM-12.0+RExt-4.1, lossy RDPCM for Intra and Inter blocks have different processing orders with respect to transform-skip and residue rotation. Below is the highlight of the corresponding encoding and decoding processes for the residue signal:

- Intra (though not IntraBC) block with transform-skip:

  - Encoder: 	RDPCM  TS(left-shift)  RR  Quantization

  - Decoder:	De-quantization  RR  inv-TS(right-shift)  inv-RDPCM

- Inter and IntraBC blocks with transform-skip:

  - Encoder: 	TS (left-shift)  RR  RDPCM  Quantization

  - Decoder: 	De-quantization  inv-RDPCM  RR  inv-TS(right-shift)

The gray highlighting, such as TS  RR and RR  inv-TS, indicates that the processes TS(inv-TS) are bundled together with RR. Software-wise, RR is carried out within the TS and inv-TS function. Spec-wise, RR is evoked under the sections specifying the TS and inv-TS decoding processes.

The decoding process inconsistency demonstrated above increases implementation difficulty, thus is considered undesirable. Furthermore, the current Range Extension text specification [5] only captures one of the processes, namely the processing order of lossy-RDPCM Intra. This document presents methods to unify the processing order of lossy RDPCM in Intra and Inter cases, in particular when residue rotation is involved. Simulation results and modifications to the text specification are provided.

# Proposed unification methods

## Method 1, unify lossy decoding process as: 
De-quantization RR  inv-TS(right-shift)  inv-RDPCM

In this first method, the encoding and decoding processes of lossy-Inter-RDPCM are modified to align with the corresponding processes for lossy-Intra-RDPCM. In this manner, at decoder side, inv-RDPCM will always be performed after RR and inv-TS(right-shift). With this unification, the residue decoding process in the current text specification, though didn’t follow the original spirit of the adopted lossy RDPCM Inter [3], can be kept.

In fact, this unification method makes inv-RDPCM always the last stage in the decoding process, regardless of lossy or lossless, as summarized below:

Unified decoding process after applying Method 1 to lossy-RDPCM Inter:

- Lossless-RDPCM: 	RR  inv-RDPCM

- Lossy-RDPCM: 	De-quantization  RR  inv-TS(right-shift)  inv-RDPCM

The corresponding results using test conditions specified in JCTVC-N1123 [6] are provided below.

Table 1 Test results of Method 1: De-quantization RR  inv-TS(right-shift)  inv-RDPCM

## Method 2, unify lossy decoding process as:
De-quantization RR inv-RDPCM inv-TS(right-shift)

In the second unification method, the decoding process is modified from Method 1 to move the inverse-TS after inverse-RDPCM, such that the inverse-RDPCM process will operate on higher precision and then being right-shifted.

Unified decoding process after applying Method 2 to lossy-RDPCM Inter and Intra:

- Lossless-RDPCM: 	RR  inv-RDPCM

- Lossy-RDPCM: 	De-quantization  RR  inv-RDPCM  inv-TS(right-shift)

The corresponding results using test conditions specified in JCTVC-N1123 [6] are provided below.

Table 2 Test results of Method 2: De-quantization RR  inv-RDPC  inv-TS(right-shift)

8.6.2 Scaling and transformation process

……

To be Added (w.r.t current spec? or to the fixed spec??)

# Conclusions

This document presents methods to unify the processing order of lossy RDPCM in Intra and Inter cases, in particular when residue rotation is involved. Simulation results and modifications to the text specification are provided. Two unification methods are proposed, summarized as below:

Method 1decoding process:

- Lossless-RDPCM: 	RR  inv-RDPCM

- Lossy-RDPCM: 	De-quantization  RR  inv-TS(right-shift)  inv-RDPCM

Method 2 decoding process:

- Lossless-RDPCM: 	RR  inv-RDPCM

- Lossy-RDPCM: 	De-quantization  RR  inv-RDPCM  inv-TS(right-shift)

# References

[1] “RCE2: Experimental results on Test 3 and Test 4, ” JCTVC-M0056, Apr. 2013

[2] “RCE2 subtest C.2: Extension of residual DPCM to lossy coding,” JCTVC-N0052, Jul. 2013

[3] “RCE2: Experimental results for Test C.1,” JCTVC-N0074, Jul. 2013

[4] “RCE2 Test B.1: Residue rotation and significance map context,” JCTVC-N0044, Jul. 2013

[5] “Range Extensions Draft 4,” JCTVC-N1005, Jul. 2013

[6] “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques”, JCTVC-N1023, Jul. 2013

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).