| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0194 |
| --- | --- |

| Title: | SCE4: Test 5.1 results on bit-depth and color-gamut scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alireza Aminlou, Kemal Ugur, Miska M. Hannuksela Visiokatu 3, Tampere, Finland Elena Alshina, Alexander Alshin 416, Maetan 3-dong, Yeongtong-gu, Suwon, Korea, 443-742 | Tel: Email: Tel: Email: | +90 536 567 2997 kemal.ugur@nokia.com +82 10 3026 1305 elena_a.alshina@samsung.com |
| Source: | Nokia, Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

SCE4 tests several tools for bit-depth and color-gamut scalability. Test 5.1 proposes to perform upsampling and bit-depth increase operation with a single operation, rather than using two cascaded steps. Experimental results show that the proposed method provides 0.78% and 0.26% coding efficiency gains for all-intra and random access results respectively. Additional tests are performed with weighted prediction enabled both for anchor and the proposal and in that case, proposed method brings 1.42% and 0.49% gain.

# Introduction & Proposal

In order to support bit-depth scalability SHM3.0.1 s/w was modified specifically for SCE4. Internal bit-depth for each layer is separately specified in configuration file and signalled to the decoder in SPS. Re-sampling process is followed by bit-depth increment. So in this implementation, bit-depth scalability introduces additional processing step.

The proposal is illustrated in Figure 1 where the bit-depth scaling process is merged with re-sampling. It results in modification of the de-scaling shift after second step of upsampling only. The goal of proposed modification is to avoid extra interlayer processing step and to minimize accuracy loss.

# Experimental results

Specific test sequences provide by Technicolor were used in SCE4. In tests described below base layer is BT709 8 bits 1920×1080 sequence, while enhancement layer is BT2020 10 bits 3840 × 2160 sequences. In addition to the spatial resolution, both bit-depth and color gamut are different between layers.

Several results are provided to test the performance of the proposed method.

## Weighted Prediction disabled

In this test, neither anchor nor the proposal enables weighted prediction (WP).

|  | AI_2x |  |  |  | RA_2x |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |
| SCE4, Test 5.1 | -0.78% | -0.87% | -0.97% |  | -0.26% | -0.07% | 0.00% |

## Weighted Prediction enabled

In this test, weighted prediction is enabled for inter-layer reference pictures for the anchor and the proposal. Weight and offsets are calculated using the same adaptive method in HM. The proposed method brings on average 1.42% and 0.49% for all-intra and random access respectively, when WP is used.

|  | AI_2x |  |  |  | RA_2x |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |
| Proposal + WP | -6.60% | -5.51% | -9.46% |  | -3.45% | -0.95% | -4.41% |
| Anchor + WP | -5.27% | -4.14% | -8.64% |  | -2.97% | -0.66% | -4.31% |

The performance of weighted prediction could be improved by some simple non-normative tricks. One such method is tested here where a fixed weight is used for inter-layer reference pictures in L1 and adaptive weight for L0. In this case, the proposed method brings on average 1.42% and 0.50% for all-intra and random access respectively, when this method of WP is used.

|  | AI_2x |  |  |  | RA_2x |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y | U | V |
| Proposal + WP (Adaptive + fixed) | -6.60% | -5.51% | -9.46% |  | -3.75% | -1.34% | -4.73% |
| Anchor + WP (Adaptive + fixed) | -5.27% | -4.14% | -8.64% |  | -3.26% | -1.08% | -4.57% |

# Specification text

The proposal is implemented to specification text as follows.

- Luma sample interpolation process

Inputs to this process are

–	the luma reference sample array rlPicSampleL,

–	a luma sample location ( xP, yP ) relative to the top-left luma sample of the current picture.

–	bit-depth of enhancement-layer luma samples, nEnhY.

Output of this process is a interpolated luma sample value intLumaSample.

- Error: Reference source not found specifies the 8-tap filter coefficients fL[ p, x ] with p = 0 ... 15 and x = 0 ... 7 used for the luma resampling process.

Table H-1 – 16-phase luma resampling filter

| phase p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p, 0 ] | fL[ p, 1 ] | fL[ p, 2 ] | fL[ p, 3 ] | fL[ p, 4 ] | fL[ p, 5 ] | fL[ p, 6 ] | fL[ p, 7 ] |
| 0 | 0 | 0 | 0 | 64 | 0 | 0 | 0 | 0 |
| 1 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 2 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 3 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 4 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 5 | −1 | 4 | -11 | 52 | 26 | -8 | 3 | -1 |
| 6 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 7 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 8 | −1 | 4 | -11 | 40 | 40 | -11 | 4 | -1 |
| 9 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 10 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 11 | −1 | 3 | -8 | 26 | 52 | -11 | 4 | -1 |
| 12 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 13 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 14 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |
| 15 | n/a | n/a | n/a | n/a | n/a | n/a | n/a | n/a |

The value of the interpolated luma sample IntLumaSample  is derived by applying the following ordered steps:

- The derivation process for reference layer sample location used in resampling as specified in subclause Error: Reference source not found is invoked with cIdx equal to 0 and luma sample location ( xP, yP ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample  as output.

- The variables xRef and xPhase are derived as follows:

xRef     = ( xRef16 >> 4 )	(H-1)

xPhase = ( xRef16 ) % 16	(H-2)

- The variables yRef and yPhase are derived as follows

yRef     = ( yRef16 >> 4 )	(H-3)

yPhase = ( yRef16 ) % 16	(H-4)

- The variables shift1, shift2 and offset are derived as follows:

shift1 = BitDepthY – 8	(H-5)

shift2 = 12 – (nEnhY – BitDepthY)	(H-6)

offset =1 << ( shift2 – 1). 	(H-7)

- The sample value tempArray[ n ] with n = 0 … 7, is derived as follows.

yPosRL = Clip3( 0, RefLayerPicHeightInSamplesL – 1, yRef + n – 1 ) 	(H-8)

refW      = RefLayerPicWidthInSamplesL

tempArray[n] = ( fL[ xPhase, 0 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef – 3), yPosRL ] +
	fL[ xPhase, 1 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef – 2), yPosRL ] +
	fL[ xPhase, 2 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef – 1), yPosRL ] +
	fL[ xPhase, 3 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef      ), yPosRL ] +
	fL[ xPhase, 4 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 1), yPosRL ] +	(H-9)
	fL[ xPhase, 5 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 2), yPosRL ] +
	fL[ xPhase, 6 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 3), yPosRL ] +
	fL[ xPhase, 7 ] * rlPicSampleL[ Clip3( 0, refW– 1, xRef + 4), yPosRL ] ) >> shift1

- The interpolated luma sample value intLumaSample is derived as follows.

intLumaSample = ( fL[ yPhase, 0 ] * tempArray [ 0 ] +	
	fL[ yPhase, 1 ] * tempArray [ 1 ] +	
	fL[ yPhase, 2 ] * tempArray [ 2 ] +	
	fL[ yPhase, 3 ] * tempArray [ 3 ] +	
	fL[ yPhase, 4 ] * tempArray [ 4 ] +	(H-10)
	fL[ yPhase, 5 ] * tempArray [ 5 ] +	
	fL[ yPhase, 6 ] * tempArray [ 6 ] +	
	fL[ yPhase, 7 ] * tempArray [ 7 ] + offset ) >> shift2

intLumaSample = Clip3( 0, ( 1 << BitDepthY) – 1 , intLumaSample )	(H-11)

- Chroma sample interpolation process

Inputs to this process are:

–	the chroma reference sample array rlPicSampleC,

–	a chroma sample location ( xPC, yPC ) relative to the top-left chorma sample of the current picture.

–	bit-depth of enhancement-layer chroma samples, nEnhC.

Output of this process is a interpolated chroma sample value intChromaSample.

- Table H-2 specifies the 4-tap filter coefficients fC[ p, x ] with p = 0 ... 15 and x = 0 ... 3 used for the chroma resampling process.

Table H-2 – 16-phase chroma resampling filter

| phase p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p, 0 ] | fC[ p, 1 ] | fC[ p, 2 ] | fC[ p, 3 ] |
| 0 | 0 | 64 | 0 | 0 |
| 1 | n/a | n/a | n/a | n/a |
| 2 | n/a | n/a | n/a | n/a |
| 3 | n/a | n/a | n/a | n/a |
| 4 | −4 | 54 | 16 | −2 |
| 5 | −6 | 52 | 20 | −2 |
| 6 | −6 | 46 | 28 | −4 |
| 7 | n/a | n/a | n/a | n/a |
| 8 | −4 | 36 | 36 | −4 |
| 9 | −4 | 30 | 42 | −4 |
| 10 | n/a | n/a | n/a | n/a |
| 11 | −2 | 20 | 52 | −6 |
| 12 | n/a | n/a | n/a | n/a |
| 13 | n/a | n/a | n/a | n/a |
| 14 | −2 | 10 | 58 | −2 |
| 15 | 0 | 4 | 62 | −2 |

The value of the interpolated chroma sample value intChromaSample is derived by applying the following ordered steps:

- The derivation process for reference layer sample location in resampling as specified in subclause Error: Reference source not foundError: Reference source not found is invoked with cIdx and chroma sample location ( xPC, yPC ) given as the inputs and ( xRef16, yRef16 ) in units of 1/16-th sample as output.

- The variables xRef and xPhase are derived by

xRef     = ( xRef16 >> 4 )	(H-12)

xPhase = ( xRef16 ) % 16	(H-13)

- The variables yRef and yPhase are derived by

yRef     = ( yRef16 >> 4 )	(H-14)

yPhase = ( yRef16 ) % 16	(H-15)

- The variables shift1, shift2 and offset are derived as follows:

shift1 = BitDepthC – 8	(H-16)

shift2 = 12 – (nEnhC – BitDepthC)	(H-17)

offset =1 << ( shift2 – 1) 	(H-18)

- The sample value tempArray[ n ] with n = 0 … 3, is derived as follows.

yPosRL = Clip3( 0 , RefLayerPicHeightInSamplesC – 1, yRef + n – 1 )	(H-19)

refWC   = RefLayerPicWidthInSamplesC	(H-20)

tempArray[n] = ( fC[ xPhase, 0 ] * rlPicSampleC[ Clip3( 0, refWC – 1, xRef – 1), yPosRL ] +
	fC[ xPhase, 1 ] * rlPicSampleC[ Clip3( 0, refWC – 1, xRef      ), yPosRL ] +
	fC[ xPhase, 2 ] * rlPicSampleC[ Clip3( 0, refWC – 1, xRef + 1), yPosRL ] +	(H-21)
	fC[ xPhase, 3 ] * rlPicSampleC[ Clip3( 0, refWC – 1, xRef + 2), yPosRL ]  ) >> shift1

- The interpolated chroma sample value intChromaSample is derived as follows.

intChromaSample = (fC[ yPhase, 0 ] * tempArray [ 0 ] +
	fC[ yPhase, 1 ] * tempArray [ 1 ] +
	fC[ yPhase, 2 ] * tempArray [ 2 ] +	(H-22)
	fC[ yPhase, 3 ] * tempArray [ 3 ] + offset ) >> shift2

intChromaSample = Clip3( 0, ( 1 << BitDepthC ) – 1 , intChromaSample )	(H-23)

# Conclusion

Nokia and Samsung Electronics suggest adding bit-depth scalability support to the next release of SHM s/w and working draft text. The usage of joint re-sampling and bit-depth conversion provides 0,8% (AI) and 0,3% (RA) Luma BD-rate gain and eliminates an extra processing step.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).