| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0195 |
| --- | --- |

| Title: | Non-SCE4: Picture and region adaptive gain-offset prediction for color space scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 Cheung.Auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution presents the results of adding picture and region adaptation to the gain-offset prediction from JCVC-L0224 for color gamut scalable video coding. The coefficients of the piecewise linear predictor is updated at the enhancement layer P picture and applied to the enhancement P picture and the B pictures following the P picture before the next Picture in encoding order. It is reported that compared with the SCE4 anchor, for 10-bit base layer all intra test case, the proposed method resulted in an average BD rate of -7.7%, -6.2%, -9.8% for Y, U, V, respectively. For 8-bit base layer all intra test case, the proposed method resulted in an average BD rate of -6.9%, -5.6%, -10.1% for Y, U, V, respectively. For 10-bit base layer random-access test case, the proposed method resulted in an average BD rate of -3.3%, -0.9%, -4.2% for Y, U, V, respectively. For 8-bit base layer random access test case, the proposed method resulted in an average BD rate of -3.0%, -0.4%, -3.7% for Y, U, V, respectively.

# Introduction

The gain-offset predictor was proposed in JCTVC-L0334 [1], it was included into SCE4 test 5.2 study [2]. The parameters of the gain-offset predictor in JCTVC-L0334 were computed once from the first picture and applied to all pictures in the video sequence. The parameters were computed by minimizing the sum of square difference between the original enhancement layer input and the up-sampled base layer reference.

Let b[c][x][y] be a pixel in the up-sampled base layer reference of color component c at location (x,y). The corresponding predicted pixel p[c][x][y] be of the enhancement layer is determined as the followings.

p[c][x][y] = (gain[c]*b[c][x][y] + offset[c] + (1<<4)) >> 5

The prediction parameters gain[c] and offset[c] are optimized for the first frame and applied to the entire sequence.

# Picture and Region Adaptive Gain-Offset Prediction

This contribution presents the results of adding region adaptation and picture adaptation to the gain-offset prediction:

- For region adaptation, each picture is divided into 1, 4, or 16 rectangular regions. Each region has its own gain and offset.

- For picture adaptation, the gain of offset is computed at every P-picture in the enhancement layer and applied to the P-picture and the B-pictures prior to the next P-picture in the encoding order. In particular, for the all intra test case, the parameters of the gain-offset parameters are updated at every picture. For the random-access test case, the parameters are updated approximately once per second.

Figure 1: A picture can be divided into 1, 4, or 16 regions and each region may have different gain-offset parameters.

Let b[c][x][y] be a pixel in the up-sampled base layer reference of color component c at location (x,y) within a region. The corresponding predicted pixel p[c][x][y] be of the enhancement layer is determined as the followings.

p[c][x][y] = (gain[c]*b[c][x][y] + offset[c] + (1<<(numBitsFraction-1))) >> numBitsFraction

where numBitsFraction is 10 in this contribution.The prediction parameters gain[c] and offset[c] in each region are optimized for the region in a P-picture and applied to the corresponding region in the P-picture and the following B-picture, if any, prior to the next P-picture.

# Test results

Table 1 reports a summary of the RD performance of the tested method with 16 regions per picture when compared with the SHM-3.0.1-CE4 anchor from TechniColor in SCE4 common test conditions. As indicated in the excel sheet, the status “Not matched” is due to different rounding from the TechniColor anchor results. Since the simulations were conducted with a heterogeneous computer cluster, timing is not accurate, and therefore, timing is not reported in Table 1.

For the 10-bit base layer all intra test case, the proposed method resulted in an average BD rate of -7.7%, -6.2%, -9.8% for Y, U, V, respectively. For the 8-bit base layer all intra test case, the proposed method resulted in an average BD rate of -6.9%, -5.6%, -10.1% for Y, U, V, respectively.

For the 10-bit base layer random-access test case, the proposed method resulted in an average BD rate of -3.3%, -0.9%, -4.2% for Y, U, V, respectively. For the 8-bit base layer random access test case, the proposed method resulted in an average BD rate of -3.0%, -0.4%, -3.7% for Y, U, V, respectively.

In summary, the proposed method is more effective for all intra test cases, and it has little or no effect on the random-asses test case.

Table 1: BD-rate of the proposed picture and region adaptive gain-offset predictor when compared with SHM-3.0.1-CE4 anchor from TechniColor.

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -7.7% | -6.2% | -9.8% | -6.9% | -5.6% | -10.1% |
| Overall (Test vs Ref) | -7.7% | -6.2% | -9.8% | -6.9% | -5.6% | -10.1% |
| Overall (Test vs single layer) | 17.0% | 21.3% | 16.2% | 20.8% | 24.6% | 18.6% |
| Overall (Ref vs single layer) | 26.8% | 29.4% | 28.9% | 29.8% | 32.1% | 32.1% |
| EL only (Test vs Ref) | -13.9% | -12.3% | -15.8% | -13.1% | -11.7% | -16.1% |
| Enc Time[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| Dec Time[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Not matched |  |  | Not matched |  |  |
|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -3.3% | -0.9% | -4.2% | -3.0% | -0.4% | -3.7% |
| Overall (Test vs Ref) | -3.3% | -0.9% | -4.2% | -3.0% | -0.4% | -3.7% |
| Overall (Test vs single layer) | 27.9% | 34.3% | 27.3% | 29.8% | 35.9% | 28.7% |
| Overall (Ref vs single layer) | 32.3% | 35.5% | 33.0% | 33.8% | 36.4% | 33.9% |
| EL only (Test vs Ref) | -5.8% | -3.2% | -6.5% | -5.4% | -2.6% | -5.9% |
| Enc Time[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| Dec Time[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Not matched |  |  | Not matched |  |  |

# Reference

- Louis Kerofsky, Andrew Segall, Seung-Hwan Kim, and Kiran Misra, “Color Gamut Scalable Video Coding: New Results”, JCTVC-L0334, 12th Meeting: Geneva, CH, 14–23 Jan. 2013.

- Andrew Segall, Philippe Bordes, Cheung Auyeung, Xiang Li, Elena Alshina, Alberto Duenas, “Description of Core Experiment SCE4: Color Gamut and Bit-Depth Scalability”, JCTVC-N1104, 14th Meeting: Vienna, Austria, AT, July 29 – Aug 2, 2013.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Specification text for the proposed (test) color gamut scalability

- F.7.3.2.3	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| use_color_prediction | u(1) |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

- F.7.3.6.1	General slice segment header syntax

| slice_segment_header( ) ( ) { | Descriptor |
| --- | --- |
| …. |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| if (first_slice_segment_in_pic_flag ){ |  |
| update_color_prediction | u(1) |
| if (update_color_prediction) { |  |
| num_bits_fraction | ue(v) |
| num_regions_log4 | ue(v) |
| numRegions = (1<<num_regions_log4)*(1<<num_regions_log4) |  |
| for (rIdx=0; rIdx<numRegions; rIdx++) |  |
| for (cIdx=0; cIdx<3; cIdx++) { |  |
| gain[rIdx][cIdx] | se(v) |
| offset[rIdx][cIdx] | se(v) |
| } |  |
| } |  |
| } |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

- H.8.1.4 		Resampling process for inter layer reference pictures

……

The following steps are applied to derive the resampled inter layer reference picture rsPic.

–	if PicWidthInSamplesL is equal to RefLayerPicWidthInSamplesL and PicHeightInSamplesL is equal to RefLayerPicHeightInSamplesL and the values of ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are all equal to 0

  - rsPic is set equal to rlPic.

–	otherwise, rsPic is derived as follows:

  - The PicOrderCntVal value of rsPic is set equal to the PicOrderCntVal value of rlPic.

When SamplePredEnabledFlag[ currLayerId ][ rLId ] is equal to 1, the picture sample resampling process as specified in subclause Error: Reference source not found is invoked with the sample arrays of reference layer picture rlPic, Luma bit-depth of base-layer samples, BaseBitDepthY, and Chroma bit-depth of base-layer samples, BaseBitDepthC, and prediction parameters, gain1, gain2, offset, breakpoint as input, and with the sample arrays of resampled picture rsPic as output.

  - When MotionPredEnabledFlag[ currLayerId ][ rLId ] is equal to 1, the picture motion field resampling process as specified in subclause Error: Reference source not found is invoked with refereence layer picture rlPic and its motion field as inputs, and with the  motion field of resampled picture rsPic as output.

H.8.1.4.1.3 Luma sample interpolation process

Inputs to this process are

–	the luma reference sample array rlPicSampleL,

–	a luma sample location ( xP, yP ) relative to the top-left luma sample of the current picture,

–	Luma bit-depth of base-layer samples, BaseBitDepthY.

–	Luma bit-depth of enhancement-layer samples, EnhBitDepthY.

- A variable cIdx specifying the color component index.

–	Enhancement-layer prediction prameters, gain1, gain2, offset, breakpoint.

…..

- The variables shift1, shift2 and offset are derived as follows:

shift1 = BaseBitDepthY – 8	(H-1)

shift2 = 20 – BaseBitDepthY 	(H-2)

offset =1 << ( shift2 – 1). 	(H-3)

….

- The interpolated luma sample value intLumaSample is derived as follows.

intLumaSample = ( fL[ yPhase, 0 ] * tempArray [ 0 ] +	
	fL[ yPhase, 1 ] * tempArray [ 1 ] +	
	fL[ yPhase, 2 ] * tempArray [ 2 ] +	
	fL[ yPhase, 3 ] * tempArray [ 3 ] +	
	fL[ yPhase, 4 ] * tempArray [ 4 ] +	(H-4)
	fL[ yPhase, 5 ] * tempArray [ 5 ] +	
	fL[ yPhase, 6 ] * tempArray [ 6 ] +	
	fL[ yPhase, 7 ] * tempArray [ 7 ] + offset ) >> shift2

intLumaSample = Clip3( 0, ( 1 << BaseBitDepthY) – 1 , intLumaSample )	(H-5)

columnWidthL = (picHeightInSamplesL + (1<<num_regions_log4) – 1) /(1<<num_regions_log4)

rowHeightL = (picWidthInSamplesL + (1<<num_regions_log4) – 1) /(1<<num_regions_log4)

rIdx = (xP/columnWidthL) + ((yP/rowHeightL) << num_regions_log4)

intLumaSample = gain[rIdx][cIdx] * (intLumasample) + offset[rIdx][cIdx]

intLumaSample = (intLumaSample + (1 << (num_bits_fraction-1))) >> num_bits_fraction

intLumaSample = Clip3( 0, ( 1 << EnhBitDepthY) – 1 , intLumaSample )

H.8.1.4.1.4 Chroma sample interpolation process

Inputs to this process are:

–	the chroma reference sample array rlPicSampleC,

–	a chroma sample location ( xPC, yPC ) relative to the top-left chorma sample of the current picture,

–	Chroma bit-depth of base-layer samples, BaseBitDepthC.

–	Chroma bit-depth of enhancement-layer samples, EnhBitDepthC.

- A variable cIdx specifying the color component index.

–	Enhancement-layer prediction prameters, gain1, gain2, offset, breakpoint.

…..

- The variables shift1, shift2 and offset are derived as follows:

shift1 = BaseBitDepthC – 8	(H-6)

shift2 = 20 – BaseBitDepthC 	(H-7)

offset =1 << ( shift2 – 1) 	(H-8)

…..

- The interpolated chroma sample value intChromaSample is derived as follows.

intChromaSample = (fC[ yPhase, 0 ] * tempArray [ 0 ] +
	fC[ yPhase, 1 ] * tempArray [ 1 ] +
	fC[ yPhase, 2 ] * tempArray [ 2 ] +	(H-9)
	fC[ yPhase, 3 ] * tempArray [ 3 ] + offset ) >> shift2

intChromaSample = Clip3( 0, ( 1 << BaseBitDepthC ) – 1 , intChromaSample )	(H-10)

columnWidthC = (picHeightInSamplesC + (1<<num_regions_log4) – 1) /(1<<num_regions_log4)

rowHeightC = (picWidthInSamplesC + (1<<num_regions_log4) – 1) /(1<<num_regions_log4)

rIdx = (xPc/columnWidthC) + ((yPc/rowHeightC) << num_regions_log4)

intChromaSample = gain[rIdx][cIdx] * (intChromaSample) + offset[rIdx][cIdx]

intChromaSample = (intChromaSample + (1 << (num_bits_fraction-1))) >> num_bits_fraction

intChromaSample = Clip3( 0, ( 1 << EnhBitDepthC) – 1 , intChromaSample )