| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0196 |
| --- | --- |

| Title: | SCE4: Results of test 5.4-model1 on piecewise linear color space predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 Cheung.Auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution contains the performance analysis of SCE4 test 5.4 model 1 on using piecewise linear predictor for color gamut scalable video coding. The coefficients of the piecewise linear predictor is designed once from the first picture of the enhancement layer input and the up-sampled based layer reference by exhaustive integer search on the breakpoints, and it is applied to the entire sequence. It is reported that compared with the SCE4 anchor, for 10-bit base layer all intra test case, the proposed method achieves an average BD rate of -4.1%, -2.8%, -6.8% for Y, U, V, respectively. For 10-bit base layer random access test case, the proposed method achieves an average BD rate of -3.1%, -0.6%, -4.7% for Y, U, V, respectively. For 8-bit base layer all intra test case, the proposed method achieves an average BD rate of -4.4%, -3.0%, -7.9% for Y, U, V, respectively. For 8-bit base layer random access test case, the proposed method achieves an average BD rate of -3.3%, -0.1%, -4.6% for Y, U, V, respectively.

# Technical description

Piecewise linear predictor was proposed in JCTVC-N0271 [1]. It was based on JCTVC-L0334 [2], but with the gain-offset predictor in JCTVC-L0334 replaced by piecewise-linear predictor. The piecewise linear predictor was included into SCE4 test 5.4 model1 study [3].

## Piecewise-linear prediction model (SCE4 test 5.4 model 1)

Let b[c][x][y] be a pixel in the up-sampled base layer reference of color component c at location (x,y). The corresponding predicted pixel p[c][x][y] be of the enhancement layer is determined as the followings.

Let d[c][x][y] = b[c][x][y] – breakpoint[c].

If d[c][x][y] <= 0

p[c][x][y] = (gain1[c]*d[c][x][y] + offset[c] + (1<<4)) >> 5

else

p[c][x][y] = (gain2[c]*d[c][x][y] + offset[c] + (1<<4)) >> 5

The prediction parameters gain1[c], gain2[c], offset[c], and breakpoint[c] are optimized for the first frame and applied to the entire sequence.

Let e[c][x][y] be the input of the enhancement layer of color component c at pixel (x,y). The prediction parameters gain1[c], gain2[c], offset[c] and breakpoint[c] are determined by exhaustive search on all integer value of breakpoint[c] which minimizes the sum of square difference of all pixels between p[c][x][y] and e[c][x][y]:

For a given breakpoint[c], the minima of the above equation can be obtained by solving a set of three linear equations. The prediction parameters are obtained by exhaustive search on all integer value of breakpoint[c] which minimizes the above prediction error.

# Test results

A summary of the RD performance of the tested method compared with the SHM-3.0.1-CE4 anchor from TechniColor in SCE4 common test conditions is reported in Table 1. As indicated in the excel sheet, the status “Not matched” is due to different rounding from the TechniColor anchor results. Since the simulations were conducted with a heterogeneous computer cluster, timing is not accurate, and therefore, timing is not reported in Table 1.

For 10-bit base layer all intra test case, the proposed method achieves an average BD rate of -4.1%, -2.8%, -6.8% for Y, U, V, respectively. For 10-bit base layer random access test case, the proposed method achieves an average BD rate of -3.1%, -0.6%, -4.7% for Y, U, V, respectively.

For 8-bit base layer all intra test case, the proposed method achieves an average BD rate of -4.4%, -3.0%, -7.9% for Y, U, V, respectively. For 8-bit base layer random access test case, the proposed method achieves an average BD rate of -3.3%, -0.1%, -4.6% for Y, U, V, respectively.

Table 1: BD-rate of SCE4 Test 5.4 Model1 when compared with SHM-3.0.1-CE4 anchor from TechniColor.

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -4.1% | -2.8% | -6.8% | -4.4% | -3.0% | -7.9% |
| Overall (Test vs Ref) | -4.1% | -2.8% | -6.8% | -4.4% | -3.0% | -7.9% |
| Overall (Test vs single layer) | 21.7% | 25.9% | 20.2% | 24.2% | 28.2% | 21.6% |
| Overall (Ref vs single layer) | 26.8% | 29.4% | 28.9% | 29.8% | 32.1% | 32.1% |
| EL only (Test vs Ref) | -7.2% | -5.8% | -9.8% | -8.5% | -6.9% | -11.8% |
| Enc Time[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| Dec Time[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Not matched |  |  | Not matched |  |  |
|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | -3.1% | -0.6% | -4.7% | -3.3% | -0.1% | -4.6% |
| Overall (Test vs Ref) | -3.1% | -0.6% | -4.7% | -3.3% | -0.1% | -4.6% |
| Overall (Test vs single layer) | 28.2% | 34.8% | 26.6% | 29.5% | 36.3% | 27.6% |
| Overall (Ref vs single layer) | 32.3% | 35.5% | 33.0% | 33.8% | 36.4% | 33.9% |
| EL only (Test vs Ref) | -5.4% | -2.6% | -6.8% | -5.9% | -2.5% | -7.0% |
| Enc Time[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| Dec Time[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| Enc Mem[%] | #DIV/0! |  |  | #DIV/0! |  |  |
| BL Match | Not matched |  |  | Not matched |  |  |

# Reference

- Cheung Auyeung and Kazushi Sato, “Color gamut scalable video coding with piecewise linear predictions and shift-offset model”, JCTVC-N0271, 14th Meeting: Vienna, Austria, AT, July 29 – Aug 2, 2013.

- Louis Kerofsky, Andrew Segall, Seung-Hwan Kim, and Kiran Misra, “Color Gamut Scalable Video Coding: New Results”, JCTVC-L0334, 12th Meeting: Geneva, CH, 14–23 Jan. 2013.

- Andrew Segall, Philippe Bordes, Cheung Auyeung, Xiang Li, Elena Alshina, Alberto Duenas, “Description of Core Experiment SCE4: Color Gamut and Bit-Depth Scalability”, JCTVC-N1104, 14th Meeting: Vienna, Austria, AT, July 29 – Aug 2, 2013.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Specification text for the proposed (test) color gamut scalability

- F.7.3.2.3	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| use_color_prediction | u(1) |
| if (use_color_prediction) { |  |
| for (cIdx=0; cIdx<3; cIdx++) |  |
| gain1[cIdx] | se(v) |
| gain2[cIdx] | se(v) |
| offset[cIdx] | se(v) |
| breakpoint[cIdx] | se(v) |
| } |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

- H.8.1.4 		Resampling process for inter layer reference pictures

……

The following steps are applied to derive the resampled inter layer reference picture rsPic.

–	if PicWidthInSamplesL is equal to RefLayerPicWidthInSamplesL and PicHeightInSamplesL is equal to RefLayerPicHeightInSamplesL and the values of ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are all equal to 0

  - rsPic is set equal to rlPic.

–	otherwise, rsPic is derived as follows:

  - The PicOrderCntVal value of rsPic is set equal to the PicOrderCntVal value of rlPic.

When SamplePredEnabledFlag[ currLayerId ][ rLId ] is equal to 1, the picture sample resampling process as specified in subclause Error: Reference source not found is invoked with the sample arrays of reference layer picture rlPic, Luma bit-depth of base-layer samples, BaseBitDepthY, and Chroma bit-depth of base-layer samples, BaseBitDepthC, and prediction parameters, gain1, gain2, offset, breakpoint as input, and with the sample arrays of resampled picture rsPic as output.

  - When MotionPredEnabledFlag[ currLayerId ][ rLId ] is equal to 1, the picture motion field resampling process as specified in subclause Error: Reference source not found is invoked with refereence layer picture rlPic and its motion field as inputs, and with the  motion field of resampled picture rsPic as output.

H.8.1.4.1.3 Luma sample interpolation process

Inputs to this process are

–	the luma reference sample array rlPicSampleL,

–	a luma sample location ( xP, yP ) relative to the top-left luma sample of the current picture,

–	Luma bit-depth of base-layer samples, BaseBitDepthY.

–	Luma bit-depth of enhancement-layer samples, EnhBitDepthY.

- A variable cIdx specifying the color component index.

–	Enhancement-layer prediction prameters, gain1, gain2, offset, breakpoint.

…..

- The variables shift1, shift2 and offset are derived as follows:

shift1 = BaseBitDepthY – 8	(H-1)

shift2 = 20 – BaseBitDepthY 	(H-2)

offset =1 << ( shift2 – 1). 	(H-3)

….

- The interpolated luma sample value intLumaSample is derived as follows.

intLumaSample = ( fL[ yPhase, 0 ] * tempArray [ 0 ] +	
	fL[ yPhase, 1 ] * tempArray [ 1 ] +	
	fL[ yPhase, 2 ] * tempArray [ 2 ] +	
	fL[ yPhase, 3 ] * tempArray [ 3 ] +	
	fL[ yPhase, 4 ] * tempArray [ 4 ] +	(H-4)
	fL[ yPhase, 5 ] * tempArray [ 5 ] +	
	fL[ yPhase, 6 ] * tempArray [ 6 ] +	
	fL[ yPhase, 7 ] * tempArray [ 7 ] + offset ) >> shift2

intLumaSample = Clip3( 0, ( 1 << BaseBitDepthY) – 1 , intLumaSample )	(H-5)

LumaGain = (intLumaSample>breakpoint[cIdx] ? gain2[cIdx] : gain1[cIdx]) 	(H-31a)

intLumaSample = LumaGain * (intLumasample – breakpoint[cIdx]) + offset[cIdx]	(H31-b)

intLumaSample = (intLumaSample + (1 << 4)) >> 5	(H31-b)

intLumaSample = Clip3( 0, ( 1 << EnhBitDepthY) – 1 , intLumaSample ) 	(H31-d)

H.8.1.4.1.4 Chroma sample interpolation process

Inputs to this process are:

–	the chroma reference sample array rlPicSampleC,

–	a chroma sample location ( xPC, yPC ) relative to the top-left chorma sample of the current picture,

–	Chroma bit-depth of base-layer samples, BaseBitDepthC.

–	Chroma bit-depth of enhancement-layer samples, EnhBitDepthC.

- A variable cIdx specifying the color component index.

–	Enhancement-layer prediction prameters, gain1, gain2, offset, breakpoint.

…..

- The variables shift1, shift2 and offset are derived as follows:

shift1 = BaseBitDepthC – 8	(H-6)

shift2 = 20 – BaseBitDepthC 	(H-7)

offset =1 << ( shift2 – 1) 	(H-8)

…..

- The interpolated chroma sample value intChromaSample is derived as follows.

intChromaSample = (fC[ yPhase, 0 ] * tempArray [ 0 ] +
	fC[ yPhase, 1 ] * tempArray [ 1 ] +
	fC[ yPhase, 2 ] * tempArray [ 2 ] +	(H-9)
	fC[ yPhase, 3 ] * tempArray [ 3 ] + offset ) >> shift2

intChromaSample = Clip3( 0, ( 1 << BaseBitDepthC ) – 1 , intChromaSample )	(H-10)

ChromaGain = (intChromaSample >breakpoint[cIdx] ? gain2[cIdx] : gain1[cIdx]) 	(H-43a)

intChromaSample = ChromaGain * (intChromaSample – breakpoint[cIdx]) + offset[cIdx]	(H43-b)

intChromaSample = (intChromaSample + (1 << 4)) >> 5	(H43-c)

intChromaSample = Clip3( 0, ( 1 << EnhBitDepthC) – 1 , intChromaSample ) 	(H43-d)