| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0197 |
| --- | --- |

| Title: | HLS: Non-significant tile set for tiled streaming with single layer HEVC extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 Cheung.Auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

The temporal motion constrained tile sets SEI message in JCTVC-N0383 was adopted for HEVC range extension text specification. This SEI message can be used to facilitate tiled streaming where different clients may request the server to send only the requested regions of interest to the clients with minimal overhead after the video is encoded. This contribution proposes to modify the temporal motion constrained tile sets SEI message to signal to the client that the client decoder does not need to decode and display the pixels outside the regions of interest so that the area outside the regions of interest can be coded efficiently to reduce network bandwidth and to reduce decoder computation and memory requirements.

Figure 1: An example of tiled streaming.

# Introduction

Tiled streaming can be used to provide individualized viewing experience to multiple viewers at the region, resolution, and time that each viewer want as in the followings.

As shown in Error: Reference source not found, a panorama video is encoded with independently decodable tiles and in multiple resolution layers in the server side to provide different clients with different region of interest and resolution at different time.

For example in Figure 2, in one instance, a director, whom may be one of the clients, may request a region of interest in one of the resolution layer as in Figure 2a. In another instance, the director may request to move the region of interest to a different location in the current resolution layer for panning as in Figure 2b or to another resolution layer for zooming as in Figure 2c. Based on the requests of the director, the server send different tile sets containing the region of interest to the clients from one of the resolution layer. In general, there may be many directors conducting the video for serving many clients in parallel with different regions of interest, resolution, and timing.

Figure 2: A video is stored in the server with multiple resolution layers. (a) A region of interest is in one of the resolution layer. (b) In panning, the region of interest moves to a new location in the same layer. (c) In zooming, the region of interest moves to a different resolution layer.

Figure 3: Director requested tiles are extracted and send to the clients by re-encoding the non-significant tiles with minimal number of bits to form a valid bitstream.  It is not necessary for a client to decode and display the pixels of the non-significant tiles to reduce computation and memory usage.

Tiled streaming can be facilitated by using the temporal motion constrained tile sets SEI message in JCTVC-N0383 adopted for HEVC range extension text specification. The temporal motion constrained tile sets SEI message can be used to signal the regions of interest requested by a director for tiled streaming. When the mc_all_tiles_exact_sample_value_match_flag is equal to one, all tiles are independently decodable, and any tiles can be collected to form a motion constrained tile sets for any clients. In particular, in response to a director, the server can signal the region of interest in a motion constrained tile set and send the region of interest to the clients.

Since the director and the clients may be different, a client may not know the intent of the director on how to process the pixels outside the regions of interest. Even in the case that the director and the user are the same, the server needed to send acknowledgement to the client on how to process the pixels outside the regions of interest. This contribution proposes to add a non-significant tile set to the temporal motion constrained tile sets SEI message to provide the following functionality:

- Provide a signal to the decoder that the decoder is not required to decode and store the pixels in the non-significant tile sets to reduce computation and memory usage.

- As shown inFigure 3, the server may re-encode the non-significant tiles with a minimal number of bits to form a valid bitstream and to save transmission bandwidth while keeping the slice segment headers intact for decoding the regions of interest.

# Proposed Syntax and Semantics

This contribution proposes to add a non-significant tile set, highlighted in yellow, to the temporal motion constrained tile sets SEI.

| temporal_motion _constrained_tile_sets ( payloadSize ) { | Descriptor |
| --- | --- |
| mc_all_tiles_exact_sample_value_match_flag | u(1) |
| each_tile_one_tile_set_flag | u(1) |
| if (!each_tile_one_tile_set_flag) { |  |
| num_sets_in_message_minus1 | ue(v) |
| if(num_sets_in_message_minus1) |  |
| non_signficant_tile_set_present_flag | u(1) |
| numSignificantSets = num_sets_in_message_minus1– non_signficant_tile_set_present_flag +1 |  |
| for( i = 0; i <= numSignificantSets; i++) { |  |
| mcts_id[ i ] | ue(v) |
| num_tile_rects_in_set_minus1[ i | ue(v) |
| for( j = 0; j <= num_tile_rects_in_set_minus1[ i ]; j++) { |  |
| iop_left_tile_index[ i ][ j ] | ue(v) |
| bottom_right_tile_index[ i ][ j ] | ue(v) |
| } |  |
| if ( ! mc_all_tiles_exact_sample_value_match_flag ) |  |
| exact_sample_value_match_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

non_significant_tile_set_present_flag equal to 1 specifies that the last tile set consists of those tiles that are not included in any earlier tile sets in the same message. The last tile sets are not required to provide decoded pixels. non_significant_tile_set_present_flag equal to 0 indicates that the last tile set may or may not consist of those tiles that are not included in any earlier tile sets in the same message.

# Patent rights declaration(s)

Sony may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).