| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0199 m30969 |
| --- | --- |

| Title: | SHVC skip pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Jack Enhorn | Tel: Email: | +46761263591 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution expresses support for the skip_picture_flag from contribution JCTVC-N0209 with the addition of handling special cases. In the method proposed for skip pictures in JCTVC-N0209, motion compensation is performed by applying the re-sampled motion field from the closest reference layer to the corresponding temporal reference pictures in the current layer’s DPB. This contribution agrees with the assessed advantage in using motion information from another layer while keeping fidelity (details) from the current layer. In addition to what is proposed in JCTVC-N0209, this contribution proposes the following:

- Use re-sampled sample values from the inter-layer reference picture when the picture used for inter prediction in the inter-layer reference picture has no corresponding picture in the enhancement layer or when motion vector prediction is not used from the inter-layer reference picture.

- Use inter prediction with zero motion from the first picture in L0 (and L1 in case of a B slice) when no inter-layer reference picture is available.

- Bitstream restriction to prohibit the use of skip picture when neither inter-layer reference pictures nor temporal reference pictures are available.

- Allow deblocking filter to be applied to skip pictures.

In the second version of the document, syntax and semantics for a flag is added as a new first section in accordance with what was suggested when the contribution was first presented.

In the third version of the document, an additional constraint is added to higher_layer_irap_skip_flag in order to ensure that slice segment syntax does not need to be parsed.

The fourth version of the document contains more explicit constraints for higher_layer_irap_skip_flag.

# VUI flag for indicating skip

| vps_vui( ){ | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < NumDirectRefLayers[ layer_id_in_nuh[ i ] ]; j++ ) { |  |
| tile_boundaries_aligned_flag[ i ][ j ] | u(1) |
| single_layer_for_non_irap_flag | u(1) |
| higher_layer_irap_skip_flag | u(1) |
| ilp_restricted_ref_layers_flag | u(1) |
| … |  |
| } |  |

single_layer_for_non_irap_flag equal to 1 indicates either that all the VCL NAL units of an access unit have the same nuh_layer_id value or that two nuh_layer_id values are used by the VCL NAL units of an access unit and the picture with the greater nuh_layer_id value is an IRAP picture. single_layer_for_non_irap_flag equal to 0 indicates that nuh_layer_id values may or may not be constrained beyond constraints specified in other parts of this Recommendation | International Standard. When single_layer_for_non_irap_flag is not present it is inferred to be equal to 0.

higher_layer_irap_skip_flag equal to 1 indicates that for every IRAP picture, for which there is another picture in the same access unit with lower value of nuh_layer_id, the following applies:

For all slices of the IRAP picture:

- slice_type shall be equal to P.

- slice_sao_luma_flag and slice_sao_chroma_flag shall both be equal to 0.

- five_minus_max_num_merge_cand shall be equal to 5.

- weighted_pred_flag shall be equal to 0 in the PPS that is refered to by the slices.

For all coding units of the IRAP picture:

- cu_skip_flag[ i ][ j ] shall be equal to 1.

When single_layer_for_non_irap_flag is equal to 0, higher_layer_irap_skip_flag shall be equal to 0. When higher_layer_irap_skip_flag is not present it is inferred to be equal to 0.

NOTE X – An encoder may set both single_layer_for_non_irap_flag and higher_layer_irap_skip_flag equal to 1 as an indication to a decoder that whenever there are two pictures in the same access unit, the one with the higher nuh_layer_id is an IRAP picture for which the decoded samples can be derived by only applying the resampling process for inter layer reference pictures specified in subclause H.8.1.4 with the other picture as input.

# Problem Statement

The skip_picture_flag syntax element was presented to indicate skipped pictures in SHVC enhancement layer [1]. The main idea is to, when a picture is signaled with skip_picture_flag equal to 1, scale the motion field from the closest reference layer and apply the motion on the samples of a temporal reference picture. For intra coded blocks the interpolated pixel values of the closest layer are used.

This feature would reduce bitrate and decoding complexity of skipped pictures. However, the proposal required both a corresponding picture in another layer (same POC) and a previous picture available for prediction of the same layer.

It would be favourable to add additional functionality to handle these special cases:

- Where the skipped enhancement layer picture does not have a previous picture in the same layer available for reference.

- Where the skipped enhancement layer picture does not have a corresponding base layer picture.

When using SHVC for Adaptive Resolution Change (ARC) where typically only one layer is used at a time these situations could occur. The first case is visualized in Figure 1 and the second case in Figure 2 with the “missing” pictures marked with red.

Figure 1 Skipped picture without base layer picture of same POC

Also for ARC at a layer up-switch it could be beneficial to encode one of the pictures as skip to reduce coding complexity and required bits. This since typically only one of the pictures would be used for output but both are required to not restrict the IRAP in the enhancement layer to be intra coded only.

Figure 2 ARC layer switch with enhancement layer picture skipped.

A use case of skipped picture could be to delay the up-switch one picture by setting the pic_output_flag to zero. This would emulate a “diagonal” prediction from a base layer picture of lower POC.

Figure 3 ARC layer switch with one picture delay.

Problems could also occur if the base layer and enhancement layer uses different prediction structures or different frame rates. To illustrate, in Figure 4 the enhancement layer is configured with a lower frame rate. It is not possible to directly apply the motion vector field from the base layer since the enhancement layer picture uses a reference picture from another time instance. Simply rescaling the motion vectors could result in severe visual artifacts.

Figure 4 Lower frame rate in enhancement layer.

# Proposed skip behaviour

### Skipped picture without available enhancement layer pictures for prediction

To evaluate a legal reference picture to use it is proposed to require:

- That there is a reference picture at the same index in the same list in the enhancement layer picture as in the base layer picture.

- That the POC of the picture at that reference index is the same in both the base layer picture and the enhancement layer picture.

If the reference is not legal, it is proposed that the samples are up-sampled from the picture in the inter layer reference picture just as is done for intra predicted blocks. The same method is used if motion vector prediction is not activated (e.g. temporal motion vector prediction is turned off).

### Skipped picture without corresponding base layer picture

It is proposed that the picture data is copied from the first reference picture in reference picture list 0 for a P-frame. A B-frame is proposed to use the first picture in both reference picture lists for bi-prediction with zero motion vectors.

### No available temporal or inter-layer reference picture

It is proposed that a bitstream conformance requirement specifies that skip_picture_flag must be equal to zero in this case.

### Deblocking enabled

We propose to enable deblocking in skip pictures in order to reduce blockiness when motion vectors from the base layer are applied in the enhancement layer. An encoder will of course still have the option of turning off deblocking for a picture.

# Proposed text changes

The proposed changes to the specification text are attached to this contribution in a separate file.

# References

[1] JCTVC- “SHVC skipped picture indication” J. Boyce, X. Xiu, Y. He, Y. Ye July 2013

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).