| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0201 |
| --- | --- |

| Title: | SCE4 Test 5.2: Color prediction with Gain-Offset model |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | proposal |  |  |
| Author(s) or | Jie Zhao Deshpande Sachin Misra Kiran Seung-Hwan Kim Andrew Segall | Email: | jzhao@sharplabs.com sdespande@sharplabs.com misrak@sharplabs.com kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This document provides the results of Core Experiment 4 (SCE4) test 5.2 on Color Gamut and Bit-Depth Scalability in SHVC. The color prediction process is described by a gain and offset model. In our test, the gain and offset are designed from the first frame of the sequence and applied to the entire sequence.  Compared to SCE4 anchor, for 8-bit base layer all intra test case, the proposed method achieves an average BD rate of -4.6%, -3.3%, -7.3% for Y, U, V, respectively. For 8-bit base layer random access test case, the proposed method achieves an average BD rate of -2.9%, -0.2%, -3.7% for Y, U, V, respectively. For 10-bit base layer all intra test case, the proposed method achieves an average BD rate of -4.0%, -2.3%, -6.0% for Y, U, V, respectively. For 10-bit base layer random access test case, the proposed method achieves an average BD rate of -xx%, -xx%, -xx% for Y, U, V, respectively. [Note: 10-bit base result will be updated once they are available]

# Introduction

The upcoming deployment of UHDTV devices and content will use a different color gamut than legacy devices.  Specifically, HD uses the BT.709 recommendation [1], while UHDTV will use the BT.2020 recommendation [2].  A key difference between these systems is that the color gamut of UHDTV is significantly larger than HD.  It is asserted that will provide a more “life like” viewing experience, which is consistent with other UHDTV characteristics, such as high resolution.

The diagram of a general scalable encoder including a prediction tool for color and bit-depth differences between the base and enhancement layers is shown in Figure 1.  The key features illustrated are that the inputs for the base and enhancement layers are represented in different color gamuts, and that the enhancement layer encoder is able to use reconstructed frame from the decoder during encoding the enhancement layer.

Figure 1 Color Space Scalable Coder

# Proposed method

The color prediction process is described by a gain and offset model. The model operates on individual color planes.  To facilitate integer calculation, a parameter describes the number of fraction bits used in the calculation in the parameter numFractionBits.  For each channel a gain[c] and offset[c] are specified.  The prediction is defined by

Pred[c][x][y] =  (gain[c]*In[x][y] + (1<<(numFractionBits-1))>> numFractionBits + offset[c]

The bit-depth scaling process is the same as proposed in [4] [5].

In our test, the gain and offset are designed at encoder side from the first frame of the sequence according to the upsampled base layer picture and the enhancement layer picture. The gain and offset are transmitted in PPS and applied to the entire sequence. The signalling is the same as described in [4]. Alternatively, the gain and offset may be updated more frequently and adaptively signalled in bitstream.

# Experimental results

Table 1 shows the summary RD performance of the tested method compared with the SHM-3.0.1-CE4 anchor in SCE4 common test conditions. Our excel sheet shows that base layer is not matched, it is due to different rounding from the TechniColor anchor results.

For 8-bit base layer all intra test case, the proposed method achieves an average BD rate of -4.6%, -3.3%, -7.3% for Y, U, V, respectively. For 8-bit base layer random access test case, the proposed method achieves an average BD rate of -2.9%, -0.2%, -3.7% for Y, U, V, respectively.

As shown in the table, For 10-bit base layer all intra test case, the proposed method achieves an average BD rate of -4.0%, -2.3%, -6.0% for Y, U, V, respectively. For 10-bit base layer random access test case, the proposed method achieves an average BD rate of -xx%, -xx%, -xx% for Y, U, V, respectively.

Table 1. Summary result of SCE4 test 5.2 Color prediction with gain offset vs. SCE4 anchor

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A+ | -4.0% | -2.3% | -6.0% | -4.6% | -3.3% | -7.3% |
| Overall (Test vs Ref) | -4.0% | -2.3% | -6.0% | -4.6% | -3.3% | -7.3% |
| Overall (Test vs single layer) | 21.8% | 26.6% | 21.2% | 23.9% | 27.8% | 22.4% |
| Overall (Ref vs single layer) | 26.8% | 29.4% | 28.9% | 29.8% | 32.1% | 32.1% |
| EL only (Test vs Ref) | -7.6% | -5.8% | -9.4% | -9.0% | -7.6% | -11.5% |
| Overall (Test EL+BL vs single EL+BL) | -19.9% | -16.5% | -20.2% | -18.5% | -16.0% | -19.7% |
|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |
|  | Y | U | V | Y | U | V |
| Class A+ | 0.0% | 0.0% | 0.0% | -2.9% | -0.2% | -3.7% |
| Overall (Test vs Ref) | 0.0% | 0.0% | 0.0% | -2.9% | -0.2% | -3.7% |
| Overall (Test vs single layer) | 32.3% | 35.5% | 33.0% | 29.9% | 36.2% | 28.8% |
| Overall (Ref vs single layer) | 32.3% | 35.5% | 33.0% | 33.8% | 36.4% | 33.9% |
| EL only (Test vs Ref) | 0.0% | 0.0% | 0.0% | -5.2% | -2.3% | -5.8% |
| Overall (Test EL+BL vs single EL+BL) | -11.7% | -8.4% | -9.5% | -13.6% | -8.5% | -13.2% |

# Conclusions

A scalable system for support different color gamuts and bit-depths in different layers of a bit-stream is proposed.  This addresses the issue of efficiently supporting a UHDTV and HD device in the same bit-stream.  Low complexity Gain Offset prediction models were examined and show that for 8 bit base layer, the proposed tool achieves 4.6% BD rate reduction for all intra test cases and 2.9% rate reduction for random access case; for 10 bit base layer, the proposed tool achieves 4.0% BD rate reduction for all intra test cases and xx% rate reduction for random access case.

We propose to include the color prediction tool and signalling in SHVC and MV-HEVC.

# References

- ITU-R Recommendation BT.709 “Parameter values for the HDTV standards for production and international programme exchange” Dec. 2010.

- ITU-R Recommendation BT.2020  “Parameter values for UHDTV systems for production and international programme exchange” April 2012.

- L. Kerofsky, A. Segall, S. Kim, K. Misra, “Color Gamut Scalable Video Coding”,JCTVC-K-0241, Shanghai, October 2012.

- L. Kerofsky, A. Segall, S. Kim, K. Misra, “Color Gamut Scalable Video Coding: New Results”, JCTVC-L-0334, Geneva, January 2013.

- S.Deshpande, L.Kerofsky, A.Segall, “On Color Gamut Scalable Video Coding,” JCTVC-M0214, April 2013, Incheon, KR.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).