| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCTVC-O0202_v3 M30974 |
| --- | --- |

| Title: | RCE1: Descriptions and Results for Experiments 1, 2, 3, and 4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu Woo-Shik Kim Jianle Chen Joel Sole Marta Karczewicz | Email: | wpu@qti.qualcomm.com wooshikk@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this document the description of RCE1 experiments are given along with their results. In the proposed methods, the chroma residual signal is predicted using the scaled luma residual signal. The scaling factor is signaled for each TU. In case of intra coding, the method is allowed only when DM mode is used as chroma prediction mode. It is reported that the average BD-rate gain achieved by the proposed method (Experiment 2) is 1.4%, 6.6%, 6.9% for Y, U, V, respectively, for YUV 4:4:4 and 18.1%, 16.6%, 17.3% for G, B, R, respectively, for RGB, in case of All Intra Main-tier, 0.5%, 8.2%, 7.2% for YUV 4:4:4 and 14.1%, 11.7%, 13.5% for RGB in case of Random Access Main-tier, and 0.2%, 5.8%, 5.4% for YUV 4:4:4 and 12.6%, 8.7%, 10.1% for RGB in case of Low Delay B Main-tier.

Version 3 of this document provided refined spec text changes in its Section 6.

# Introduction

The correlation among different color components can be reduced by the method in JCTVC-N0266 [1] presented at the 14th JCT-VC meeting resulting in increased compression performance. RCE1 was formed to study the performance and the complexity associated with the proposal. Four experiments are included in RCE1, where the first experiment is the method proposed in JCTVC-N0266, and the others are variations which were discussed during the 14th JCT-VC meeting and included in the RCE1 description [2].

While Experiment 1 achieves good coding efficiency improvement, Experiment 2 shows similar performance with reduced implementation complexity as it can facilitate parallel processing. In Experiment 3, PU based signaling is tested for inter-coded blocks, but it does not improve the performance compared to Experiment 1 and 2. Experiment 4 is the combination of Experiment 2 and Experiment 3.

# Description of RCE1 experiments

## RCE1 Experiment 1

An adaptive inter color component residual prediction scheme is proposed in JCTVC-N0266 [1], where a prediction is performed between luma residual signal and chroma residual signal. The predictor is generated using simple operations like one multiplication and one bit-shift operation. Specifically, the chroma residual signal is predicted using the luma residual signal at the encoder side as:

ΔrC(x,y)=rC(x,y)−α×rL'(x,y)>>3					 (1)

and it is compensated at the decoder side as:

rC'(x,y)=ΔrC'(x,y)+α×rL'(x,y)>>3	 				(2)

where rC denotes the chroma residual sample at a position ,
rL' denotes the reconstructed residual sample of the luma component, 
ΔrC denotes the predicted signal using inter-color prediction,

ΔrC'denotes the reconstructed signal after coding and decoding ΔrC, and

rC' denotes the reconstructed chroma residual.
α is allowed to be chosen from −8, −4, −2, −1, 0, 1, 2, 4, 8, inclusively. This approach allows good utilization of the correlation between luma and chroma residual signals, even when they are negatively correlated. The absolute value of α, abs(α) is mapped using the following table, Table 1, before binarization. M(α) is binarized using truncated unary code and coded using CABAC. If α is not zero, another bin is used to code the sign of α. At the decoder side, after CABAC decoding, M(α) is inversely mapped back to abs(α).

Table 1. α Mapping Table.

| abs(α) | M(α) |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 2 |
| 4 | 3 |
| 8 | 4 |

The scaling parameter α is calculated at the encoder side and signaled. The decoder does not need to re-calculate α, so there is no additional computing operation except equation (2) and no additional memory access at the decoder side.

This prediction is performed both for intra- and inter-coded blocks. However, in case of intra coding, only those with DM chroma mode are allowed to use this prediction.

For each TU, if the luma component’s cbf is zero, α is not signaled and no prediction is performed. Otherwise, α is signaled for each chroma component, respectively.

In case of RGB sequence, G comoponent is treated as Y component in YCbCr component.

## RCE1 Experiment 2

In this experiment, at the encoder side, when calculating the α, the luma residual before scaling and transformation (i.e. the difference between original luma sample and its prediction) is used rather than the reconstructed luma residual signal as in Experiment 1. Specifically, at the encoder side instead of (1) below equation is used.

ΔrC(x,y)=rC(x,y)−α×rL(x,y)>>3					 (3)

This can reduce the implementation complexity by facilitating pipelining and parallel processing as the prediction can be performed without waiting until reconstruction of luma residual signal. This requires only encoder side modification, and the decoding part is the same as Experiment 1.

## RCE1 Experiment 3

For inter prediction, α is calculated for each PU rather than for each TU, and signaled in the first TU in each PU whose corresponding luma cbf is not equal to 0.

## RCE1 Experiment 4

This experiment is a combination of Experiment 2 and Experiment 3.

# Experimental Results

The experiments employ the test set and the test condition as described in the RExt common test conditions Error: Reference source not found for YCbCr 4:4:4 and RGB test sequences. YCbCr 4:2:2 test sequences are not tested.

The HEVC RExt reference software version 4.1 was used as the anchor for the integration of the tested methods as well as for generation of the anchor results.

## Experiment 1

In this experiment, the performance of JCTVC-N0266 is evaluated using HM 12.0 RExt-4.1. Table 2 is the summary results of Experiment 2.

Table 2. Performance of RCE1 Experiment 1 vs. HEVC RExt4.1 anchor.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -18.1% | -16.6% | -17.3% | -14.2% | -13.5% | -13.8% | -10.8% | -10.4% | -10.5% |
| YCbCr 4:4:4 | -1.4% | -6.5% | -6.7% | -1.7% | -4.4% | -6.1% | -1.9% | -3.2% | -4.4% |
| Overall | -10.3% | -11.9% | -12.4% | -8.4% | -9.3% | -10.2% | -6.7% | -7.0% | -7.6% |
|  | -10.3% | -12.0% | -12.5% | -8.4% | -9.4% | -10.4% | -6.6% | -7.1% | -7.3% |
| Enc Time[%] | 112% |  |  | 114% |  |  | 115% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -14.5% | -11.9% | -13.7% | -11.4% | -8.7% | -10.7% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -8.5% | -7.3% | -0.9% | -6.2% | -7.6% |  |  |  |
| Overall | -8.0% | -10.3% | -10.7% | -6.5% | -7.6% | -9.2% |  |  |  |
|  | -7.9% | -11.0% | -10.5% | -6.7% | -9.3% | -9.6% |  |  |  |
| Enc Time[%] | 113% |  |  | 118% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -13.6% | -9.2% | -10.8% | -10.4% | -7.0% | -8.5% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -6.1% | -5.8% | -0.7% | -4.2% | -6.4% |  |  |  |
| Overall | -7.3% | -7.8% | -8.5% | -5.9% | -5.7% | -7.5% |  |  |  |
|  | -7.2% | -6.7% | -8.0% | -6.1% | -6.5% | -7.5% |  |  |  |
| Enc Time[%] | 106% |  |  | 108% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |  |  |  |

## Experiment 2

Table 3 is the summary results of Experiment 2.

Table 3. Performance of RCE1 Experiment 2 vs. HEVC RExt4.1 anchor.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -18.1% | -16.6% | -17.3% | -14.2% | -13.5% | -13.8% | -10.8% | -10.4% | -10.5% |
| YCbCr 4:4:4 | -1.4% | -6.6% | -6.9% | -1.8% | -4.4% | -6.2% | -1.9% | -3.2% | -4.4% |
| Overall | -10.3% | -11.9% | -12.4% | -8.4% | -9.3% | -10.3% | -6.7% | -7.1% | -7.7% |
|  | -10.3% | -12.0% | -12.5% | -8.4% | -9.4% | -10.4% | -6.7% | -7.2% | -7.3% |
| Enc Time[%] | 116% |  |  | 118% |  |  | 119% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  | 103% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -14.1% | -11.7% | -13.5% | -11.1% | -8.6% | -10.5% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -8.2% | -7.2% | -0.9% | -6.0% | -7.5% |  |  |  |
| Overall | -7.8% | -10.1% | -10.6% | -6.3% | -7.4% | -9.1% |  |  |  |
|  | -7.7% | -10.7% | -10.4% | -6.5% | -9.5% | -9.4% |  |  |  |
| Enc Time[%] | 111% |  |  | 114% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -12.6% | -8.7% | -10.1% | -9.8% | -6.5% | -7.9% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -5.8% | -5.4% | -0.6% | -3.9% | -6.0% |  |  |  |
| Overall | -6.8% | -7.3% | -7.9% | -5.5% | -5.3% | -7.0% |  |  |  |
|  | -6.7% | -6.5% | -7.6% | -5.6% | -6.0% | -7.0% |  |  |  |
| Enc Time[%] | 110% |  |  | 112% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |  |  |  |

## Experiment 3

Table 4 is the summary results of Experiment 3.

Table 4. Performance of RCE1 Experiment 3 vs. HEVC RExt4.1 anchor.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -18.1% | -16.6% | -17.3% | -14.2% | -13.5% | -13.8% | -10.8% | -10.4% | -10.5% |
| YCbCr 4:4:4 | -1.4% | -6.5% | -6.7% | -1.7% | -4.4% | -6.1% | -1.9% | -3.2% | -4.4% |
| Overall | -10.3% | -11.9% | -12.4% | -8.4% | -9.3% | -10.2% | -6.7% | -7.0% | -7.6% |
|  | -10.3% | -12.0% | -12.5% | -8.4% | -9.4% | -10.4% | -6.6% | -7.1% | -7.3% |
| Enc Time[%] | 127% |  |  | 130% |  |  | 130% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -14.2% | -11.8% | -13.6% | -11.2% | -8.7% | -10.5% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -8.3% | -7.2% | -0.8% | -6.1% | -7.3% |  |  |  |
| Overall | -7.8% | -10.2% | -10.6% | -6.4% | -7.5% | -9.0% |  |  |  |
|  | -7.8% | -11.0% | -10.4% | -6.5% | -9.4% | -9.3% |  |  |  |
| Enc Time[%] | 154% |  |  | 158% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 102% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -13.0% | -9.1% | -10.5% | -10.0% | -6.8% | -8.1% |  |  |  |
| YCbCr 4:4:4 | -0.1% | -6.2% | -5.6% | -0.5% | -4.3% | -5.9% |  |  |  |
| Overall | -7.0% | -7.7% | -8.2% | -5.6% | -5.6% | -7.1% |  |  |  |
|  | -6.9% | -7.1% | -7.8% | -5.8% | -6.3% | -7.0% |  |  |  |
| Enc Time[%] | 140% |  |  | 145% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |  |  |  |

## Experiment 4

Table 5 is the summary results of Experiment 4.

Table 5. Performance of RCE1 Experiment 4 vs. HEVC RExt4.1 anchor.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -18.1% | -16.6% | -17.3% | -14.2% | -13.5% | -13.8% | -10.8% | -10.4% | -10.5% |
| YCbCr 4:4:4 | -1.4% | -6.6% | -6.9% | -1.8% | -4.4% | -6.2% | -1.9% | -3.2% | -4.4% |
| Overall | -10.3% | -11.9% | -12.4% | -8.4% | -9.3% | -10.3% | -6.7% | -7.1% | -7.7% |
|  | -10.3% | -12.0% | -12.5% | -8.4% | -9.4% | -10.4% | -6.7% | -7.1% | -7.3% |
| Enc Time[%] | 131% |  |  | 132% |  |  | 132% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  | 103% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -13.9% | -11.8% | -13.4% | -11.0% | -8.6% | -10.4% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -8.3% | -7.1% | -0.9% | -6.1% | -7.1% |  |  |  |
| Overall | -7.7% | -10.1% | -10.5% | -6.3% | -7.4% | -8.9% |  |  |  |
|  | -7.6% | -10.8% | -10.3% | -6.3% | -9.2% | -9.2% |  |  |  |
| Enc Time[%] | 146% |  |  | 150% |  |  |  |  |  |
| Dec Time[%] | 110% |  |  | 109% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -12.3% | -8.8% | -10.0% | -9.6% | -6.5% | -7.8% |  |  |  |
| YCbCr 4:4:4 | -0.1% | -5.9% | -5.3% | -0.4% | -4.2% | -5.6% |  |  |  |
| Overall | -6.6% | -7.4% | -7.8% | -5.3% | -5.4% | -6.8% |  |  |  |
|  | -6.5% | -6.7% | -7.5% | -5.4% | -6.0% | -6.7% |  |  |  |
| Enc Time[%] | 137% |  |  | 140% |  |  |  |  |  |
| Dec Time[%] | 110% |  |  | 107% |  |  |  |  |  |

## Performance evaluation of Experiment 2 on screen contents

Additional experiments are performed to evaluate the performance of Experiment 2 on screen contents coding. AhG8 test condition is followed along with the recommended test sequences (only 4:4:4 sequences are tested). Table 6 is the summary results.

Table 6. Performance of RCE1 Experiment 2 vs. HEVC RExt4.1 anchor on screen contents.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U |  | V |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  | 0.0% |  |
| SC RGB 444 | -26.0% | -25.7% | -26.0% | -24.3% | -24.0% | -24.1% | -22.6% | -22.2% |  | -22.3% |  |
| Animation RGB 444 | -25.9% | -25.7% | -24.5% | -25.2% | -24.3% | -23.5% | -22.5% | -21.1% |  | -20.6% |  |
| SC YUV 444 | -4.5% | -8.0% | -7.9% | -4.6% | -6.4% | -7.0% | -4.2% | -5.2% |  | -5.7% |  |
| Animation YUV 444 | -2.6% | -11.5% | -7.2% | -3.1% | -9.3% | -6.7% | -3.3% | -7.0% |  | -5.7% |  |
| RangeExt | -1.7% | -5.6% | -11.1% | -2.0% | -4.4% | -9.2% | -2.2% | -3.7% |  | -6.8% |  |
| SC(444) GBR Optional | -24.2% | -24.5% | -24.8% | -24.8% | -25.0% | -25.7% | -24.9% | -25.1% |  | -25.7% |  |
| SC(444) YUV Optional | -7.2% | -8.3% | -7.3% | -6.5% | -5.8% | -6.4% | -5.5% | -4.5% |  | -5.5% |  |
| Enc Time[%] | 113% |  |  | 114% |  |  | 114% |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 101% |  |  |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |
| SC RGB 444 | -23.2% | -23.1% | -23.5% | -22.3% | -22.2% | -22.4% |  |  |  |  |  |
| Animation RGB 444 | -20.0% | -19.9% | -18.4% | -20.7% | -19.8% | -18.7% |  |  |  |  |  |
| SC YUV 444 | -3.5% | -7.8% | -7.8% | -3.7% | -6.3% | -7.0% |  |  |  |  |  |
| Animation YUV 444 | -1.0% | -11.1% | -6.3% | -1.5% | -10.3% | -6.6% |  |  |  |  |  |
| RangeExt | -0.8% | -8.5% | -11.2% | -1.0% | -5.9% | -10.7% |  |  |  |  |  |
| SC(444) GBR Optional | -23.4% | -23.5% | -23.7% | -24.4% | -24.5% | -25.0% |  |  |  |  |  |
| SC(444) YUV Optional | -6.0% | -7.9% | -6.4% | -5.7% | -6.0% | -5.8% |  |  |  |  |  |
| Enc Time[%] | 109% |  |  | 110% |  |  |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |  |  |
| SC RGB 444 | -20.9% | -20.3% | -20.8% | -20.5% | -20.0% | -20.4% |  |  |  |  |  |
| Animation RGB 444 | -18.9% | -16.9% | -15.6% | -19.4% | -17.0% | -16.0% |  |  |  |  |  |
| SC YUV 444 | -2.7% | -6.8% | -7.1% | -3.2% | -5.4% | -6.1% |  |  |  |  |  |
| Animation YUV 444 | -0.4% | -9.3% | -4.1% | -1.0% | -8.8% | -4.7% |  |  |  |  |  |
| RangeExt | -0.4% | -5.2% | -7.5% | -0.6% | -3.0% | -7.5% |  |  |  |  |  |
| SC(444) GBR Optional | -21.1% | -21.1% | -21.5% | -23.0% | -22.9% | -23.6% |  |  |  |  |  |
| SC(444) YUV Optional | -8.6% | -10.2% | -9.1% | -7.5% | -7.8% | -7.8% |  |  |  |  |  |
| Enc Time[%] | 107% |  |  | 108% |  |  |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |  |  |  |  |  |

# Conclusion

In this document the description of RCE1 experiments are given with the results. Among four experiments, Experiment 2 shows good coding efficiency improvement with reduced implementation complexity as it can facilitate parallel processing.

# References

- W. Pu, W.-S. Kim, J. Chen, K. Rapaka, L. Guo, J. Sole, and M. Karczewicz, “Non RCE1: Inter color component residual prediction,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0266, August 2013.

- W.-S. Kim and T. Nguyen, “HEVC Range Extensions Core Experiment 1 (RCE1): Inter-component decorrelation methods,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1121, August 2013.

- D. Flynn, K. Sharman, and C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1006, April 2013.

# Spec text changes

In this section the proposed draft text is given for Experiment 1 and 2 as below.

## Picture parameter set RBSP syntax (7.3.2.3) and semantics (7.4.3.3)

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pps_extension1_flag ) { |  |
| inter_component_resi_pred_enabled_flag | u(1) |
| ... |  |

inter_component_resi_pred_enabled_flag equal to 1 specifies that inter-component residual prediction may be used in each picture referring to the PPS. inter_component_resi_pred_enabled_flag equal to 0 specifies that inter-compoent residual prediction is not used in each picture referring to the PPS. When inter_component_resi_pred_enabled_flag is not present, it is inferred to be equal to 0. When ChromaArrayType is not equal to 3, inter_component_resi_pred_enabled_flag shall be 0.

## Transform unit syntax (7.3.8.10) and semantics (7.4.9.10)

| transform_unit( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( log2TrafoSize > 2 ¦ ¦ ChromaArrayType = = 3 ) { |  |
| for( tIdx = 0; tIdx < ( ChromaArrayType = = 2 ? 2 : 1 ); tIdx++) { |  |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] && ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ IntraPredModeC[ x0 ][ y0 ] = = 4 ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ) && inter_component_resi_pred_enabled_flag ) |  |
| inter_ component_resi_pred( x0, y0, 1 ) |  |
| if( cbf_cb[ x0 ][ y0 + ( tIdx << log2TrafoSizeC ) ][ trafoDepth ] ) |  |
| residual_coding( x0, y0 + ( tIdx << log2TrafoSizeC ), log2TrafoSizeC, 1 ) |  |
| } |  |
| for( tIdx = 0; tIdx < ( ChromaArrayType = = 2 ? 2 : 1 ); tIdx++) { |  |
| if( cbf_luma[ x0 ][ y0 ][ trafoDepth ] && ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ IntraPredModeC[ x0 ][ y0 ] = = 4 ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ) && inter_component_resi_pred_enabled_flag ) |  |
| inter_ component_resi_pred( x0, y0, 2 ) |  |
| if( cbf_cr[ x0 ][ y0 + ( tIdx << log2TrafoSizeC ) ][ trafoDepth ] ) |  |
| residual_coding( x0, y0 + ( tIdx << log2TrafoSizeC ), log2TrafoSizeC, 2 ) |  |

## Inter-component residual prediction syntax (7.3.8.x) and semantics (7.4.9.x)

| inter_component_resi_pred( x0, y0, cIdx ) { | Descriptor |
| --- | --- |
| res_scale_abs_val[ cIdx ][ x0 ][ y0 ] | ae(v) |
| if( scale_abs_val[ cIdx ][ x0 ][ y0 ] != 0 ) |  |
| res_scale_sign_flag[ cIdx ][ x0 ][ y0 ] | ae(v) |
| } |  |

res_scale_abs_val[ cIdx ][ x0 ][ y0 ] specifies the absolute value of the scaling factor ResScaleVal[ cIdx ][ x0 ][ y0 ] used in inter-component residual prediction. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 1 for Cb, and equal to 2 for Cr. When res_scale_abs_val[ cIdx ][ x0 ][ y0 ] is not present, it is set to 0.

res_scale_sign_flag[ cIdx ][ x0 ][ y0 ] equal to 0 sepcifies that ResScaleVal[ cIdx ][ x0 ][ y0 ] has a positive value. res_scale_sign_flag[ cIdx ][ x0 ][ y0 ] equal to 1 specifies that ResScaleVal[ cIdx ][ x0 ][ y0 ] has a negative value.

The variable ResScaleVal[ cIdx ][ x0 ][ y0 ] is derived as follows:

if( res_scale_abs_val[ cIdx ][ x0 ][ y0 ]  = =  0 )
	ResScaleVal[ cIdx ][ x0 ][ y0 ] = 0		(x-y)
else
	ResScaleVal[ cIdx ][ x0 ][ y0 ]  =  ( 1 − 2 * res_scale_sign_flag[ cIdx ][ x0 ][ y0 ] ) *
		( 1  < <  ( res_scale_abs_val[ cIdx ][ x0 ][ y0 ] – 1 ) )

## General decoding process for intra blocks (8.4.4.1)

…

–	Otherwise (splitFlag is equal to 0), for the variable blkIdx proceeding over the values 0..( cIdx > 0  &&  ChromaArrayType  = =  2 ? 1 : 0 ), the following ordered steps apply:

…

- If inter_component_resi_pred_enabled_flag is equal to 1, the (nCbS)x(nCbS) residual sample array of the current coding block resSamples is modified as follows, for i = 0..nTbS − 1, j = 0..nTbS − 1:

  - resSamples[ xTb0  + i, yTb0  + j ] += ( ResScaleVal[ cIdx ][ xTb0 ][ yTb0 ] * resSamplesL[ xTb0 + i, yTb + j ] ) >> 3, (8-x)

where resSamplesL is the corresponding luma residual sample array of the current transform block.

8. The picture reconstruction process …

## General (8.5.4.1)

–	Otherwise (rqt_root_cbf is equal to 1), the following ordered steps apply:

…

- When ChromaArrayType is not equal to 0, the decoding process for chroma residual blocks as specified in subclause Error: Reference source not found below is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable cIdx set equal to 1, the variable nCbSw set equal to nCbSwC, the variable nCbSh set equal to nCbShC, the (nCbSL)x(nCbSL) array resSamplesL when inter_component_resi_pred_enabled_flag is equal to 1, and the (nCbSwC)x(nCbShC) array resSamplesCb as inputs, and the output is a modified version of the (nCbSwC)x(nCbShC) array resSamplesCb.

- When ChromaArrayType is not equal to 0, the decoding process for chroma residual blocks as specified in subclause Error: Reference source not found below is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ) set equal to ( 0, 0 ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable cIdx set equal to 2, the variable nCbSw set equal to nCbSwC, the variable nCbSh set equal to nCbShC, the (nCbSL)x(nCbSL) array resSamplesL when inter_component_resi_pred_enabled_flag is equal to 1, and the (nCbSwC)x(nCbShC) array resSamplesCr as inputs, and the output is a modified version of the (nCbSwC)x(nCbShC) array resSamplesCr.

## Decoding process for chroma residual blocks (8.5.4.3)

Inputs to this process are:

…

–	an array resSamplesL of luma residual samples.

…

–	Otherwise (splitChromaFlag is equal to 0), for the variable blkIdx proceeding over the values 0..( ChromaArrayType  = =  2 ? 1 : 0 ), the following ordered steps apply:

…

- The (nCbS)x(nCbS) residual sample array of the current coding block resSamples is modified as follows, for i = 0..nTbS − 1, j = 0..nTbS − 1:

if( ChromaArrayType  = =  3 )
	resSamples[ xCb + xB0  + i, yCb + yBN  + j ] = transformBlock[ i, j ]	(8-x)
			+ ( ( ResScaleVal[ cIdx ][ xCb + xB0, yCb + yB0 ] *
				resSamplesL[ xCb + xB0 + i, yCb + yBN + j ] )  >>  3 )

else

resSamples[ ( xCb + xB0 ) / SubWidthC + i, ( yCb + yBN ) / SubHeightC + j ] = transformBlock[ i, j ]	(8-1)

## CABAC parsing process for slice segment data

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process (in Section 9.3.2.2)

| … | … | … | … | .. | … |
| --- | --- | --- | --- | --- | --- |
| inter_component_resi_pred( ) | res_scale_abs_val[ ][ ][ ] | Table 9-xx | 0…2 | 3…5 | 6…8 |
|  | res_scale_sign_flag[ ][ ][ ] | Table 9-xx | 0 | 1 | 2 |

Table 9-xx – Values of initValue for ctxIdx of res_scale_abs_val[ ][ ][ ] (in Section 9.3.2.2)

| Initialization variable | ctxIdx of res_scale_abs_val[ ][ ][ ] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

Table 9-xx – Values of initValue for ctxIdx of res_scale_sign_flag[ ][ ][ ] (in Section 9.3.2.2)

| Initialization variable | ctxIdx of res_scale_sign_flag[ ][ ][ ] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 |  |  | 2 |  |  |  |  |  |  |  |  |  |
| initValue | 154 | 154 |  |  | 154 |  |  |  |  |  |  |  |  |  |
| Table 9-2 – Syntax elements and associated binarizations (in Section 9.3.3.1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  | … |  |  |  |  | … |  | … |  |  |  |  |
| inter_component_resi_pred( ) |  |  | res_scale_abs_val[ ][ ][ ] |  |  |  |  | TR |  | cMax = 3, cRiceParam = 0 |  |  |  |  |
|  |  |  | res_scale_sign_flag[ ][ ][ ] |  |  |  |  | FL |  | cMax = 1 |  |  |  |  |
| Table 9-3 – Assignment of ctxInc to syntax elements with context coded bins (in Section 9.3.4.2.1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  | … |  |  | … |  | … |  | … | … | … |  |
| res_scale_abs_val[ 0 ][ ][ ] res_scale_abs_val[ 1 ][ ][ ] |  |  |  | 0 3 |  |  | 1 4 |  | 2 5 |  | 2 5 | na | na |  |
| res_scale_sign_flag[0][ ][ ] res_scale_sign_flag[1][ ][ ] |  |  |  | 0 1 |  |  | na |  | na |  | na | na | na |  |

- – Association of ctxIdx and syntax elements for each initializationType in the initialization process

-

-

| - |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

| - |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

- Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).