| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0203 |
| --- | --- |

| Title: | RCE 3: Cross-Check of Tool A.1 from Fujitsu |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Name(s) Zhan Ma Jing Ye Haoping Yu | Tel: Email: | +1-408-330-5142 zhan.ma@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

This document reports the crosscheck results for RCE 3 test A.1 sub-categories 3 and 5, i.e., termed as test A.1.3 and test A.1.5 for respective lossless and lossy scenarios. Results reported in test A.1.3 and A.1.5 are confirmed in this report and will be detailed in the following sections.

# Introduction

Technology detailed in RCE 3 test A.1 was proposed in JCTVC N0079 [2] to simplified the SAP idea originally developed in [1].  Instead of 35 potential modes defined in [1], only 13 modes are selected which are DC, Planar and another 11 angular modes.  These 11 angular modes could be seen as the sub-sampled modes from original 33 angular modes in HEVC version 1.  On the other hand, instead of using the filtering pixels for predictions, nearest neighbor is used for prediction.  In addition to the 13 modes proposed in [2], this core experiments also test different variants for this technology, i.e.,

- Test A.1.1 N0079 original. 13 modes of intra prediction for lossless coding

- Test A.1.2 N0079_variant 1: Keep all modes of intra prediction in HEVC V1. The simplified SAP is applied for 9 angular modes (strict diagonal and half diagonal modes, mode 2, 6, 10, 14, 18, 22, 26, 30, 34) for lossless coding.

- Test A.1.3 N0079_variant 2: Keep all modes of intra prediction in HEVC V1. The simplified SAP is applied for 5 angular modes (strict diagonal modes, mode 2, 10, 18, 26, 34) for lossless coding.

- Test A.1.4 N0079 variant 3: Keep all modes of intra prediction in HEVC V1. The simplified SAP is applied for 9 angular modes (strict diagonal and half diagonal modes, mode 2, 6, 10, 14, 18, 22, 26, 30, 34) for lossy coding.

- Test A.1.5 N0079_variant 4: Keep all modes of intra prediction in HEVC V1. The simplified SAP is applied for 5 angular modes (strict diagonal modes, mode 2, 10, 18, 26, 34) for lossy coding.

Here, in this report, test A.1.3 and test A.1.5 are crosschecked.  In terms of the performance, for A.1.3, the influence on bit-rate for the All-Intra main configuration is 0.8%%, 0.2%, 0.7%, 0.5%, 0.6%, 0.6%, 0.2%, 1.9% and 1.4% for Class F, Class B, SC_RGB, Animation RGB, SC_YUV, Animation YUV, RangeExt, SC_GBR option and SC_YUV option respectively. For A.1.5, there is averaged loss observed. The overall summary is tabularized as follows.

Table 1: Crosscheck results for test A.1.3 (lossless)

Table 2: Crosscheck results for test A.1.5 (lossy)

As shown in Table 1, there is performance loss for LB, especially the Class Optional sequences. For lossy case, only LB has some marginal gain (in SC YUV 444 and SC 444 YUV Optional) as shown in Table 2. As an example, there is noticeable loss for some sequence in LB Main-tier, especially sc_video_conferencing_doc_sharing_1280x720_30_8bit_300_rgb.rgb (3.0% BD-Rate loss).

Please also note that, because of the heterogeneous computing nodes deployed for the simulation, the timing information is not reliable.

# Conclusion

This document reports the crosscheck results for simplified SAP proposed in [2].  Performance is summarized in this report. More detailed results are provided in the attached excel file.

# References

- M. Zhou, “AHG7: Sample-based angular intra prediction for HEVC range extension,” Document of Joint Collaborative Team on Video Coding, JCTVC-L0176, Jan. 2013.

- J. Zhu, W. Zheng, and K.Kazui, “Non-RCE2: Simplified sample based intra prediction for lossless coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0079, July. 2013.