| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0205 |
| --- | --- |

| Title: | AHG8: Line-based Intra Block Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Chun-Chi Chen, Tsui-Shan Chang, Ru-Ling Liao, 2Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 1cheerchen.cs98g@g2.nctu.edu.tw 2wpeng@cs.nctu.edu.tw |
|  | Chun-Lung Lin, 3Fan-Di Jou 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan |  | +886-3-582-0100 3Fandi@itri.org.tw |
| Source: | National Chiao Tung University (NCTU)/ITRI International, ITRI International |  |  |

_____________________________

# Abstract

This contribution introduces a line-based intra block copy (IntraBC) mode which relaxes the restriction that the basic unit for intra block copy must be 2Nx2N. CUs of size 2Nx2N are split equally into multiple block partitions, termed as lines, of size Mx2N or 2NxM, where M represents a power-of-two integer and is smaller than 2N. For each line, pixels are predicted in the same way as the original IntraBC except that 1) the block size is of Mx2N or 2NxM and 2) blocks of size Mx2N and 2NxM are restricted to only perform horizontal search and vertical search, respectively. Therefore, there are 2N/M intra displacement vectors per CU needed to be searched and signaled. Experiments are conducted using the HM-12.0+RExt-4.1 software and the RCE3 common test conditions to evaluate three variants (M=1, 4, and N) of this mode in the All-Intra configuration. The search range is limited to one left CTU and part of the current CTU. The average BD-rate reduction provided from the line-based IntraBC mode for screen contents (Class F/SC RGB 444/SC YUV444) is reported as follows:

- M=1: (MT) 3.0/10.6/7.5, (HT) 2.7/10.7/8.3, (SHT) 2.4/10.6/8.9, (Avg. Enc.) 114%, (Avg. Dec.) 97%;

- M=4: (MT) 2.2/7.0/6.8, (HT) 1.9/6.4/6.4, (SHT) 1.5/6.0/6.1, (Avg. Enc.) 113%, (Avg. Dec.) 94%;

- M=N: (MT) 2.5/7.0/6.7, (HT) 2.2/6.4/6.4, (SHT) 1.9/5.9/6.1, (Avg. Enc.) 114%, (Avg. Dec.) 94%.

# Introduction

At the last JCT-VC meeting, the intra block copy (IntraBC) mode is adopted in the draft HEVC Range Extension standard. Error: Reference source not found demonstrates that the intra-frame displacement vector (DV) estimation for a current CU is performed by searching a match within the search range covering the reconstructed regions of the left and the current CTUs. A CU-level flag is signaled to switch adaptively between IntraBC mode and those prediction modes specified in HEVC.

To further improve the coding efficiency of the IntraBC mode, we propose a line-based IntraBC mode. Its basic idea is to perform the IntraBC mode at a smaller block partition level than 2Nx2N with the intention to find a compromise between the prediction accuracy of the IntraBC mode and the required DV overhead. The technique was shown that the best of its configuration (M=1) achieves BD-rate savings ranging from 7.0% to 7.3% in the RCE3 All-Intra test conditions [1]. For the test M=N, although the BD-rate savings drop to 4.6-5.4%, the design is compatible with the symmetric block partitions (2NxN and Nx2N) as specified in HEVC.

- A demonstration of the IntraBC mode, where the gray area represents the search range of a current CU.

# Line-based Intra Block Copy

The line-based IntraBC mode first splits a current CU of size 2Nx2N equally into smaller block partitions, which are referred hereafter to as lines, of size Mx2N or 2NxM. A flag, when coded in the proposed mode, is introduced for allowing each CU to choose adaptively between the two partitioning types. With reference to Figure 2, the intra DV search for these lines is as follows:

- For each line of size Mx2N, only horizontal intra DV search is performed, and the search range is limited to the reconstructed region (the gray area of Figure 2 (a)) of the left and the current CTUs.

- For each line of size 2NxM, only vertical intra DV search is performed and the search range is limited to the reconstructed region (the gray area of Figure 2 (b)) of the current CTU.

A fast encoding option is enabled to skip evaluating line-based IntraBC mode for CUs of size larger than 16x16. Then, the intra DVs are signaled in the same way as signalling MVD except that 1) none of DV prediction techniques is applied and 2) the sign bit of each intra DV need not be signaled.

| (a) | (b) |
| --- | --- |

- (a) Horizontal search for lines of size Mx2N. (b) Vertical search for lines of size 2NxM.

# Experimental Results

Experiments were conducted using the HM-12.0+RExt-4.1 software [2] and the RCE3 common test conditions [1] to collect the BD-rate and the encoding and decoding times of the proposed techniques in the configurations of M=1, 4, and N.

- BD-rate savings and processing time ratios for M=1 (1x2N, 2Nx1).

| (M=1) | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -3.0 | -2.6 | -2.5 | -2.7 | -2.4 | -2.4 | -2.4 | -2.1 | -2.2 |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| SC RGB 444 | -10.6 | -10.4 | -10.6 | -10.7 | -10.5 | -10.6 | -10.6 | -10.3 | -10.4 |
| Animation RGB 444 | -0.3 | -0.3 | -0.3 | -0.1 | -0.2 | -0.2 | -0.1 | -0.1 | -0.1 |
| SC YUV 444 | -7.5 | -5.0 | -5.2 | -8.3 | -5.5 | -5.7 | -8.9 | -6.0 | -6.1 |
| Animation YUV 444 | -0.1 | -0.3 | -0.2 | -0.1 | -0.1 | -0.1 | 0.0 | -0.1 | -0.1 |
| RangeExt | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| SC(444) GBR Optional | -13.5 | -13.5 | -13.5 | -14.1 | -14.0 | -14.2 | -14.6 | -14.5 | -14.7 |
| SC(444) YUV Optional | -13.4 | -13.1 | -13.0 | -15.0 | -14.6 | -14.5 | -16.8 | -16.3 | -16.7 |
| Enc Time[%] | 114% |  |  | 114% |  |  | 114% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  | 97% |  |  |

- BD-rate savings and processing time ratios for M=4 (4x2N, 2Nx4).

| (M=4) | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.2 | -2.1 | -2.1 | -1.9 | -1.8 | -1.8 | -1.5 | -1.5 | -1.5 |
| Class B | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| SC RGB 444 | -7.0 | -6.9 | -7.0 | -6.4 | -6.4 | -6.4 | -6.0 | -6.0 | -6.0 |
| Animation RGB 444 | -0.2 | -0.2 | -0.2 | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| SC YUV 444 | -6.8 | -4.8 | -4.9 | -6.4 | -4.6 | -4.6 | -6.1 | -4.2 | -4.2 |
| Animation YUV 444 | -0.1 | -0.2 | -0.2 | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | -0.1 |
| RangeExt | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| SC(444) GBR Optional | -7.7 | -7.9 | -7.9 | -7.6 | -7.6 | -7.7 | -7.5 | -7.5 | -7.7 |
| SC(444) YUV Optional | -8.6 | -8.5 | -8.6 | -8.6 | -8.5 | -8.5 | -8.7 | -8.6 | -8.6 |
| Enc Time[%] | 112% |  |  | 113% |  |  | 113% |  |  |
| Dec Time[%] | 94% |  |  | 94% |  |  | 94% |  |  |

- BD-rate savings and processing time ratios for M=N (Nx2N, 2NxN).

| (M=N) | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -2.5 | -2.5 | -2.6 | -2.2 | -2.2 | -2.2 | -1.9 | -1.9 | -1.9 |
| Class B | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| SC RGB 444 | -7.0 | -6.9 | -7.0 | -6.4 | -6.4 | -6.4 | -5.9 | -5.9 | -5.9 |
| Animation RGB 444 | -0.2 | -0.2 | -0.2 | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| SC YUV 444 | -6.7 | -4.8 | -4.8 | -6.4 | -4.5 | -4.5 | -6.1 | -4.2 | -4.2 |
| Animation YUV 444 | -0.1 | -0.2 | -0.2 | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | 0.0 |
| RangeExt | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| SC(444) GBR Optional | -7.7 | -7.7 | -7.7 | -7.6 | -7.5 | -7.6 | -7.5 | -7.4 | -7.6 |
| SC(444) YUV Optional | -8.5 | -8.3 | -8.4 | -8.4 | -8.3 | -8.4 | -8.6 | -8.5 | -8.5 |
| Enc Time[%] | 113% |  |  | 114% |  |  | 114% |  |  |
| Dec Time[%] | 94% |  |  | 94% |  |  | 94% |  |  |

# Conclusion

In this contribution, a line-based IntraBC mode is proposed. Three variants (M=1, 4, and N) of this mode is evaluated. The test, M=N, achieves most of the coding gains while remaining almost the same runtime complexities as the other tests (M=1 and 4), and its block partitioning type is exactly the same as PUs of size 2NxN and Nx2N in HEVC Version 1. In viewing of this, it may be worth considering allowing intra block copy to be performed on such PU’s.

# References

- A. Saxena, D.-K. Kwon, M. Naccari, and C. Pang, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques,” JCTVC-N1123, Vienna, Jul. 2013.

- JCT-VC, “HM-12.0+RExt-4.1,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-12.0+RExt-4.1/

# Patent rights declaration(s)

NCTU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).