| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0207_r1 |
| --- | --- |

| Title: | AhG5 and AhG18: Bypass bins alignment depending on the Rice parameter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

A method was presented in JCTVC-M0178 for aligning the CABAC process prior to coding bypass data by setting the range equal to 256. The method reportedly allows simultaneous decoding of multiple bypass bins. Each alignment incurs a penalty of 0.55 bits on average. This contribution proposes to apply the aligning only when the Rice parameter is above a threshold and the sign data hiding condition is met, thus applying the alignment when large amount of bypass data is expected to be coded. The method is also combined with the Rice initialization method D1 of RCE2 (JCTVC-O0239). The performance impact is reported to be negligible for the CTC settings and an average loss of 0.3% for the high bit-depth AhG18 settings, which goes down to 0.1% on top of RCE2 D1.

# Introduction

HEVC Range Extensions encompasses high bit-depths and extended chroma formats that impose increased demands on the throughput of the codec, especially when the ‘high precision’ process adopted in the 14th JCTVC meeting is enabled.

The CABAC engine has two operating modes to code bins: the context mode and the bypass mode. Bypass mode assumes an equi-probable model and is simpler than the context mode. Bypass allows easier parallelization because it does not require context derivation and adaptation. Parallelization is further facilitated when the binary code is regular (e.g., unary or fixed length code).

Aligning the bitstream allows even further parallelization of the bypass bin decoding. An alignment method was proposed in JCTVC-M0178 [1] that simply sets the range equal to 256. The average penalty for the alignment is 0.55 bits. Therefore, the alignment should be applied only when large amount of bypass data is expected for the sub-block (coefficient group) in order to minimize performance loss and also, to allow the benefits of the parallelization techniques when they are actually needed.

This proposal targets both goals by aligning CABAC only when the Rice parameter is above a threshold. The amount of residual data being coded is highly correlated with the Rice parameter value because large coefficients increase the Rice parameter by means of the Rice parameter update. For example, 95% of the sub-blocks have Rice equal to 4 for the high bit-depth test conditions, while for the common test conditions that number is less than 1%.

# Technical description

The proposed method is to align the arithmetic coder by setting the range to 256 only when the Rice parameter is above a threshold.

The method can also be naturally combined with the one in RCE2 test D1 [2], since test D1 sets an initial value for the Rice parameter for each sub-block which can be used for the condition on aligning.

## Method 1: for the current specification

The Rice parameter at the end of each 4×4 sub-block is stored. At the beginning of the next sub-block (same TU or different one) the stored parameter is used. If the parameter is 4, then the arithmetic coder is aligned prior to coding the bypass data (i.e., flags coeff_sign_flag and coeff_abs_level_remaining).

In order to trigger the alignment, it is also checked whether the difference between the position of the last non-zero and the first non-zero in the sub-block is above 3. This re-uses the condition of sign data hiding and guarantees a minimum of bypass bins to be coded in the sub-block.

In summary, the method applies the following before decoding the sign data:

if ( prevGoRiceParam > 3 && ( lastNZPosInCG - firstNZPosInCG >= 4 )  )  {

m_uiRange = 256;

}

## Method 2: on top of RCE2 test D1

RCE2 test D1 method derives an initial value of the Rice parameter for each 4×4 sub-block and allows a maximum Rice parameter above 4. In that case, the proposed method can use a threshold larger than 4.

However, this proposal on top of RCE2 test D1 can just use the derived initial Rice in the condition for aligning the arithmetic coder. Therefore, there is no need to store the Rice parameter from the previous 4×4 subblock. In addition, the threshold for alignment can be set to 5, so that the method is equivalent to the current HEVC specification whenever the Rice parameter is within the range of the values allowed in HEVC (that is, from 0 to 4).

In order to trigger the alignment, it is also checked whether the difference between the position of the last non-zero and the first non-zero in the sub-block is above 3.

In summary, the method applies the following before decoding the sign data:

if ( uiGoRiceParam > 4 && ( lastNZPosInCG - firstNZPosInCG >= 4 )  )  {

m_uiRange = 256;

}

## Method 3: condition using the number of non-zero coefficients

Method 3 is similar to method 1. It uses the number of significant coefficients in the sub-block (numNonZero) in the condition for aligning the arithmetic coder:

if ( prevGoRiceParam > 3 | | numNonZero > 8 )  {

m_uiRange = 256;

}

# Results

The proposal is implemented in HM12.0_Rext4.1 and in HM12.0_Rext4.1+Test D1 (setting 1). Simulations are performed for AhG18 conditions and common test conditions (CTC). The performance is compared to the anchor in terms of BD-rate savings.

## Results for AhG18

Table 1 shows the results for the high bit-depths settings.

Table 1. Results for method 1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | 0.4% | 0.4% | 0.4% |
| 14-bit | 0.3% | 0.3% | 0.3% |
| 16-bit | 0.2% | 0.2% | 0.2% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2 left shows the results of RCE2 test D1 plus the proposed method. The right part shows the performance of the method using RCE2 test D1 as anchor.

Table 2. Results for method 2 (left) and method 2 with respect to RCE2 test D1 (right)

| All Intra. Y; U; V. 12-bit; -7.1%; -6.8%; -6.8%. 14-bit; -18.5%; -18.2%; -18.1%. 16-bit; -28.5%; -28.3%; -28.4%. Enc Time[%]; 93%. Dec Time[%]; 83%. | All Intra. Y; U; V. 12-bit; 0.0%; 0.0%; 0.0%. 14-bit; 0.2%; 0.2%; 0.2%. 16-bit; 0.2%; 0.2%; 0.2%. Enc Time[%]; 106%. Dec Time[%]; 97%. |
| --- | --- |

Table 3. Results for method 3

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | 0.4% | 0.4% | 0.4% |
| 14-bit | 0.3% | 0.3% | 0.3% |
| 16-bit | 0.2% | 0.2% | 0.2% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 96% |  |  |

## Results for CTC

No performance impact is observed for the methods for the common test conditions for methods 1 and 2.

Table 4. Results for method 1

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  | 104% |  |  | 104% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 107% |  |  | 106% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| YCbCr 4:2:2 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | -0.3% | -0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 104% |  |  | 104% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |  |  |  |

Table 5. Results for method 2

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 104% |  |  | 104% |  |  | 104% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  | 103% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 104% |  |  | 105% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 103% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |

Table 6. Results for method 3

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.4% | 0.4% | 0.4% | 0.5% | 0.5% | 0.5% | 0.6% | 0.6% | 0.6% |
| YCbCr 4:4:4 | 0.3% | 0.3% | 0.3% | 0.4% | 0.4% | 0.4% | 0.5% | 0.5% | 0.5% |
| YCbCr 4:2:2 | 0.3% | 0.4% | 0.4% | 0.4% | 0.4% | 0.4% | 0.5% | 0.5% | 0.5% |
| Overall | 0.3% | 0.4% | 0.4% | 0.4% | 0.4% | 0.4% | 0.5% | 0.6% | 0.5% |
|  | 0.3% | 0.4% | 0.4% | 0.4% | 0.5% | 0.4% | 0.5% | 0.6% | 0.5% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.2% | 0.2% | 0.2% | 0.3% | 0.4% | 0.3% |  |  |  |
| YCbCr 4:4:4 | 0.2% | 0.1% | 0.2% | 0.2% | 0.2% | 0.2% |  |  |  |
| YCbCr 4:2:2 | 0.2% | 0.2% | 0.2% | 0.3% | 0.2% | 0.2% |  |  |  |
| Overall | 0.2% | 0.2% | 0.2% | 0.3% | 0.3% | 0.2% |  |  |  |
|  | 0.2% | 0.0% | 0.2% | 0.3% | 0.3% | 0.2% |  |  |  |
| Enc Time[%] | 103% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.1% | 0.2% | 0.1% | 0.3% | 0.3% | 0.2% |  |  |  |
| YCbCr 4:4:4 | 0.2% | 0.1% | 0.0% | 0.2% | 0.2% | 0.1% |  |  |  |
| YCbCr 4:2:2 | 0.1% | -0.2% | 0.0% | 0.2% | 0.1% | 0.0% |  |  |  |
| Overall | 0.1% | 0.0% | 0.1% | 0.2% | 0.2% | 0.1% |  |  |  |
|  | 0.1% | -0.1% | 0.1% | 0.2% | 0.2% | 0.1% |  |  |  |
| Enc Time[%] | 102% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# Conclusions

A simple condition for aligning the arithmetic coder for bypass bins is presented. The method enables higher throughput and has smaller penalty in performance when applied on top of the Rice initialization method in JCTVC-O0239.

# References

- K. Sharman, N. Saunders, J. Gamei, “AHG5: Range Extensions and High Bit Depths”, JCTVC-M0178, Incheon, KR, April 2013.

- L. Guo, J. Sole, R. Joshi, M. Karczewicz, K. Sharman, N. Saunders, J. Gamei, “RCE2: Results of Test D1 on Rice Parameter Initialization”, JCTVC-O0239, Geneva, CH, October 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).