| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0208_r1 |
| --- | --- |

| Title: | AhG5 and AhG18: Bypass of the coefficient level flags for higher throughput |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 joels@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

A method is presented for coding the coeff_abs_level_greaterX_flags in bypass mode, reportedly enabling a higher throughput codec. The flags are coded as part of the syntax element coeff_abs_level_remaining when the Rice parameter is above a threshold. The method is also combined with the Rice initialization method D1 of RCE2 (JCTVC-O0239). The performance impact of the method is reported to be negligible for the high bit-depth AhG18 and CTC settings.

# Introduction

HEVC Range Extensions encompasses high bit-depths and extended chroma formats that impose increased demands on the throughput of the codec, especially when the ‘high precision’ process adopted in the 14th JCT-VC meeting is enabled.

The CABAC engine has two operating modes to code the bins: the context mode and the bypass mode. For the context-coded mode, the engine uses the selected context model to code the binary symbol. After each bin is coded, the model is updated. This mode is highly sequential and has strong data dependences.

Meanwhile, the bypass mode assumes an equi-probable model. This mode is simpler and allows a coding speedup and easier parallelization because it does not require context derivation and adaptation. Parallelization is further facilitated when the binary code is regular (e.g., unary or fixed length code).

This proposal targets the reduction of the context-coded bins in the residual data. The method is activated when the Rice parameter is above a threshold, which in turn occurs when the residual data values are high. Therefore, the approach improves throughput in the worst-case and average scenarios.

# Technical description

The amount of residual data being coded is highly correlated with the Rice parameter value because a large coefficient magnitude increases the Rice parameter by means of the following update formula:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-1)

At high bit-rates, the Rice parameter saturates to its maximum value. Then, all the context-coded bins are used (coded_sub_block_flag, significance_coeff_flag, and coeff_abs_level_greaterX_flag), and large values of coeff_abs_level_remaining are coded (in bypass mode).

The greater than 1 and 2 flags can be coded in bypass when the Rice parameter is large without any performance penalty. The flags are coded as part of the remaining level. This improves throughput and removes a coding loop over the 4×4 sub-blocks.

The proposed method can be naturally combined with the one proposed in RCE2 test D1 [1], since that method sets an initial value for the Rice parameter for each sub-block which can be used as the condition for bypassing the coding of the level flags.

## Method for the current specification

The Rice parameter at the end of each 4×4 sub-block is stored. At the beginning of the next sub-block (same TU or different one) that parameter is used. If the parameter is 4, then the level flags are bypass. Otherwise, the usual coding process is applied.

The process can be summarized as follows:

| HEVC | Proposed Method 1 |
| --- | --- |
| Encode coded_sub_block_flag Loop to encode significance_coeff_flag Loop to encode coeff_abs_level_greater1_flag Loop to encode coeff_abs_level_greater2_flag Loop to encode coeff_sign_flag Loop to encode coeff_abs_level_remaining | Encode coded_sub_block_flag Loop to encode significance_coeff_flag If ( previousRiceGolomb < 4 ) Loop to encode coeff_abs_level_greater1_flag Loop to encode coeff_abs_level_greater2_flag Loop to encode coeff_sign_flag Loop to encode coeff_abs_level_remaining |

When previousRiceGolomb=4, the syntax element coeff_abs_level_remaining always encodes the absolute level of the coefficient minus 1.

## Method on the top of RCE2 test D1

RCE2 test D1 method derives an initial value of the Rice parameter for each 4×4 sub-block and allows a maximum Rice parameter above 4. In that case, the proposed method can use a threshold larger than 4.

However, this proposal can be even simpler on top of RCE2 test D1 by just using the derived initial Rice parameter as the condition for bypassing the level flags. Therefore, there is no need to store the previous Rice parameter. Also, the threshold can be set to 5, so the method is equivalent to the current HEVC specification whenever the Rice parameter is within the range of values allowed in HEVC (from 0 to 4).

Proposed Method 2

Encode coded_sub_block_flag Loop to encode significance_coeff_flag If ( initialRiceGolomb < 5 ) Loop to encode coeff_abs_level_greater1_flag Loop to encode coeff_abs_level_greater2_flag Loop to encode coeff_sign_flag Loop to encode coeff_abs_level_remaining

# Results

The proposal is implemented in HM12.0_Rext4.1 and in HM12.0_Rext4.1+Test D1 (setting 1). Simulations are done for AhG18 conditions and common test conditions (CTC). The performance is compared to the anchor in terms of BD-rate savings. Bin count savings are also provided.

## Results for AhG18

Table 1 shows the results for the high bit-depths settings.

Table 1. Results for method 1 (left) and method 2 (right)

| All Intra. Y; U; V. 12-bit; 0.6%; 0.7%; 0.7%. 14-bit; 0.2%; 0.3%; 0.3%. 16-bit; 0.0%; 0.0%; 0.0%. Enc Time[%]; 98%. Dec Time[%]; 94%. | All Intra. Y; U; V. 12-bit; -7.1%; -6.8%; -6.8%. 14-bit; -18.7%; -18.4%; -18.4%. 16-bit; -28.7%; -28.5%; -28.6%. Enc Time[%]; 92%. Dec Time[%]; 89%. |
| --- | --- |

Table 2 shows the results of RCE2 test D1 plus the proposed method using RCE2 test D1 as anchor.

Table 2. Results for method 2 on with RCE2 test D1 as anchor

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | 0.0% | 0.0% | 0.0% |
| 14-bit | -0.1% | 0.0% | 0.0% |
| 16-bit | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 103% |  |  |

Table 3 shows the percentage of used bins for method 1 with respect to the anchor. A reduction of 26% of content-coded bins is achieved. The reduction is up to 95% for the cofficient level flags.

Table 3. Percentage of bins for method 1 respect to anchor

| Bin category | Bin usage w.r.t. anchor |
| --- | --- |
| Total | 96.8% |
| Context-coded | 73.9% |
| Bypass | 101.3% |
| Greater than 1 | 5.2% |
| Greater than 2 | 5.2% |

## Results for CTC

Very small performance impact is observed for method 1 (table 4) for common test conditions and no impact for method 2 (table 5).

Table 4. Results for method 1

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 105% |  |  | 105% |  |  | 106% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Enc Time[%] | 107% |  |  | 107% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:2:2 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 106% |  |  | 106% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |

Table 5. Results for method 2

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106% |  |  | 105% |  |  | 106% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 95% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 107% |  |  | 106% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |  |  |  |

# Conclusions

A simple method to encode in bypass the level flags is presented. The method enables higher throughput and has no penalty in performance when applied on the top of the Rice initialization method in JCTVC-O0239.

# References

- M. Karczewicz, L. Guo, J. Sole, R. Joshi, K. Sharman, N. Saunders, J. Gamei, “RCE2: Results of Test D1 on Rice Parameter Initialization”, JCTVC-O0239, Geneva, CH, October 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).