| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0212 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0072 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On CL-RAS pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-5801 aramasub@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes that two non-IRAP NAL unit types be used for indication of cross-layer random access skipped (CL-RAS) pictures, i.e. enhancement layer pictures that are not leading pictures and that are not correctly decodable when the decoding process starts at an initial IRAP access unit that contains non-IRAP pictures. Distinguishing these pictures with NAL unit types allows systems entities (such as media-aware network elements) to easily identify pictures that can be discarded, similar as for RASL pictures. The contribution also explains the conformance of bitstreams that have CL-RAS pictures removed.

# Introduction

The decoding process for SHVC and MV-HEVC can start at any initial IRAP access unit. When the initial IRAP access unit contains non-IRAP pictures at enhancement layers, the correct decoding of pictures in those layers may not be possible due to unavailability of temporal reference pictures. IRAP AUs that contain non-IRAP pictures will be referred to as non-aligned IRAP AUs or IRAP AUs that are non-aligned, and the enhancement layer pictures that are not leading pictures and that may not correctly decodable when the decoding process starts from an IRAP access unit are referred to as cross-layer random access skipped (CL-RAS) pictures.

Enabling the identification of CL-RAS pictures would use two out of the 14 reserved non-IRAP NAL unit types. In streaming scenarios a middle box may transmit a bitstream such that the decoder starts decoding by random accessing the bitstream at an initial IRAP AU that is not aligned. When the bandwidth is limited, it may be desirable to not include CL-RAS pictures. Under the current SHVC and MV-HEVC specifications, a bitstream generated by removing these CL-RAS may not be conforming. The decoding process and the conformance need to be specified such that even when CL-RAS pictures are removed from the bitstream, the bitstream would still be a conformant bitstream.

All CL-RAS pictures are trailing pictures. In previous discussions in the JCT-VC meetings, it was also commented that identifying these pictures as CL-RAS pictures (with a separate NAL unit type) would prevent their identification as TSA/STSA pictures when applicable. However, in most typical use cases, when temporal scalability is enabled in the bitstream, the value of vps_temporal_id_nesting_flag and sps_temporal_id_nesting_flag are also set to 1, thereby indicating that all temporal switching is possible at all pictures in the bitstream, and that it is not needed to separately identify the pictures as TSA or STSA pictures.

# Proposal

In this proposal, the CL-RAS pictures are associated with the first initial IRAP AU that precedes the picture in decoding order. These pictures would not be correctly decodable when the decoding process starts from the associated initial IRAP access unit. Figure 1 shows an example with CL-RAS pictures, and illustrates the association of these pictures with an initial IRAP access unit. When the decoding process starts from a particular IRAP access unit (in which case this would then also be an initial IRAP AU, if not so already), the associated CL-RAS pictures may be removed and the bitstream would still be conforming.

Figure 1: Association of CL-RAS pictures with IRAP access units. AU y does not have any association CL-RAS pictures in this bitstream

The changes proposed to the MV-HEVC WD5 are highlighted below in green or red strikethrough.

## Definition of CL-RAS pictures

clean random access skipped (CL-RAS) picture: A coded picture for which each VCL NAL unit has nal_unit_type equal to CL_RAS_R or CL_RAS_N.

associated CL-RAS pictures: CL-RAS pictures for an initial IRAP AU where the CL-RAS pictures within a CVS with layer ID value nuh_layer_id contained in layerSetLayerIdList that succeed the first picture in an initial IRAP AU and precede, in decoding order, an IRAP picture with layer ID value nuh_layer_id.

## Changes to NAL unit header semantics

The specifications in subclause 7.4.2.2 apply with following modifications and additions.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7 1.

Table 7 1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 1 | TRAIL_N TRAIL_R | Coded slice segment of a non-TSA, non-STSA trailing picture slice_segment_layer_rbsp( ) | VCL |
| 2 3 | TSA_N TSA_R | Coded slice segment of a TSA picture slice_segment_layer_rbsp( ) | VCL |
| 4 5 | STSA_N STSA_R | Coded slice segment of an STSA picture slice_segment_layer_rbsp( ) | VCL |
| 6 7 | RADL_N RADL_R | Coded slice segment of a RADL picture slice_segment_layer_rbsp( ) | VCL |
| 8 9 | RASL_N RASL_R | Coded slice segment of a RASL picture slice_segment_layer_rbsp( ) | VCL |
| 10 11 | CL_RAS_N CL_RAS_R | Coded slice segments of a CL-RAS pictures slice_segment_layer_rbsp( ) | VCL |
| 10 12 14 | RSV_VCL_N10 RSV_VCL_N12 RSV_VCL_N14 | Reserved non-IRAP sub-layer non-reference VCL NAL unit types | VCL |
| 11 13 15 | RSV_VCL_R11 RSV_VCL_R13 RSV_VCL_R15 | Reserved non-IRAP sub-layer reference VCL NAL unit types | VCL |
| 16 17 18 | BLA_W_LP BLA_W_RADL BLA_N_LP | Coded slice segment of a BLA picture slice_segment_layer_rbsp( ) | VCL |
| 19 20 | IDR_W_RADL IDR_N_LP | Coded slice segment of an IDR picture slice_segment_layer_rbsp( ) | VCL |
| 21 | CRA_NUT | Coded slice segment of a CRA picture slice_segment_layer_rbsp( ) | VCL |
| 22 23 | RSV_IRAP_VCL22 RSV_IRAP_VCL23 | Reserved IRAP VCL NAL unit types | VCL |
| 24..31 | RSV_VCL24.. RSV_VCL31 | Reserved non-IRAP VCL NAL unit types | VCL |
| 32 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 33 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 34 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 35 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 36 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 37 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 38 | FD_NUT | Filler data filler_data_rbsp( ) | non-VCL |
| 39 40 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 41..47 | RSV_NVCL41.. RSV_NVCL47 | Reserved | non-VCL |
| 48..63 | UNSPEC48.. UNSPEC63 | Unspecified | non-VCL |

All coded slice segment NAL units with a particular value of nuh_layer_id in an access unit shall have the same value of nal_unit_type. A picture is also referred to as having a nal_unit_type equal to the nal_unit_type of the coded slice segment NAL units of the picture.

If a picture has nal_unit_type equal to TRAIL_N, TSA_N, STSA_N, RADL_N, RASL_N, CL_RAS_N, RSV_VCL_N12, or RSV_VCL_N14, the picture is a sub-layer non-reference picture. Otherwise, the picture is a sub-layer reference picture.

When one picture picA of a layer layerA has nal_unit_type equal to TSA_N or TSA_R, each picture in the same access unit as picA in a direct or indirect reference layer of layerA shall have nal_unit_type equal to TSA_N or TSA_R.

When one picture picA of a layer layerA has nal_unit_type equal to STSA_N or STSA_R, each picture in the same access unit as picA in a direct or indirect reference layer of layerA shall have nal_unit_type equal to STSA_N or STSA_R.

nuh_layer_id specifies the identifier of the layer.

When nal_unit_type is equal to AUD_NUT, the value of nuh_layer_id shall be equal to the minimum of the nuh_layer_id values of all VCL NAL units in the access unit.

When nal_unit_type is equal to VPS_NUT, the value of nuh_layer_id shall be equal to 0. Decoder shall ignore NAL units with nal_unit_type equal to VPS_NUT and nuh_layer_id greater than 0.

When nal_unit_type is equal to CL_RAS_R or CL_RAS_N, the value of nuh_layer_id shall be greater than 0.

## Changes to the general decoding process

F.8.1	General decoding process

The specifications in subclause 8.1 apply with following additions.

When the current AU is an initial IRAP AU, the following applies:

- If some external means not specified in this Specification is available to set the variable NoClrasPicPresentFlag to a value for the current AU, the variable NoClrasPicPresentFlag is set equal to the value provided by the external means.

- Otherwise, the variable NoClrasPicPresentFlag is set equal to 0.

NOTE – NoClrasPicPresentFlag equal to 0 specifies that all the CL-RAS pictures associated with the initial IRAP AU are not present in the bitstream. NoClrasPicPresentFlag equal to 1 specifies that all the CL-RAS pictures that are associated with the initial IRAP AU.

When the current picture has nuh_layer_id greater than 0, the following applies.

–	Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the following decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the following decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1 and 2 being assigned to SL, SCb, and SCr, respectively.

NOTE – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

–	The decoding process operates as follows for the current picture CurrPic.

–	For the decoding of the slice segment header of the first slice, in decoding order, of the current picture, the decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

–	When ViewScalExtLayerFlag[ nuh_layer_id ] is equal to 1, the decoding process for a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

–	After all slices of the current picture have been decoded, the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

## Picture timing SEI message syntax

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| if( frame_field_info_present_flag ) { |  |
| pic_struct | u(4) |
| source_scan_type | u(2) |
| duplicate_flag | u(1) |
| } |  |
| if( CpbDpbDelaysPresentFlag ) { |  |
| irap_cross_layer_cpb_params_present_flag | u(1) |
| au_cpb_removal_delay_minus1 | u(v) |
| pic_dpb_output_delay | u(v) |
| if( sub_pic_hrd_params_present_flag ) |  |
| pic_dpb_output_du_delay | u(v) |
| if( irap_cross_layer_cpb_params_present_flag ) { |  |
| au_alt_cpb_removal_delay_minus1 | u(v) |
| au_alt_pic_dpb_delay | u(v) |
| } |  |
| if( sub_pic_hrd_params_present_flag && sub_pic_cpb_params_in_pic_timing_sei_flag ) { |  |
| num_decoding_units_minus1 | ue(v) |
| du_common_cpb_removal_delay_flag | u(1) |
| if( du_common_cpb_removal_delay_flag ) |  |
| du_common_cpb_removal_delay_increment_minus1 | u(v) |
| for( i = 0; i <= num_decoding_units_minus1; i++ ) { |  |
| num_nalus_in_du_minus1[ i ] | ue(v) |
| if( !du_common_cpb_removal_delay_flag && i < num_decoding_units_minus1 ) |  |
| du_cpb_removal_delay_increment_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |

## Changes to Picture timing SEI message semantics

The semantics of all the syntax elements remain the same, except for those that are highlighted below.

irap_cross_layer_cpb_params_present_flag equal to 1 specifies the presence of the au_alt_cpb_removal_delay_minus1 and au_alt_pic_dpb_delay syntax elements. When not present, the value of irap_cross_layer_cpb_params_present_flag is inferred to be equal to 0. When the associated picture is neither a CRA picture nor a BLA picture, the value of irap_cross_layer_cpb_params_present_flag shall be equal to 0.

NOTE 1 – No more than one of the values of sub_pic_hrd_params_present_flag, irap_cpb_params_present_flag and irap_cross_layer_cpb_params_present_flag can be equal to 1.

au_cpb_removal_delay_minus1 plus 1 specifies the number clock ticks between the nominal CPB removal time of the access unit associated with the picture timing SEI message and the preceding access unit in decoding order that contained a buffering period SEI message. This value is also used to calculate an earliest possible time of arrival of access unit data into the CPB for the HSS. The syntax element is a fixed length code whose length in bits is given by AuCpbRemovalDelayLength.

NOTE 2 – The value of AuCpbRemovalDelayLength that determines the length (in bits) of the syntax element au_cpb_removal_delay_minus1 is derived from the value of au_cpb_removal_delay_length_minus1 or au_bp_cpb_removal_delay_length_minus1 coded in the VPS or the SPS that is active for the coded picture associated with the picture timing SEI message, although au_cpb_removal_delay_minus1 specifies a number of clock ticks relative to the removal time of the preceding access unit containing a buffering period SEI message, which may be an access unit of a different CVS.

au_alt_cpb_removal_delay_minus1 plus 1 specifies the alternate number clock ticks between the nominal CPB removal time of the access unit associated with the picture timing SEI message and the preceding access unit in decoding order that contained a buffering period SEI message. This value is also used to calculate an earliest possible time of arrival of access unit data into the CPB for the HSS. The syntax element is a fixed length code whose length in bits is given by AuCpbRemovalDelayLength.

NOTE 3 – The value of AuCpbRemovalDelayLength that determines the length (in bits) of the syntax element au_alt_cpb_removal_delay_minus1 is derived from the value of au_cpb_removal_delay_length_minus1 or au_bp_cpb_removal_delay_length_minus1 coded in the VPS or the SPS that is active for the coded picture associated with the picture timing SEI message, although au_alt_cpb_removal_delay_minus1 specifies a number of clock ticks relative to the removal time of the preceding access unit containing a buffering period SEI message, which may be an access unit of a different CVS.

- The variable UseAuCpbRemovalDelayMinus1 is derived as follows:

- If either of the following conditions is true, UseAuCpbRemovalDelayMinus1 is set to be equal to au_alt_cpb_removal_delay_minus1.

  - the preceding initial IRAP access unit in decoding order has NoClRasPicPresentFlag equal to 1, irap_cross_layer_cpb_params_present_flag is equal to 1

  - if the current AU is an initial IRAP AU that has NoClRasPicPresentFlag equal to 1 and irap_cross_layer_cpb_params_present_flag is equal to 1,

- Otherwise, UseAuCpbRemovalDelayMinus1 is set to be equal to au_cpb_removal_delay_minus1.

- The variable AuCpbRemovalDelayMsb of the current picture is derived as follows:

- If the current picture is associated with a buffering period SEI message that is applicable to at least one of the operation points to which the picture timing SEI message applies, AuCpbRemovalDelayMsb is set equal to 0.

- Otherwise, let prevAnchorPic be the the previous picture in decoding order that has TemporalId equal to 0, that is not a RASL, RADL or sub-layer non-reference picture, and that is within the same buffering period as the current picture, and the following applies:

- If prevAnchorPic is associated with a buffering period SEI message that is applicable to at least one of the operation points to which the picture timing SEI message applies, AuCpbRemovalDelayMsb is set equal to 0.

- Otherwise, the following applies:

- Let maxCpbRemovalDelay be equal to 2AuCpbRemovalDelayLength.

- Let prevAuCpbRemovalDelayMinus1 and prevAuCpbRemovalDelayMsb be set equal to UseAuCpbRemovalDelayMinus1au_cpb_removal_delay_minus1 and AuCpbRemovalDelayMsb, respectively, of prevAnchorPic.

- AuCpbRemovalDelayMsb is derived as follows:

if(UseAuCpbRemovalDelayMinus1au_cpb_removal_delay_minus1  <=  prevAuCpbRemovalDelayMinus1 )
	AuCpbRemovalDelayMsb = prevAuCpbRemovalDelayMsb + maxCpbRemovalDelay	(D-1)
else
	AuCpbRemovalDelayMsb = prevAuCpbRemovalDelayMsb

The variable AuCpbRemovalDelayVal is derived as follows:

AuCpbRemovalDelayVal = AuCpbRemovalDelayMsb + UseAuCpbRemovalDelayMinus1au_cpb_removal_delay_minus1 + 1	(D-2)

The value of AuCpbRemovalDelayVal shall be in the range of 1 to 232, inclusive. Within one buffering period, the AuCpbRemovalDelayVal values for any two access units shall not be the same.

pic_dpb_output_delay is used to compute the DPB output time of the picture when SubPicHrdFlag is equal to 0. It specifies how many clock ticks to wait after removal of the last decoding unit in an access unit from the CPB before the decoded picture is output from the DPB.

NOTE 4 – A picture is not removed from the DPB at its output time when it is still marked as "used for short-term reference" or "used for long-term reference".

The length of the syntax element pic_dpb_output_delay is given in bits by dpb_output_delay_length_minus1 + 1. When sps_max_dec_pic_buffering_minus1[ minTid ] is equal to 0, where minTid is the minimum of the OpTid values of all operation points the picture timing SEI message applies to, pic_dpb_output_delay shall be equal to 0.

pic_alt_dpb_output_delay is the alternate value used to compute the DPB output time of the picture when SubPicHrdFlag is equal to 0 and CL-RAS pictures associated with the preceding initial IRAP AU in decoding order are removed from the bitstream. It specifies how many clock ticks to wait after removal of the last decoding unit in an access unit from the CPB before the decoded picture is output from the DPB.

The length of the syntax element pic_alt_dpb_output_delay is given in bits by dpb_output_delay_length_minus1 + 1. When sps_max_dec_pic_buffering_minus1[ minTid ] is equal to 0, where minTid is the minimum of the OpTid values of all operation points the picture timing SEI message applies to, pic_alt_dpb_output_delay shall be equal to 0.

The output time derived from the pic_dpb_output_delay or pic_alt_dpb_output_delay of any picture that is output from an output timing conforming decoder shall precede the output time derived from the pic_dpb_output_delay of all pictures in any subsequent CVS in decoding order.

The picture output order established by the values of this syntax elementpic_dpb_output_delay or pic_alt_dpb_output_delay shall be the same order as established by the values of PicOrderCntVal.

For pictures that are not output by the "bumping" process because they precede, in decoding order, an IRAP picture with NoRaslOutputFlag equal to 1 that has no_output_of_prior_pics_flag equal to 1 or inferred to be equal to 1, the output times derived from pic_dpb_output_delay or pic_alt_dpb_output_delay shall be increasing with increasing value of PicOrderCntVal relative to all pictures within the same CVS.

## Cross-layer HRD parameters SEI message syntax

| cross_layer_hrd_parameters( payloadSize ) { | Descriptor |
| --- | --- |
| cross_layer_video_parameter_set_id | ue(v) |
| for( i = 0; i < vps_num_hrd_parameters; i++ ) |  |
| for( j = 0; j < vps_max_sub_layers_minus1; j++ ) |  |
| for( k = 0; k < CrossLayerCpbCnt[ i ][ j ]; k++ ) { |  |
| applicable_poc_cnt[ i ][ j ][ k ] | u(32) |
| cross_layer_bit_rate_value_minus1[ i ][ j ][ k ] | ue(v) |
| cross_layer_cpb_size_value_minus1[ i ][ j ][ k ] | ue(v) |
| } |  |
| } |  |

## Cross-layer HRD parameters SEI message semantics

This SEI message may and only may be associated with IRAP AUs.

cross_layer_video_parameter_set_id indicates and shall be equal to the value of the vps_video_parameter_set_id of the VPS that is referred to by the VCL NAL units of the access unit associated with the SEI message. The value of cross_layer_video_parameter_set_id shall be in the range of 0 to 15, inclusive.

The variable CrossLayerCpbCnt[ i ][ j ] is set equal to cpb_cnt_minus1[ j ] + 1 of the i-th set of HRD parameters signalled in the VPS.

applicable_poc_cnt[ i ][ j ][ k ] specifies the access unit until which cross_layer_bit_rate_value_minus1[ i ][ j ][ k ] and cross_layer_cpb_size_value_minus1[ i ][ j ][ k ] are applicable. If there is an access unit that follows the current access unit (i.e. the intial IRAP AU associated with the current SEI message) in decoding order in the CVS, and this access unit contains a picture picA with nuh_layer_id equal to 0 and has PicOrderCntVal equal to the PicOrderCntVal of the picture in the current access unit with nuh_layer_id equal to 0 plus the value of applicable_poc_cnt, the picture picA is referred to as the cross-layer refresh point access unit. Otherwise, the first access unit in decoding order order with picture that has PicOrderCntVal greater than the PicOrderCntVal of the current picture plus the value of recovery_poc_cnt and nuh_layer_id equal to 0 is referred to as the cross-layer refresh point access unit. The cross-layer refresh point access unit shall not precede the current access unit in decoding order. All decoded access units that precede the cross-layer refresh point access unit in decoding order are specified to use cross_layer_bit_rate_value_minus1[ i ][ j ][ k ] and cross_layer_cpb_size_value_minus1[ i ][ j ][ k ] when NoClRasPicPresentFlag associated with the current initial IRAP AU is equal to 0. The value of applicable_poc_cnt shall be in the range of −231 to 231 – 1, inclusive.

cross_layer_bit_rate_value_minus1[ i ][ j ][ k ] (together with bit_rate_scale) specifies the maximum input bit rate for the i-th CPB when the CPB operates at the access unit level. cross_layer_bit_rate_value_minus1[ i ][ j ][ k ] shall be in the range of 0 to 232 − 2, inclusive. For any k > 0, cross_layer_bit_rate_value_minus1[ i ][ j ][ k ] shall be greater than cross_layer_bit_rate_value_minus1[ i ][ j ][ k - 1 ].

When SubPicHrdFlag is equal to 0, the bit rate in bits per second is given by:

CrossLayerBitRate[ i ] = ( cross_layer_bit_rate_value_minus1[ i ][ j ][ k ] + 1 ) * 2( 6 + bit_rate_scale )	(E-3)

When SubPicHrdFlag is equal to 0 and the cross_layer_bit_rate_value_minus1[ i ] syntax element is not present, the value of BitRate[ i ] is inferred to be equal to CpbBrVclFactor * MaxBR for VCL HRD parameters and to be equal to CpbBrNalFactor * MaxBR for NAL HRD parameters, where MaxBR, CpbBrVclFactor and CpbBrNalFactor are specified in subclause Error: Reference source not found.

cross_layer_cpb_size_value_minus1[ i ][ j ][ k ] is used together with cross_layer_cpb_size_scale to specify the i-th CPB size when the CPB operates at the access unit level. cross_layer_cpb_size_value_minus1[ i ][ j ][ k ] shall be in the range of 0 to 232 − 2, inclusive. For any k greater than 0, cross_layer_cpb_size_value_minus1[ i ][ j ][ k ] shall be less than or equal to cross_layer_cpb_size_value_minus1[ i ][ j ][ k − 1 ].

When SubPicHrdFlag is equal to 0, the CPB size in bits is given by:

CrossLayerCpbSize[ i ] = (cross_layer_cpb_size_value_minus1[ i ][ j ][ k ] + 1 ) * 2( 4 + cpb_size_scale )	(E-4)

When SubPicHrdFlag is equal to 0 and the cross_layer_cpb_size_value_minus1[ i ] syntax element is not present, the value of CpbSize[ i ] is inferred to be equal to CpbBrVclFactor * MaxCPB for VCL HRD parameters and to be equal to CpbBrNalFactor * MaxCPB for NAL HRD parameters, where MaxCPB, CpbBrVclFactor and CpbBrNalFactor are specified in subclause Error: Reference source not found.

## Changes to Annex C Section C.1

C.1	General

This annex specifies the hypothetical reference decoder (HRD) and its use to check bitstream and decoder conformance.

Two types of bitstreams or bitstream subsets are subject to HRD conformance checking for this Specification. The first type, called a Type I bitstream, is a NAL unit stream containing only the VCL NAL units and NAL units with nal_unit_type equal to FD_NUT (filler data NAL units) for all access units in the bitstream. The second type, called a Type II bitstream, contains, in addition to the VCL NAL units and filler data NAL units for all access units in the bitstream, at least one of the following:

–	additional non-VCL NAL units other than filler data NAL units,

–	all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes, and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex Error: Reference source not found).

Figure C-1 shows the types of bitstream conformance points checked by the HRD.

Figure C-1 – Structure of byte streams and NAL unit streams for HRD conformance checks

The syntax elements of non-VCL NAL units (or their default values for some of the syntax elements), required for the HRD, are specified in the semantic subclauses of clause Error: Reference source not found, Annexes Error: Reference source not found and Error: Reference source not found.

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled through the hrd_parameters( ) syntax structure, which may be part of the SPS syntax structure or the VPS syntax structure.

Multiple tests may be needed for checking the conformance of a bitstream, which is referred to as the bitstream under test. For each test, the following steps apply in the order listed:

- An operation point under test, denoted as TargetOp, is selected. The layer identifier list OpLayerIdList of TargetOp consists of the list of nuh_layer_id values, in increasing order of nuh_layer_id values, present in the bitstream subset associated with TargetOp, which is a subset of the nuh_layer_id values present in the bitstream under test. The OpTid of TargetOp is equal to the highest TemporalId present in the bitstream subset associated with TargetOp.

- TargetDecLayerIdList is set equal to OpLayerIdList of TargetOp, HighestTid is set equal to OpTid of TargetOp, and the sub-bitstream extraction process as specified in clause Error: Reference source not found is invoked with the bitstream under test, HighestTid, and TargetDecLayerIdList as inputs, and the output is assigned to BitstreamToDecode.

- The hrd_parameters( ) syntax structure and the sub_layer_hrd_parameters( ) syntax structure applicable to TargetOp are selected. If TargetDecLayerIdList contains all nuh_layer_id values present in the bitstream under test, the hrd_parameters( ) syntax structure in the active SPS (or provided through an external means not specified in this Specification) is selected. Otherwise, the hrd_parameters( ) syntax structure in the active VPS (or provided through some external means not specified in this Specification) that applies to TargetOp is selected. Within the selected hrd_parameters( ) syntax structure, if BitstreamToDecode is a Type I bitstream, the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( vcl_hrd_parameters_present_flag )" is selected and the variable NalHrdModeFlag is set equal to 0; otherwise (BitstreamToDecode is a Type II bitstream), the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows either the condition "if( vcl_hrd_parameters_present_flag )" (in this case the variable NalHrdModeFlag is set equal to 0) or the condition "if( nal_hrd_parameters_present_flag )" (in this case the variable NalHrdModeFlag is set equal to 1) is selected. When BitstreamToDecode is a Type II bitstream and NalHrdModeFlag is equal to 0, all non-VCL NAL units except filler data NAL units, and all  leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes, and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex Error: Reference source not found), when present, are discarded from BitstreamToDecode, and the remaining bitstream is assigned to BitstreamToDecode.

- An access unit associated with a buffering period SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) applicable to TargetOp is selected as the HRD initialization point and referred to as access unit 0.

- For each access unit in BitstreamToDecode starting from access unit 0, the buffering period SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the access unit and applies to TargetOp is selected, the picture timing SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the access unit and applies to TargetOp is selected, and when SubPicHrdFlag is equal to 1 and sub_pic_cpb_params_in_pic_timing_sei_flag is equal to 0, the decoding unit information SEI messages (present in BitstreamToDecode or available through external means not specified in this Specification) that are associated with decoding units in the access unit and apply to TargetOp are selected.

- A value of SchedSelIdx is selected. The selected SchedSelIdx shall be in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where cpb_cnt_minus1[ HighestTid ] is found in the sub_layer_hrd_parameters( HighestTid ) syntax structure as selected above.

- When the coded picture in access unit 0 has nal_unit_type equal to CRA_NUT or BLA_W_LP, and irap_cpb_params_present_flag in the selected buffering period SEI message is equal to 1, either of the following applies for selection of the initial CPB removal delay and delay offset:

–	If NalHrdModeFlag is equal to 1, the default initial CPB removal delay and delay offset represented by nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. Otherwise, the default initial CPB removal delay and delay offset represented by vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. The variable DefaultInitCpbParamsFlag is set equal to 1.

–	If NalHrdModeFlag is equal to 1, the alternative initial CPB removal delay and delay offset represented by nal_initial_alt_cpb_removal_delay[ SchedSelIdx ] and nal_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. Otherwise, the alternative initial CPB removal delay and delay offset represented by vcl_initial_alt_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. The variable DefaultInitCpbParamsFlag is set equal to 0, and the RASL access units associated with access unit 0 are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

- When NoClRasPicPresentFlag is equal to 1 and access unit 0 is associated with a cross-layer HRD parameters SEI message, the values of BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ] are set equal to CrossLayerBitRate[ SchedSelIdx ] and CrossLayerCpbSize[ SchedSelIdx ], respectively.

- When sub_pic_hrd_params_present_flag in the selected hrd_parameters( ) syntax structure is equal to 1, the CPB is scheduled to operate either at the access unit level (in which case the variable SubPicHrdFlag is set equal to 0) or at the sub-picture level (in which case the variable SubPicHrdFlag is set equal to 1).

For each operation point under test, the number of bitstream conformance tests to be performed is equal to n0 * n1 * ( n2 * 2 + n3 ) * n4, where the values of n0, n1, n2, n3, and n4 are specified as follows:

–	n0 is derived as follows:

–	If BitstreamToDecode is a Type I bitstream, n0 is equal to 1.

–	Otherwise (BitstreamToDecode is a Type II bitstream), n0 is equal to 2.

–	n1 is equal to cpb_cnt_minus1[ HighestTid ] + 1.

–	n2 is the number of access units in BitstreamToDecode that each is associated with a buffering period SEI message applicable to TargetOp and for each of which both of the following conditions are true:

–	nal_unit_type is equal to CRA_NUT or BLA_W_LP for the VCL NAL units;

–	The associated buffering period SEI message applicable to TargetOp has irap_cpb_params_present_flag equal to 1.

–	n3 is the number of access units in BitstreamToDecode BitstreamToDecode that each is associated with a buffering period SEI message applicable to TargetOp and for each of which one or both of the following conditions are true:

–	nal_unit_type is equal to neither CRA_NUT nor BLA_W_LP for the VCL NAL units;

–	The associated buffering period SEI message applicable to TargetOp has irap_cpb_params_present_flag equal to 0.

–	n4 is derived as follows:

–	If sub_pic_hrd_params_present_flag in the selected hrd_parameters( ) syntax structure is equal to 0, n4 is equal to 1;

–	Otherwise, n4 is equal to 2.

When BitstreamToDecode is a Type II bitstream, the following applies:

–	If the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( vcl_hrd_parameters_present_flag )" is selected, the test is conducted at the Type I conformance point shown in Figure C-1, and only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

–	Otherwise (the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( nal_hrd_parameters_present_flag )" is selected), the test is conducted at the Type II conformance point shown in Figure C-1, and all bytes of the Type II bitstream, which may be a NAL unit stream or a byte stream, are counted for the input bit rate and CPB storage.

NOTE 1 – NAL HRD parameters established by a value of SchedSelIdx for the Type II conformance point shown in Figure C-1 are sufficient to also establish VCL HRD conformance for the Type I conformance point shown in Figure C-1 for the same values of InitCpbRemovalDelay[ SchedSelIdx ], BitRate[ SchedSelIdx ] (or CrossLayerBitRate[ SchedSelIdx ] when applicable), and CpbSize[ SchedSelIdx ] (or CrossLayerBitRate[ SchedSelIdx ] when applicable), for the VBR case (cbr_flag[ SchedSelIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive. For example, when decoding a CVS conforming to one or more of the profiles specified in Annex Error: Reference source not found using the decoding process specified in clauses Error: Reference source not found through Error: Reference source not found, when NAL HRD parameters are provided for the Type II conformance point that not only fall within the bounds set for NAL HRD parameters for profile conformance in item Error: Reference source not found of subclause Error: Reference source not found but also fall within the bounds set for VCL HRD parameters for profile conformance in item Error: Reference source not found of subclause Error: Reference source not found, conformance of the VCL HRD for the Type I conformance point is also assured to fall within the bounds of item Error: Reference source not found of subclause Error: Reference source not found.

All VPSs, SPSs and PPSs referred to in the VCL NAL units, and the corresponding buffering period, picture timing and decoding unit information SEI messages shall be conveyed to the HRD, in a timely manner, either in the bitstream (by non-VCL NAL units), or by other means not specified in this Specification.

In Annexes Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found, the specification for "presence" of non-VCL NAL units that contain VPSs, SPSs, PPSs, buffering period SEI messages, picture timing SEI messages, or decoding unit information SEI messages is also satisfied when those NAL units (or just some of them) are conveyed to decoders (or to the HRD) by other means not specified in this Specification. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

NOTE 2 – As an example, synchronization of such a non-VCL NAL unit, conveyed by means other than presence in the bitstream, with the NAL units that are present in the bitstream, can be achieved by indicating two points in the bitstream, between which the non-VCL NAL unit would have been present in the bitstream, had the encoder decided to convey it in the bitstream.

When the content of such a non-VCL NAL unit is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the non-VCL NAL unit is not required to use the same syntax as specified in this Specification.

NOTE 3 – When HRD information is contained within the bitstream, it is possible to verify the conformance of a bitstream to the requirements of this subclause based solely on information contained in the bitstream. When the HRD information is not present in the bitstream, as is the case for all "stand-alone" Type I bitstreams, conformance can only be verified when the HRD data are supplied by some other means not specified in this Specification.

The HRD contains a coded picture buffer (CPB), an instantaneous decoding process, a decoded picture buffer (DPB), and output cropping as shown in Figure C-2.

Figure C-2 – HRD buffer model

For each bitstream conformance test, the CPB size (number of bits) is CpbSize[ SchedSelIdx ] as specified in subclause Error: Reference source not found, where SchedSelIdx and the HRD parameters are specified above in this subclause. The DPB size (number of picture storage buffers) is sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1.

The variable SubPicHrdPreferredFlag is either specified by external means, or when not specified by external means, set equal to 0.

When the value of the variable SubPicHrdFlag has not been set by step 8 above in this subclause, it is derived as follows:

SubPicHrdFlag = SubPicHrdPreferredFlag  &&  sub_pic_hrd_params_present_flag	(C-1)

If SubPicHrdFlag is equal to 0, the HRD operates at access unit level and each decoding unit is an access unit. Otherwise the HRD operates at sub-picture level and each decoding unit is a subset of an access unit.

NOTE 4 – If the HRD operates at access unit level, each time a decoding unit that is an entire access unit is removed from the CPB. Otherwise (the HRD operates at sub-picture level), each time a decoding unit that is a subset of an access unit is removed from the CPB. In both cases, each time an entire decoded picture is output from the DPB, though the picture output time is derived based on the differently derived CPB removal times and the differently signalled DPB output delays.

The following is specified for expressing the constraints in this annex:

–	Each access unit is referred to as access unit n, where the number n identifies the particular access unit. Access unit 0 is selected per step 4 above. The value of n is incremented by 1 for each subsequent access unit in decoding order.

–	Each decoding unit is referred to as decoding unit m, where the number m identifies the particular decoding unit. The first decoding unit in decoding order in access unit 0 is referred to as decoding unit 0. The value of m is incremented by 1 for each subsequent decoding unit in decoding order.

NOTE 5 – The numbering of decoding units is relative to the first decoding unit in access unit 0.

–	Picture n refers to the coded picture or the decoded picture of access unit n.

The HRD operates as follows:

–	The HRD is initialized at decoding unit 0, with the both the CPB and the DPB being set to be empty (the DPB fullness is set equal to 0).

NOTE 6 – After initialization, the HRD is not initialized again by subsequent buffering period SEI messages.

–	Data associated with decoding units that flow into the CPB according to a specified arrival schedule are delivered by the HSS.

–	The data associated with each decoding unit are removed and decoded instantaneously by the instantaneous decoding process at the CPB removal time of the decoding unit.

–	Each decoded picture is placed in the DPB.

–	A decoded picture is removed from the DPB when it becomes no longer needed for inter prediction reference and no longer needed for output.

For each bitstream conformance test, the operation of the CPB is specified in subclause Error: Reference source not found, the instantaneous decoder operation is specified in clauses Error: Reference source not found through Error: Reference source not found, the operation of the DPB is specified in subclause Error: Reference source not found, and the output cropping is specified in subclause  and subclause Error: Reference source not found.

HSS and HRD information concerning the number of enumerated delivery schedules and their associated bit rates and buffer sizes is specified in subclauses Error: Reference source not found and Error: Reference source not found. The HRD is initialized as specified by the buffering period SEI message specified in subclauses Error: Reference source not found and Error: Reference source not found. The removal timing of decoding units from the CPB and output timing of decoded pictures from the DPB is specified using information in picture timing SEI messages (specified in subclauses Error: Reference source not found and Error: Reference source not found) or in decoding unit information SEI messages (specified in subclauses Error: Reference source not found and Error: Reference source not found). All timing information relating to a specific decoding unit shall arrive prior to the CPB removal time of the decoding unit.

The requirements for bitstream conformance are specified in subclause Error: Reference source not found, and the HRD is used to check conformance of bitstreams as specified above in this subclause and  to check conformance of decoders as specified in subclause Error: Reference source not found.

NOTE 7 – While conformance is guaranteed under the assumption that all picture-rates and clocks used to generate the bitstream match exactly the values signalled in the bitstream, in a real system each of these may vary from the signalled or specified value.

All the arithmetic in this annex is performed with real values, so that no rounding errors can propagate. For example, the number of bits in a CPB just prior to or after removal of a decoding unit is not necessarily an integer.

The variable ClockTick is derived as follows and is called a clock tick:

ClockTick = vui_num_units_in_tick  vui_time_scale	(C-2)

The variable ClockSubTick is derived as follows and is called a clock sub-tick:

ClockSubTick = ClockTick  ( tick_divisor_minus2 + 2 )

## Changes to the Annex C Section C.2.2

C.2.2	Timing of decoding unit arrival

- If SubPicHrdFlag is equal to 0, the variable subPicParamsFlag is set equal to 0, and the process in specified in the remainder of this subclause is invoked with a decoding unit being considered as an access unit, for derivation of the initial and final CPB arrival times for access unit n.

- Otherwise (SubPicHrdFlag is equal to 1), the process in specified in the remainder of this subclause is first invoked with the variable subPicParamsFlag set equal to 0 and a decoding unit being considered as an access unit, for derivation of the initial and final CPB arrival times for access unit n, and then invoked with subPicParamsFlag set equal to 1 and a decoding unit being considered as a subset of an access unit, for derivation of the initial and final CPB arrival times for the decoding units in access unit n.

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are derived as follows:

–	If one or more of the following conditions are true, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_alt_cpb_removal_delay[ SchedSelIdx ] and nal_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_alt_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message syntax elements are selected as specified in subclause :

–	Access unit 0 is a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_RADL or BLA_N_LP, and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1.

–	Access unit 0 is a BLA access unit for which the coded picture has nal_unit_type equal to BLA_W_LP or is a CRA access unit, and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1, and one or more of the following conditions are true:

–	UseAltCpbParamsFlag for access unit 0 is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	The value of subPicParamsFlag is equal to 1.

–	Otherwise, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message syntax elements are selected as specified in subclause .

The time at which the first bit of decoding unit m begins to enter the CPB is referred to as the initial arrival time initArrivalTime[ m ].

The initial arrival time of decoding unit m is derived as follows:

–	If the decoding unit is decoding unit 0 (i.e. m = 0), initArrivalTime[ 0 ] = 0,

–	Otherwise (the decoding unit is decoding unit m with m > 0), the following applies:

–	If cbr_flag[ SchedSelIdx ] is equal to 1, the initial arrival time for decoding unit m is equal to the final arrival time (which is derived below) of decoding unit m − 1, i.e.

if( !subPicParamsFlag )
	initArrivalTime[ m ] = AuFinalArrivalTime[ m − 1 ]	(C-3)
else
	initArrivalTime[ m ] = DuFinalArrivalTime[ m − 1 ]

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 0), the initial arrival time for decoding unit m is derived as follows:

if( !subPicParamsFlag )
	initArrivalTime[ m ] = Max( AuFinalArrivalTime[ m − 1 ], initArrivalEarliestTime[ m ] )	(C-4)
else
	initArrivalTime[ m ] = Max( DuFinalArrivalTime[ m − 1 ], initArrivalEarliestTime[ m ] )

where initArrivalEarliestTime[ m ] is derived as follows:

–	The variable tmpNominalRemovalTime is derived as follows:

if( !subPicParamsFlag )
	tmpNominalRemovalTime = AuNominalRemovalTime[ m ] 	(C-5)
else
	tmpNominalRemovalTime = DuNominalRemovalTime[ m ]

where AuNominalRemovalTime[ m ] and DuNominalRemovalTime[ m ] are the nominal CPB removal time of access unit m and decoding unit m, respectively, as specified in subclause Error: Reference source not found.

–	If decoding unit m is not the first decoding unit of a subsequent buffering period, initArrivalEarliestTime[ m ] is derived as follows:

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime − ( InitCpbRemovalDelay[ SchedSelIdx ]
		+ InitCpbRemovalDelayOffset[ SchedSelIdx ] )  90000	(C-6)

–	Otherwise (decoding unit m is the first decoding unit of a subsequent buffering period), initArrivalEarliestTime[ m ] is derived as follows:

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime −
		( InitCpbRemovalDelay[ SchedSelIdx ]  90000 )	(C-7)

The final arrival time for decoding unit m is derived as follows:

if( !subPicParamsFlag )
		AuFinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ]  BitRate[ SchedSelIdx ]	(C-8)
else
		DuFinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ]  BitRate[ SchedSelIdx ]

- where sizeInbits[ m ] is the size in bits of decoding unit m, counting the bits of the VCL NAL units and the filler data NAL units for the Type I conformance point or all bits of the Type II bitstream for the Type II conformance point, where the Type I and Type II conformance points are as shown in Figure C-1.

The values of SchedSelIdx, BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] are constrained as follows:

–	If the content of the selected hrd_parameters( ) or the cross-layer HRD parameters SEI messaage, when applicable) syntax structures for the access unit containing decoding unit m and the previous access unit differ, the HSS selects a value SchedSelIdx1 of SchedSelIdx from among the values of SchedSelIdx provided in the selected hrd_parameters( ) syntax structures (or the cross-layer HRD parameters SEI messaage, when applicable)for the access unit containing decoding unit m that results in a BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] for the access unit containing decoding unit m. The value of BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] may differ from the value of BitRate[ SchedSelIdx0 ] or CpbSize[ SchedSelIdx0 ] for the value SchedSelIdx0 of SchedSelIdx that was in use for the previous access unit.

–	Otherwise, the HSS continues to operate with the previous values of SchedSelIdx, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ].

When the HSS selects values of BitRate[ SchedSelIdx ] or CpbSize[ SchedSelIdx ] that differ from those of the previous access unit, the following applies:

–	The variable BitRate[ SchedSelIdx ] comes into effect at the initial CPB arrival time of the current access unit.

–	The variable CpbSize[ SchedSelIdx ] comes into effect as follows:

–	If the new value of CpbSize[ SchedSelIdx ] is greater than the old CPB size, it comes into effect at the initial CPB arrival time of the current access unit.

–	Otherwise, the new value of CpbSize[ SchedSelIdx ] comes into effect at the CPB removal time of the current access unit.

## Changes to Annex C Section C.3.3

C.3.3	Picture output

The processes specified in this subclause happen instantaneously at the CPB removal time of access unit n, AuCpbRemovalTime[ n ].

When picture n has PicOutputFlag equal to 1, its DPB output time DpbOutputTime[ n ] is derived as follows, where the variable firstPicInBufferingPeriodFlag is equal to 1 if access unit n is the first access unit of a buffering period and 0 otherwise:

if( !SubPicHrdFlag ) {
	DpbOutputTime[ n ] = AuCpbRemovalTime[ n ] + ClockTick * picDpbOutputDelay	(C-9)
	if( firstPicInBufferingPeriodFlag )
		DpbOutputTime[ n ]  −=  ClockTick * DpbDelayOffset
} else
	DpbOutputTime[ n ] = AuCpbRemovalTime[ n ] + ClockSubTick * picSptDpbOutputDuDelay

where picDpbOutputDelay is the value of pic_dpb_output_delay in the picture timing SEI message associated with access unit n, and picSptDpbOutputDuDelay is the value of pic_spt_dpb_output_du_delay, when present, in the decoding unit information SEI messages associated with access unit n, or the value of pic_dpb_output_du_delay in the picture timing SEI message associated with access unit n when there is no decoding unit information SEI message associated with access unit n or no decoding unit information SEI message associated with access unit n has pic_spt_dpb_output_du_delay present. The value of picDpbOutputDelay is set as follows:

- If either of the following conditions is true, picDpbOutputDelay is chosen as the value of pic_alt_dpb_output_delay in the picture timing SEI message associated with access unit n.

  - the preceding initial IRAP AU in decoding order has NoClRasPicPresentFlag equal to 1 and irap_cross_layer_cpb_params_present_flag is equal to 1.

  - the current AU is an initial IRAP AU that has NoClRasPicPresentFlag equal to 1 and irap_cross_layer_cpb_params_present_flag is equal to 1.

- Otherwise, picDpbOutputDelay is the value of pic_alt_dpb_output_delay in the picture timing SEI message associated with access unit n.

NOTE – When the syntax element pic_spt_dpb_output_du_delay is not present in any decoding unit information SEI message associated with access unit n, the value is inferred to be equal to pic_dpb_output_du_delay in the picture timing SEI message associated with access unit n.

The output of the current picture is specified as follows:

–	If PicOutputFlag is equal to 1 and DpbOutputTime[ n ] is equal to AuCpbRemovalTime[ n ], the current picture is output.

–	Otherwise, if PicOutputFlag is equal to 0, the current picture is not output, but will be stored in the DPB as specified in subclause Error: Reference source not found.

–	Otherwise (PicOutputFlag is equal to 1 and DpbOutputTime[ n ] is greater than AuCpbRemovalTime[ n ] ), the current picture is output later and will be stored in the DPB (as specified in subclause Error: Reference source not found) and is output at time DpbOutputTime[ n ] unless indicated not to be output by the decoding or inference of no_output_of_prior_pics_flag equal to 1 at a time that precedes DpbOutputTime[ n ].

When output, the picture is cropped, using the conformance cropping window specified in the active SPS for the picture.

When picture n is a picture that is output and is not the last picture of the bitstream that is output, the value of the variable DpbOutputInterval[ n ] is derived as follows:

DpbOutputInterval[ n ] = DpbOutputTime[ nextPicInOutputOrder ] − DpbOutputTime[ n ]	(C-10)

where nextPicInOutputOrder is the picture that follows picture n in output order and has PicOutputFlag equal to 1.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).