| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0215 |
| --- | --- |

| Title: | On phase alignment of up-sampling process in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Liwei Guo Xiang Li Marta Karczewicz Shufei Fan 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8028 cjianle@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes signaling a flag to indicate phase alignment between reference layer picture and enhancement layer picture in down-sampling process and accordingly a matched up-sampling is used in decoding process. With the proposed method, SHVC can take both zero position aligned sequences and central position aligned sequences as input. It is reported that for central position aligned sequences, the proposed method can achieves −8.8%, −6.3% and −5.4% luma bit rate saving, respectively for AI, RA and LD-B configurations.

# Introduction

In spatial scalable video coding, base layer (BL) pictures are down-sampled versions of enhancement layer (EL) pictures. As a result, up-sampling needs to be applied to BL layer pictures for inter-layer texture prediction. In the current SHVC draft text Error: Reference source not found, up-sampling for inter-layer prediction is designed based on an assumption that the sampling grids of BL and EL pictures are aligned at the zero position, as shown in Figure 1.(a). However, this assumption does not always hold, because the down-sampling process is not part of the standard and different systems may do it in different ways. When input BL and EL sequences are aligned at a non-zero position, using the up-sampling process defined in the current SHVC will cause inaccurate inter layer prediction and thus loss of spatial scalable coding performance.

It’s worth-noting that, in image/video scaling algorithm, the original picture and scaled picture are usually aligned at central position, as shown in Figure 1.(b). This scheme is widely used as industrial convention, such as in scaling module in the camera and display products. Most commercial software such as Matlab and Photoshop also use such sample alignment in picture scaling module. Therefore, it is highly desired that SHVC codec can take such sequences as BL and EL input so that the SHVC codec can employ the down-sampler in existing commercial products or use existing down-sampled sequence as input.

(a) Zero position aligned 			(b) Central position aligned

Figure 1 Sample location of reference layer and enhancement layer

# Indication of phase alignment

Based SCE1 results of 14th JCT-VC meeting, it was shown that the current SHVC CTC (with zero position aligned input) achieves slight better coding performance (0.3% bit rate gain) than with central position aligned input. On the other hand, central position aligned scheme is an industrial convention and supporting such scheme enables SHVC to work  with existing down-sampling modules in industrial products. Therefore it is desirable that SHVC can support both phase alignment scheme to take both advantages.

In this contribution, a flag is transmitted in VPS extension to indicate which sample position alignment scheme is used in the generation of reference layer picture. In the up-sampling process, accordingly a matched reference layer sample location (phase) derivation process is used.

# Experiment results

Sequences with two different down-sampled base layer versions are used in the experiment: Sequences with zero-position aligned (i.e. sequences in SHVC CTC) and sequences with central position aligned [1]. Other simulation settings follow common test conditions [2].

In the first experiment, sequences with central position aligned are used as input. The anchor is generated using the current SHVC software SHM-v3.0.1 where the unmatched phase derivation method is used. With the help of the signaling of phase alignment, the proposed method can select the matched up-sampling phase. It is shown in Table 1 that it achieves, averagely, 4.8% ~ 9.4% luma BD-rate reduction.

Table 1 Performance comparison between proposed method and SHVC for central position aligned input

In the second experiment, two different down-sampled base layer inputs are used in the experiment. Table 2 shows that, when the matched up-sampling process is applied, the zero position aligned input has, averagely, 0.2% ~ 0.4% luma bit rate gain compared to the central position aligned input.

Table 2 Performance comparison between zero position aligned and central position aligned input

# Conclusion

This contribution proposes signaling a flag to indicate phase alignment in down-sampling process and accordingly a matched up-sampling is used in decoding process. This method provides the advantages that the SHVC can use 1) the existing down-sampling modules widely employed in the commercial product in SHVC encoder and 2) the input resource generated by the widely used down-sampling modules. It is also observed that the additional complexity to support the proposed scheme is negligible. Considering the useful functionality and potential complexity reduction at encoder side, we recommend adopting the proposed technique into SHVC draft.

# References

- X. Li, “SCE1: Summary report of SHVC Core Experiment on support for additional re-sampling phase shifts” ”, JCTVC-N0031, Vienna, Austria, Jul. 2013.

- J. Dong, Y. He, and Y. Ye, "Downsampling filter for anchor generation for scalable extensions of HEVC," m23485, 99th MPEG meeting, San Jose, USA, Feb. 2012.

- X. Li, J. Boyce, P. Onno, and Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-N1009, Vienna, Austria, Jul. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Modification of specification text

Change in VPS extension syntax and semantics

| VPS_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| cross_layer_phase_alignment_flag | u(1) |
| … |  |
| } |  |

cross_layer_phase_alignment_flag equal to 1 specifies the locations of the luma sample grids are cross-layer aligned at central position. cross_layer_phase_alignment_flag equal to 0 specifies luma sample grids are cross-layer aligned at the top-left sample position.

Change H.6.2 Derivation process for reference layer sample location used in resampling

Inputs to this process are

–	a variable cIdx specifying the color component index,

–	a sample location ( xP, yP ) relative to the top-left sample of the color component of the current picture specified by cIdx.

Output of this process is a sample location ( xRef16, yRef16 ) specifying the reference layer sample location in units of 1/16-th sample relative to the top-left sample of the reference layer picture.

The variables offsetX and offsetY are derived as follows:

offsetX = ScaledRefLayerLeftOffset / ( ( cIdx = = 0)  ?  1 :  SubWidthC)	(H-1)
offsetY = ScaledRefLayerTopOffset / ( ( cIdx = = 0)  ?  1 :  SubHeightC)	(H-2)

The variables phaseY and addY are derived as follows:

phaseX = (cIdx = = 0) ? (cross_layer_phase_alignment_flag <<1) : cross_layer_phase_alignment_flag	(H-3)
phaseY = (cIdx = = 0) ? (cross_layer_phase_alignment_flag <<1) :cross_layer_phase_alignment_flag + 1	(H-6)

addX = ( ScaleFactorX * phaseX + 2 ) >> 2		(H-7)
addY = ( ScaleFactorY * phaseY + 2 ) >> 2		(H-8)

The variables xRef16 and yRef16 are derived as follows:

xRef16 = ( ( ( xP – offsetX ) * ScaleFactorX + addX + ( 1 << 11 ) ) >> 12 )  – ( phaseX << 2 )	(H-8)
yRef16 = ( ( ( yP – offsetY ) * ScaleFactorY + addY + ( 1 << 11 ) ) >> 12 ) – ( phaseY << 2 )	(H-9)