| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0216_v3 |
| --- | --- |

| Title: | On slice level information derivation and motion field mapping for resampled interlayer reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Vadim Seregin Xiang Li Krishna Rapaka Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-651-8028 cjianle@qti.qualcomm.com vseregin@qti.qualcomm.com lxiang@qti.qualcomm.com krapaka@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reported that inter-layer motion prediction in the current SHVC does not work properly when there are multiple slices in the reference layer picture. Two solutions are proposed to solve the issue.

- Solution 1: generate one single slice for the resampled picture and copy the slice type and reference picture lists information from the first slice of the reference layer picture; in addition, impose a bit-stream conformance constraint that, when there are multiple slices in the reference layer picture and those slices have different slice type or reference picture lists, the inter-layer motion prediction from that reference layer picture is disallowed for the current particular picture.

- Solution 2: generate a single slice for the resampled picture with new reference picture lists to include all the reference pictures derived from the corresponding reference picture lists of all slices in the reference layer picture. During motion mapping, reference indices of the reference blocks are adjusted to be referring to the reference picture with the same POC as in the reference layer picture.

Both methods solve the reported issue of motion field mapping in multiple slice cases with none or negligible additional complexity. It’s recommended to adopt one of them in the next version of SHVC draft and reference software.

# Introduction and Problem Statement

In SHVC, inter-layer motion prediction is enabled by using the resampled reference layer picture as the collocated picture for TMVP derivation. The block level motion data of the re-sampled picture which need to be used in TMVP derivation is generated by motion field mapping process. In addition to motion data information, the TMVP derivation process also needs some high level information of the collocated picture, including picture order count, slice type and reference picture lists. To enable inter-layer from resampled interlayer reference picture; those information of resampled interlayer reference picture shall also be appropriately derived.

Slice type and reference picture list information is carried in slice level rather picture level. In case of multiple slices existing in the reference layer picture, the slice partition is also needed to properly derive the slice level information of the re-sampled picture. As shown in figure 1, the slice partition can be derived by up-scaling the slice partition map of the reference layer picture. However the up-scaled slice partition boundary may cross the enhancement layer CTB which breaks the rules of HEVC slice definition. In this sense, the current SHVC would have problem to enable inter-layer motion prediction when there are multiple slice in the reference layer picture.

This contribution proposes two solutions to solve aforementioned problem in the current SHVC draft.

Figure 1. Slice mapping process for spatial scalability with 1.5x ratio

# Proposed methods

In this proposal, two methods that can solve the aforementioned problem are proposed.

## Solution 1:

In this solution, a simplified slice level information derivation method is proposed. In this method, only one slice is generated for the whole re-sampled picture, the slice level information, including slice_type and reference picture lists are set equal to those of the first slice in the corresponding reference picture.

The method is mostly applicable when there are multiple slices in the reference layer picture. If slice type and reference picture list of all slices in the corresponding reference layer picture are identical, there is no need to have slice partition mapping process. Instead, the whole resampled interlayer picture can be associated to a single slice and the slice level information including slice_type and reference picture list can be simply set equal to those of any slice in the corresponding reference picture.

While this method would not work when the slices in reference layer picture have different slice type or reference picture lists. Therefore, to avoid abovementioned slice partition mapping problem, an encoder restriction is imposed that, when there are multiple slices in the reference layer picture and those slices have different slice type or reference picture lists, the inter-layer motion prediction is disallowed for the current particular picture.

## Solution 2:

In the second solution, to avoid constraints of the first method imposed on the slice type and reference picture lists to be identical, one slice is derived and assigned to the entire resampled picture regardless of the reference layer picture slice partitioning. Slice type of the resampled picture slice is set equal to B slice if there is at least one B slice in the reference layer picture; otherwise it is set equal to P slice if there is at least one P slice in the reference layer picture; otherwise it is set equal to I slice. The method can be divided into the following steps:

- Create reference picture lists

Each reference picture list (list 0 or list 1) in the resampled picture slice is derived by including all the reference pictures from the corresponding reference picture list of every slice in the reference layer picture. Starting from the first slice and going to the last slice of the reference layer picture, reference picture with POC is included into the corresponding reference picture list of the resampled picture slice if the there is no picture with the same POC already exists in the reference picture list. Figure 2 shows the example of slice type and reference pictures picture derivation for the resampled picture where the reference layer picture has three slices.

Figure 2 reference picture lists derivation

- Adjust reference index during motion field mapping process

During motion mapping, when the reference index of 16x16 reference block is going to be assigned to the corresponding block of the resampled picture, that reference index associated with the reference picture POC is converted first to the reference index corresponding to the reference picture with the same POC in the reference picture list of the resampled picture slice, created in the first step.

# Conclusion

This contribution reported the issue of inter-layer motion prediction in the current SHVC for the case of multiple slices in the reference layer picture. Two solutions are proposed to solve the issue and both methods solved the reported issue of motion field mapping in multiple slice cases with none or negligible additional complexity. It’s recommended to adopt one of them in the next version of SHVC draft and reference software.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Specification text

Modifications of spec text are highlighted in yellow.

## Method 1

Change in H.8.1.4	Resampling process for inter layer reference pictures

–	otherwise, rsPic is derived as follows:

  - The PicOrderCntVal value of rsPic is set equal to the PicOrderCntVal value of rlPic.

  - The variable rsSlice is defined as a slice associated to the rsPic and all coding tree blocks in rsPic are included in rsSlice. The value of segment header syntax elements slice_type, num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 of rsSlice are set equal to the value of slice_type, num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 of any particular slice segment header of rlPic, respectively. When rsSlice is a P or B slice, for i in the range of 0 to num_ref_idx_l0_active_minus1 of rsSlice, inclusive, reference picture with index i in reference picture list 0 of rsSlice is set equal to reference picture with index i in reference picture list 0 of any particular slice segment header of rlPic. When rsSlice is a B slice, for i in the range of 0 to num_ref_idx_l1_active_minus1 of rsSlice, inclusive, reference picture with index i in reference picture list 1 of rsSlice is set equal to reference picture with index i in reference picture list 1 of any particular slice segment header of rlPic.

  - When SamplePredEnabledFlag[ currLayerId ][ rLId ] is equal to 1, the picture sample resampling process as specified in subclause Error: Reference source not found is invoked with the sample arrays of reference layer picture rlPic as input, and with the sample arrays of resampled picture rsPic as output.

  - When MotionPredEnabledFlag[ currLayerId ][ rLId ] is equal to 1, the picture motion field resampling process as specified in subclause Error: Reference source not found is invoked with refereence layer picture rlPic and its motion field as inputs, and with the  motion field of resampled picture rsPic as output.

Change in H.11.1.2	Scalable Main profile

Bitstreams conforming to the scalable main profile shall obey the following constraints:

–	...

–	When the collocated picture for temporal motion vector prediction specified by collocated_ref_idx is an inter-layer picture that is derived by subclause H.8.1.4, the following constraints apply:

  - The slice segment header syntax elements slice type, num_ref_idx_l0_active_minus1 and num_ref_idx_l1_active_minus1 shall be the same in all slice segment headers of the corresponding reference layer picture, which is used as input of subclause H.8.1.4 to derive this resampled inter-layer reference picture.

  - When present, the variable RefPicList0[ i ] and RefPicList1[ i ] shall be the same in all slice segment headers of the corresponding reference layer picture, which is used as input of subclause H.8.1.4 to derive this resampled inter-layer reference picture for each possible value of i.

## Method 2

Change in H.8.1.4 Resampling process for inter layer reference pictures

…

The following steps are applied to derive the resampled inter layer reference picture rsPic.

–	if PicWidthInSamplesL is equal to RefLayerPicWidthInSamplesL and PicHeightInSamplesL is equal to RefLayerPicHeightInSamplesL and the values of ScaledRefLayerLeftOffset, ScaledRefLayerTopOffset, ScaledRefLayerRightOffset and ScaledRefLayerBottomOffset are all equal to 0

  - rsPic is set equal to rlPic.

–	otherwise, rsPic is derived as follows:

  - The PicOrderCntVal value of rsPic is set equal to the PicOrderCntVal value of rlPic.

  - The single slice rsSlice associated with the rsPic is derived to include all the coding tree units from the rsPic. Slice type and reference picture lists of rsSlice are derived as follows:

    - slice_type of rsSlice is set equal to B slice if there is at least one B slice in the reference layer picture rlPic; otherwise slice_type of rsSlice is set equal to P slice if there is at least one P slice in the reference layer picture rlPic; otherwise slice_type of rsSlice is set equal to I slice.

    - Reference picture list 0 rsRefPicList0 and reference picture list 1 rsRefPicList1 of rsSlice first emptied. When rsSlice is a P or B slice, rsRefPicList0 is derived by including reference pictures with unique POCs, in reference index order, from the reference picture list 0 of every slice, in order from the first slice to the last slice, in rlPic. A new reference picture is not added to rsRefPicList0 when a reference picture with the same POC value has been already included in rsRefPicList0. num_ref_idx_l0_active_minus1 of rsSlice is set equal to the number of pictures in the rsRefPicList0 minus 1.

    - When reSlice is a B slice, rsRefPicList1 is derived by including reference pictures with unique POCs, in reference index order, from the reference picture list 1 of every slice, in order from the first slice to the last slice, in rlPic. A new reference picture is not added to rsRefPicList1 when a reference picture with the same POC value has been already included in rsRefPicList1. num_ref_idx_l1_active_minus1 of rsSlice is set equal to the number of pictures in the rsRefPicList1 minus 1.

  - When SamplePredEnabledFlag[ currLayerId ][ rLId ] is equal to 1, the picture sample resampling process as specified in subclause H.8.1.4.1 is invoked with the sample arrays of reference layer picture rlPic as input, and with the sample arrays of resampled picture rsPic as output.

  - When MotionPredEnabledFlag[ currLayerId ][ rLId ] is equal to 1, the picture motion field resampling process as specified in subclause H.8.1.4.2 is invoked with refereence layer picture rlPic and its motion field as inputs, and with the  motion field of resampled picture rsPic as output.

Change in H.8.2.4.2.1 Derivation process for inter layer motion parameters

- …

- For each X = 0, 1, the following applies:

        - The variables refIdxLX[ xP ][ yP ] and predFlagLX[ xP ][ yP ] are derived as follows:

refIdxLX[ xP ][ yP ] = refIdxLXRL[ xRL ][ yRL ]	(H-1)

predFlagLX[ xP ][ yP ] = predFlagLXRL[ xRL ][ yRL ]	(H-49)

- If predFlagLX[ xP ][ yP ] is equal to 1, refIdxLX[ xP ][ yP ] is set equal to the reference index of the reference picture in the RefPicListX with the POC value equal to that of the reference picture specified by refIdxLXRL[ xRL ][ yRL ].

- Otherwise refIdxLX[ xP ][ yP ] = −1	(H-50)

…