| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0218 |
| --- | --- |

| Title: | Evaluation of Palette Mode Coding on HM-12.0+RExt-4.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Marta Karczewicz Joel Sole Rajan Joshi 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the 14-th JCTVC meeting, contribution JCTVC-N0249 proposed a palette-based coding for screen contents. In this contribution, the method described in N0249 has been implemented on top of the latest Range Extension software HM-12.0+RExt-4.1 and the performance has been provided.

# Introduction

In the 14-th JCTVC meeting (July/August, 2013, Vienna), a palette-based coding mode has been proposed in JCTVC-N0249. In this contribution, the method in N0249 has been implemented on top of HM-12.0+RExt-4.1 and the performance has been evaluated.

# Method Description

The following is a description of the palette-based coding in JCTVC-N0249. The palette-based coding is designed as a CU mode. The encoding process is as follows:

- Transmission of the palette: the palette size is first transmitted followed by the palette elements. The maximum palette size is set to 24.

- Transmission of pixel values: the pixels in the CU are encoded in a raster scan order. For each position, a flag is first transmitted to indicate whether the “run mode” or “copy above mode” is being used.

  - “Run mode”: In “run mode”, a palette index is first signaled followed by “palette_run” (e.g., M). No further information needs to be transmitted for the current position and the following M positions as they have the same palette index as signaled. The palette index (e.g., i) is shared by all three color components, which means that the reconstructed pixel values are (Y, U, V) = (paletteY[i], paletteU[i], paletteV[i]) (assuming the color space is YUV)

  - “Copy above mode”: In “copy above mode”, a value “copy_run” (e.g., N) is transmitted to indicate that for the following N positions (including the current one), the palette index is equal to the palette index of the one that is at the same location in the row above.

- Transmission of residue: the palette indices transmitted in Stage 2 are converted back to pixel values and used as the prediction. Residue information is transmitted using HEVC residue coding and is added to the prediction for the reconstruction.

# Experimental results

The proposed method is implemented on HM-12.0+RExt-4.1. AHG8 test conditions specified in [2] are used. Decoding time is measured on a homogenous cluster.

Table 1 Results of palette-based coding for AHG8 lossless coding

| Avg Bit-Rate Saving | AI-Main | RA-Main | LB-Main |
| --- | --- | --- | --- |
| Class F | 0.1% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 17.4% | 12.7% | 10.3% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 11.7% | 9.5% | 7.5% |
| Animation YUV 444 | 0.2% | 0.2% | 0.2% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 40.8% | 37.5% | 32.5% |
| SC YUV 444 Optional | 32.6% | 29.0% | 25.4% |
| Enc Time[%] | 126% | 105% | 101% |
| Dec Time[%] | 90% | 98% | 97% |

Table 2 Results of palette-based coding for AHG8 lossy coding

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.2% | -0.9% | -0.6% | -0.4% | -1.0% | -0.7% | -0.5% | -1.0% | -0.8% |
| Class B | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -13.8% | -13.1% | -13.6% | -14.8% | -14.4% | -14.7% | -15.1% | -15.2% | -15.3% |
| Animation RGB 444 | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| SC YUV 444 | -4.9% | -6.9% | -8.2% | -7.2% | -8.3% | -9.5% | -9.0% | -9.8% | -10.5% |
| Animation YUV 444 | 0.1% | 0.0% | -0.1% | 0.1% | -0.2% | -0.1% | 0.0% | -0.2% | -0.2% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -37.2% | -36.7% | -37.1% | -40.1% | -40.4% | -41.1% | -43.5% | -44.5% | -45.4% |
| SC(444) YUV Optional | -18.8% | -23.8% | -24.2% | -23.6% | -26.4% | -28.0% | -28.6% | -32.1% | -33.6% |
| Enc Time[%] | 114% |  |  | 115% |  |  | 116% |  |  |
| Dec Time[%] | 92% |  |  | 91% |  |  | 91% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -1.5% | -0.8% | -0.1% | -1.2% | -0.6% |  |  |  |
| Class B | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC RGB 444 | -13.2% | -12.5% | -13.1% | -13.6% | -13.2% | -13.7% |  |  |  |
| Animation RGB 444 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC YUV 444 | -4.6% | -7.5% | -9.0% | -6.8% | -8.9% | -10.0% |  |  |  |
| Animation YUV 444 | 0.2% | 0.0% | 0.1% | 0.1% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -31.1% | -30.3% | -30.7% | -36.4% | -36.1% | -36.4% |  |  |  |
| SC(444) YUV Optional | -17.5% | -25.0% | -26.1% | -21.2% | -25.6% | -27.3% |  |  |  |
| Enc Time[%] | 107% |  |  | 106% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.2% | -1.6% | -0.4% | 0.2% | -1.0% | -0.2% |  |  |  |
| Class B | 0.2% | 0.2% | -0.2% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC RGB 444 | -10.3% | -9.8% | -10.5% | -12.0% | -11.6% | -12.3% |  |  |  |
| Animation RGB 444 | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |  |  |  |
| SC YUV 444 | -3.7% | -6.6% | -7.4% | -5.5% | -7.5% | -8.4% |  |  |  |
| Animation YUV 444 | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.0% |  |  |  |
| RangeExt | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |  |  |  |
| SC(444) GBR Optional | -27.7% | -27.4% | -27.3% | -35.5% | -35.4% | -36.0% |  |  |  |
| SC(444) YUV Optional | -17.1% | -21.4% | -22.2% | -20.3% | -22.7% | -24.2% |  |  |  |
| Enc Time[%] | 103% |  |  | 104% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |  |  |  |

|  |  |  |  |  |  |  |  |  | V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | -0.8% |
|  |  |  |  |  |  |  |  |  | 0.0% |
|  |  |  |  |  |  |  |  |  | -15.3% |
|  |  |  |  |  |  |  |  |  | -0.1% |
|  |  |  |  |  |  |  |  |  | -10.5% |
|  |  |  |  |  |  |  |  |  | -0.2% |
|  |  |  |  |  |  |  |  |  | 0.0% |
|  |  |  |  |  |  |  |  |  | -45.4% |
|  |  |  |  |  |  |  |  |  | -33.6% |

# Conclusions

This contribution evaluates the performance of palette-based coding mode (which was previously proposed in JCTVC-N0249) on top of the latest Range Extension software HM-12.0+RExt-4.1.

# Draft texts

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if (palette_mode_flag) { |  |
| palette_size_minus1[x0][y0] | ae(v) |
| for (cIdx = 0; cIdx < 3; cIdx ++) |  |
| for( i = 0; i <=palette_size_minus1[x0][y0]; i ++ ) |  |
| palette[x0][y0][ cIdx][i] | ae(v) |
| scanPos = 0 |  |
| while (scanPos < nCbS * nCbS) { |  |
| run_mode_flag | ae(v) |
| if (run_mode_flag) |  |
| { |  |
| palette_index | ae(v) |
| } |  |
| run | ae(v) |
| runPos= 0 |  |
| while (runPos <= run){ |  |
| xC = scanPos% nCbS |  |
| yC = scanPos/ nCbS |  |
| if (run_mode_flag){ |  |
| paletteMap[ x0 ][ y0 ] [ xC ][ yC ] = palette_index |  |
| } |  |
| else{ |  |
| paletteMap[ x0 ][ y0 ] [ xC ][ yC ] = paletteMap[ x0 ][ y0 ] [ xC ][ yC -1 ] |  |
| } |  |
| runPos ++ |  |
| scanPos++ |  |
| } |  |
| } |  |
| } |  |
| else { |  |
| if( slice_type != I ) |  |
| ….. |  |
| } |  |

## Semantics

palette_mode_flag[x0][ y0] equal to 1 specifies that the current coding unit is encoded using palette mode. palette_mode_flag  equal to 0 specifies the current coding unti is encoded using a mode other than palette mode.

When palette_mode_flag is not present, it is inferred to be equal to 0.

In the following sematics, the array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

palette_size_minus1[x0][y0] plus 1 specifies the number of elements in the palette. palette_size_minus1[x0][y0] shall be the range of 0 to 23.

When palette_size_minus1 [x0][y0] is not present, it is inferred to be equal to 0.

palette[x0][y0][cIdx][i] specifies the i-th element in the palette for the color component cIdx.

When palette[x0][y0][cIdx][i] is not present, it is inferred to be equal to 0.

run_mode_flag equal to 1 specifies that the decoding is swtiched to “run mode” where one or more following locations have the same palette index. The run_mode_flag equal to 0 specifies that the decoding is switched to “copy above mode” where  one or more following locations have the same palette index as their corresponding location in the above row.

When run_mode_flag is not present, it is inferred to be equal to 0.

copy_above_flag equal to 1 specifies that the decoding is switched to “copy from above mode” where the decoded pixel value is equal to the pixel at the same location in the above row. copy_above_flag equal to 0 specifies that the decoding is switched to either “run mode” or “pixel mode”.

When copy_above_flag is not present, it is inferred to be equal to 0.

palette_index represents an index in the palette.

When palette_index is not present, it is inferred to be equal to 0.

run represents the number of consecutive locations with the same palette index ( “run_mode_flag = 1”) or the number of consecurive locations with the same palette index as the position in the above row.

##### Palette mode prediction process

Inputs to this process are:

–	a location ( xB, yB ) specifying the top-left sample of the current prediction block relative to the top left sample of the current coding block,

–	the width and height of this coding block, nCbS and nCbS,

–	one (nCbS)x(nCbS) array paletteMap

–	a variable cIdx specifying colour component index.

–	an array palette[xB][yB][cIdx] of size (palette_size_minus1+1)

Outputs of this process are:

–	the (nPbW)x(nPbH) array predSamples of prediction sample values.

The variable nPbW and nPbH are derived as follows.

–	If cIdx is equal to 0, nPbW is set equal to nCbS and nPbH is set equal to nCbS.

–	Otherwise, nPbW is set equal to nCbS/SubWidthC and nPbH is set equal to nCbS/SubHeightC.

For each sample location ( x = 0..nPbW−1, y = 0..nPbH−1 ) inside the prediction sample array predSamples, the corresponding prediction sample value predSamples[ x, y ] is derived as follows:

- predSamples[ x, y ] = palette[xB][yB][cIdx][paletteMap[xP][yP]]

- The variables xP and yP are derived by

xP = x/ SubWidthC

yP = y/ SubHeightC

# References

- L. Guo, M. Karczewicz, J. Sole, and R. Joshi, “Non-RCE3: Modified Palette Mode for Screen Content Coding”, JCTVC-N0249, Vienna, Austria, Jul. 2013.

- C. Rosewarne, J. Sole, K. Sharman, and S.-H Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization and update methods”, JCTVC-N1122, Vienna, Austria, Jul. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).