| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0219 |
| --- | --- |

| Title: | RCE2: Results of Test C1 on Rice Parameter Initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Liwei Guo Joel Sole Rajan Joshi 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of RCE2 Test C1. In this test, the initial Rice parameter for each 4×4 sub-block is deteremind based statistics collected from previously coded sub-blocks. The statistics are collected at most once per sub-block, and is converted into the initial Rice parameter with a linear equation.

# Introduction

In HEVC, the binarization of the syntax element coeff_abs_level_remaining depends on the Rice parameter. The parameter (cRiceParam) is set to 0 at the beginning of each 4×4 sub-block of transform coefficients. The proposed scheme initializes the Rice parameter for each 4×4 sub-block to a value that depends on the previously coded coefficients, the component index, and the value of the transform skip flag (isTSFlag).

# Technical Details

In the proposed scheme, the 4×4 sub-blocks are divided into different categories (“sbType”). For the current sub-block, its initial Rice parameter is derived based on the previously coded sub-blocks in the same category.

The categorization is based on whether the block is transform skip block or not (isTSFlag). A PPS flag useSeparateStatsFlag is introduced to enable further splitting sub-blocks based on whether it is the first color component or not (isLumaFlag)

sbType = (useSeparateStatsFlag? 2*isLumaFlag:0) + isTSFlag;

Statistics are maintained for each type of sub-block depending on the absolute coefficient value (uiLevel):

statCoeff += (uiLevel == statCoeff) ? 0 : ( ( uiLevel < statCoeff ) ? -1 : 1);

statCoeff = min(320, statCoeff);

This variable is updated at most once per 4×4 sub-block using the value of the first coded coeff_abs_level_remaining of the sub-block. The entries of statCoeff are 9-bit and they are reset to 0 at the beginning of the slice (like the CABAC contexts).

The value of statCoeff is used to initialize the Rice parameter at the beginning of each 4×4 sub-block as follows:

cRiceParam = statCoeff<16? (statCoeff+1)>>2: 4+(statCoeff+40)>>6);

The calculated cRiceParam is further clipped,

cRiceParam = Clip3 (minRicePara, maxRicePara, cRiceParam)

with minRicePara = 0 for internal bit-depth equal to or smaller than 10, and minRicePara =2 otherwise.

# Experimental results

The proposed method is implemented on HM-12.0+RExt-4.1. AHG8 (lossless/lossy) and AHG18 (lossy) test conditions specified in [2] are used. Decoding time is measured on a homogenous cluster.

For AHG8 test and AHG 18 test, the maximum rice parameter is set to be 7 and 16 respectively. The useSeparateStatsFlag is set to be 1 for YUV sequences and 0 for RGB sequences.

Additional results for AHG8 test with maximum rice parameter being 4 are provided in the attached Excel files.

Table 1 Results of Method A for AHG8 lossless coding

| Avg Bit-Rate Saving | AI-Main | RA-Main | LB-Main |
| --- | --- | --- | --- |
| Class F | 1.4% | 0.7% | 0.4% |
| Class B | 0.1% | 0.0% | 0.0% |
| SC RGB 444 | 8.6% | 7.0% | 6.3% |
| Animation RGB 444 | 0.2% | 0.0% | 0.0% |
| SC YUV 444 | 5.2% | 4.3% | 3.9% |
| Animation YUV 444 | 0.2% | 0.1% | 0.1% |
| RangeExt | 3.1% | 2.3% | 2.3% |
| SC GBR 444 Optional | 20.6% | 20.2% | 19.9% |
| SC YUV 444 Optional | 15.4% | 15.2% | 14.3% |
| Enc Time[%] | 98% | 101% | 98% |
| Dec Time[%] | 98% | 99% | 98% |

Table 2 Results of Method A for AHG8 lossy coding

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.4% | -0.5% | -0.5% | -0.7% | -0.8% | -0.7% | -1.1% | -1.1% | -1.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.3% | -1.4% | -1.4% | -2.5% | -2.4% | -2.4% | -3.9% | -4.1% | -4.1% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.5% | -0.5% | -0.5% | -1.1% | -1.0% | -1.0% | -1.8% | -1.9% | -1.8% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -5.5% | -5.3% | -5.3% | -8.9% | -8.7% | -9.1% | -12.4% | -13.2% | -13.3% |
| SC(444) YUV Optional | -1.6% | -1.8% | -1.7% | -3.4% | -3.3% | -3.2% | -5.9% | -5.8% | -6.4% |
| Enc Time[%] | 98% |  |  | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.4% | -0.4% | -0.5% | -0.5% | -0.5% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.2% | -1.2% | -1.2% | -2.2% | -2.2% | -2.2% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.4% | -0.4% | -0.3% | -0.8% | -0.8% | -0.8% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -5.3% | -5.1% | -5.0% | -8.5% | -8.4% | -8.7% |  |  |  |
| SC(444) YUV Optional | -1.5% | -1.6% | -1.4% | -3.3% | -3.2% | -3.0% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -1.1% | -0.5% | -0.3% | -0.7% | -0.5% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |
| SC RGB 444 | -0.8% | -0.8% | -0.8% | -1.5% | -1.4% | -1.5% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.4% | -0.4% | -0.6% | -0.5% | -0.3% | -0.6% |  |  |  |
| Animation YUV 444 | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% |  |  |  |
| SC(444) GBR Optional | -4.9% | -5.0% | -4.9% | -9.0% | -9.0% | -9.6% |  |  |  |
| SC(444) YUV Optional | -1.6% | -1.6% | -1.5% | -2.7% | -2.6% | -2.7% |  |  |  |
| Enc Time[%] | 94% |  |  | 94% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 3 Results of Method A for AHG18 lossy coding - 6 bit HM forward transform

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G | B | R |
| 12-bit | -7.1% | -6.8% | -6.8% |
| 14-bit | -18.7% | -18.4% | -18.4% |
| 16-bit | -28.4% | -28.2% | -28.2% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 86% |  |  |

Table 4 Results of Method A for AHG18 lossy coding – high precision forward transform

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G | B | R |
| 12-bit | -6.9% | -7.0% | -7.0% |
| 14-bit | -18.3% | -18.1% | -18.1% |
| 16-bit | -27.9% | -27.6% | -27.6% |
| Enc Time[%] | 88% |  |  |
| Dec Time[%] | 81% |  |  |

# Conclusions

This contribution presents the results of RCE2 Test C1. The tested method determines the initial Rice parameter for each 4×4 sub-block based on previously coded coefficients. Coding efficiency improvements have been observed in all the test cases.

# Draft texts

Please find draft texts in the accompanying documents.

# References

- M. Karczewicz, J. Sole, and R. Joshi, “Non-RCE2: Rice Parameter Initialization”, JCTVC-N0181, Vienna, Austria, Jul. 2013.

- C. Rosewarne, J. Sole, K. Sharman, and S.-H Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization and update methods”, JCTVC-N1122, Vienna, Austria, Jul. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).