| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0220 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0077 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On first decodable CRA picture of the layer-wise startup |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jung Won Kang Hahyun Lee Jinho Lee Jin Soo Choi | Tel: Email: | +82-42-860-5137 jungwon@etri.re.kr hanilee@etri.re.kr jinosoul@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

# Abstract

In the layer-wise startup decoding process, if the first correctly decodable picture of a particular layer is a CRA picture, NoRaslOutputFlag of the first correctly decodable CRA picture has to be set equal to 1 in order not to output the associated RASL pictures. It is asserted that, in the layer-wise startup decoding process of current SHVC and MV-HEVC draft text, there is no clear description to set NoRaslOutputFlag of the first correctly decodable CRA picture to 1. It is also asserted that the decoding process for generating unavailable reference pictures specified in F.8.1.3 shall be invoked for the first decodable CRA picture with nuh_layer_id > 0 because all reference pictures with nuh_layer_id in the DPB are marked as “unused for reference” when the current picture is a CRA picture with NoRaslOutputFlag = 1 and nuh_layer > 0. This contribution proposes two alternative solutions to set NoRaslOutputFlag of the first correctly decodable CRA picture to 1 and a condition to invoke the decoding process for generating unavailable reference pictures when nuh_layer_id is greater than 0.

# Problem Statement

At the 14th JCT-VC meeting, layer-wise startup decoding process has been adopted in SHVC and MV-HEVC draft text to allow starting the decoding process from an IRAP picture with NoRaslOutputFlag = 1 (with nuh_layer_id = 0) when some of the pictures in the same access unit and with nuh_layer_id greater than 0 are non-IRAP pictures.

In the current SHVC and MV-HEVC draft text for layer-wise startup, two variables, FirstPicLayerDecodedFlag[nuh_layer_id] and LayerInitialisedFlag[nuh_layer_id], are defined.

FirstPicLayerDecodedFlag[nuh_layer_id] is used to indicate whether the first picture with nuh_layer_id has been decoded. If a picture with nuh_layer_id is in the initial IRAP picture of the layer-wise startup decoding process, FirstPicLayerDecodedFlag[nuh_layer_id] is set equal to 0 before decoding that picture and set equal to 1 after decoding that picture.

LayerInitialisedFlag[nuh_layer_id] is used to indicate whether an IRAP picture with nuh_layer_id has been correctly decoded after all its reference layers have also been correctly decoded.

Figure 1 shows examples of bitstream for layer-wise startup. Figure 1 (a) presents a typical bitstream structure for layer-wise startup. In addition, the bitstream structure, where more frequent IRAP pictures are used in the higher enhancement layer, shown in Figure 1 (b) is also not disallowed in the current SHVC and MV-HEVC draft text.

In the layer-wise startup decoding process of the bitstream presented in Figure 1, the first (correctly) decodable CRA picture has to be a first CRA picture with a particular layer, nuh_layer_id = k, and LayerInitialsedFlag[refLayerId]=1 for all values of refLayerId equal to RefLayerId[k][j], where j is in the range of 0 to NumDirectRefLayers[k]-1, inclusive.

When decoding the first decodable CRA picture with nuh_layer_id = k, NoRaslOutputFlag for that CRA picture has to be set equal to 1 in order not to output the associated RASL pictures while outputting the CRA picture in the DPB. If NoRaslOutputFlag is not set equal to 1 for the CRA picture, PicOutputFlag of the RASL pictures associated to the CRA picture is going to be set equal to pic_output_flag signalled in slice segment header of the RASL pictures because LayerInitialisedFlag[k] is set equal to 1 while decoding the CRA picture.

It should be noted that PicOutputFlag is set as follows.

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, if LayerInitialisedFlag[ nuh_layer_id ] is equal to 0, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

(a)

(b)

Fig. 1. Examples of bitstream for layer-wise startup

In addition, when the current picture is a CRA picture with NoRaslOutputFlag = 1 and nuh_layer > 0, all reference pictures with the same layer as the current picture in the DPB are marked as “unused for reference.” Because of this marking process, the decoding process for generating unavailable reference pictures specified in F.8.1.3 shall be invoked when decoding the first decodable CRA picture with nuh_layer_id > 0.

However, in the current SHVC and MV-HEVC draft text, there is no clear description to set NoRaslOutputFlag and to invoke the decoding process for generating unavailable reference picture when decoding the first decodable CRA pictures with nuh_layer_id > 0.

It should be noted that the decoding process for generating unavailable reference pictures specified in F.8.1.3 is invoked only when FirstPicInLayerDecodedFlag[nuh_layer_id] is equal to 0 and nuh_layer_id is greater than 0.

Therefore, this contribution proposes two alternative solutions to set NoRaslOutputFlag for the first decodable CRA picture with nuh_layer_id > 0 to 1 and a condition to invoke the decoding process for generating unavailable reference pictures specified in F.8.1.3.

# Proposal

## On NoRaslOutputFlag of the first decodable CRA picture

If the first decodable CRA picture is not the first picture with nuh_layer_id = k in the CVS, in decoding order, there are two possible solutions in order to set NoRaslOutputFlag equal to 1.

Alternative 1

First, NAL unit type of the first decodable CRA picture is changed to BLA_W_LP, BLA_W_RADL, or BLA_N_LP. However, it could be difficult to change NAL unit type when there are a lot of pictures, especially more than one IRAP pictures described in Figure 2, preceding the first decodable CRA picture, in decoding order.

Alternative 2

Second, NoRaslOutputFlag is set to equal to 1 when the current picture is an IRAP picture, LayerInitialisedFlag[k] = 0, and LayerInitialsedFlag[refLayerId]=1 for all values of refLayerId equal to RefLayerId[k][j], where j is in the range of 0 to NumDirectRefLayers[k]-1, inclusive. In this solution, LayerInitialisedFlag[k] shall be set equal to 1 after setting NoRaslOutputFlag to 1.

## On invoking the decoding process for generating unavailable reference pictures when nuh_layer_id is greater than 0

It is proposed to add a condition to invoke the decoding process for generating unavailable reference pictures when nuh_layer_id is greater than 0.

The Newly introduced text is shown in red.

F.8.1.1 Decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0

Each picture referred to in this subclause is a complete coded picture.

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The processes in subclause Error: Reference source not found specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause Error: Reference source not found. This needs to be invoked only for the first slice segment of a picture. It is a requirement of bitstream conformance that PicOrderCntVal shall remain unchanged within an access unit.

–	The decoding process for RPS in subclause Error: Reference source not found is invoked, wherein only reference pictures with a nuh_layer_id equal to that of CurrPic may be marked as "unused for reference" or "used for long-term reference" and any picture with a different value of nuh_layer_id is not marked. This needs to be invoked only for the first slice segment of a picture.

–	When FirstPicInLayerDecodedFlag[ nuh_layer_id ] is equal to 0 or the current picture is the IRAP picture with NoRaslOutputFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause Error: Reference source not found is invoked, which needs to be invoked only for the first slice segment of a picture.

# Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).