| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0221 |
| --- | --- |

| Title: | RExt: On motion compensation memory bandwidth constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Broadcom Corporation, USA | Email: | minhua@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

Motion compensation memory bandwidth has increasingly become a bottleneck in chip designs as video fidelity increase in the HEVC range extension. It is proposed to reduce the worst case luma reference block size from the current 28800 to 22080 per 64x64 region in Main tier HEVC range extension profiles. The constraint is expressed in terms of the total luma reference block size per picture, with the luma reference block size for an intra prediction unit defined as 0, and for an inter prediction unit of size nPbW x nPbH defined as (nPbW+ 7)*( nPbH + 7) if it is a uni-directional prediction unit, and as 2*(nPbW+ 7)*( nPbH + 7) if it is a bi-directional prediction unit. Experimental results show that the proposed constraint can reduce the worst case memory bandwidth by about 23% without causing quality loss.

# Introduction

As video resolution moves to UHD, motion compensation memory bandwidth has increasingly become a bottleneck for video chip designs. This problem becomes worse for HEVC range extension[1] as it is designed to support higher chroma formats and higher bit-depths. Since the HEVC range extension profiles may find application in cost-sensitive consumer market, there is a strong need to reduce the motion memory bandwidth requirements in the HEVC range extension to enable cost-effective video solutions.

In the HEVC version 1 [2] technology for memory bandwidth reduction has focused on disabling the inter PUs of small sizes (e.g. 4x4), and disabling bi-directional prediction for small PUs (e.g. 4x8, 8x4). Other techniques including restricting the motion vectors to integer precision was also under consideration.

However, since the HEVC version 1 has been finalized and the HEVC range extension decoders must be able to decode HEVC version 1 bitstreams, further restricting bi-prediction or motion vector accuracy does not seem to be desirable from the backward compatibility point of view.

Instead of changing syntax to impose the further memory bandwidth restriction, it is proposed to limit the maximum number of motion vectors a picture can have. The restriction is expressed in terms of the maximum reference block size per 64x64 region averaged over a picture, where the reference block size of a PU is computed based on the PU size and the filter tap length of the luma interpolation filter.

# Proposed Constraint

Figure 1. Illustration of luma reference block size calculation of a PU

As shown in Figure 1, for an inter PU of size PUwidth[k] x PUheight[k] the reference block size is defined as:

refBlkSizePU[k]=(PUwidth[k]+7)∗(PUheight[k]+7)ifPUisuni−directional2∗(PUwidth[k]+7)∗(PUheight[k]+7)ifPUisbi−directional <eq.1>

For an intra PU of size PUwidth[k] x PUheight[k], the reference block size is defined as 0.

Where k is index of PUs in a CTU.

Let pic_width_in_CTUs and pic_height_in_CTUs be the number of CTU columns and rows of a picture, respectively. For the i-th CTU in the picture, the total reference block size is computed by adding up the reference block sizes of PUs in the CTU, and as:

refBlkSizeCTU[i]=krefBlkSizePU[k]                                      <eq.2>

Where i = 0, 1, 2, …, pic_width_in_CTUs*pic_height_in_CTUs -1.

The proposed restriction is imposed as follows:

A conforming bitstream must satisfy the following condition:

i=0pic_size_in_CTUs -1refBlkSizeCTU[i]≤max_refBlkSizeper64x64⋅N2⋅picsizeinCTUs1024                    <eq.3>

Where pic_size_in_CTUs =  pic_width_in_CTUs *pic_height_in_CTUs is the number of CTUs a picture has, and 2Nx2N (N = 8, 16 or 32) is the CTU size used by the picture.

The maximum reference size per 64x64 region, max_refBlksize_per64x64, should be specified the same across levels by considering the worst case motion compensation memory bandwidth requirements after this new restriction is applied. Table 1 summarizes the worst case reference block size of a 64x64 CTU for different PU sizes. In the current HEVC range extension design, the 8x8 bidirectional (bipred) PUs are bottleneck for memory bandwidth, which requires a total of 28800 of reference block data for a CTU of size 64x64. To reduce the memory bandwidth requirements, the next cut-off point is the worst case memory bandwidth required for 16x8 or 8x16 16x4/16x12 or 4x16/12x16 bidirectional PUs, which is roughly 23% less than the current worst case number (i.e 8x8 bipred) according to Table 1.

| PU size | List 0/1 luma reference block size | List 1 luma reference block size | Total luma reference block size for a 64x64 CTU |
| --- | --- | --- | --- |
| 8x8 bipred | 225 | 225 | 28800 |
| 16x8, 8x16, 16x4, 16x12, 4x16, 12x16 bipred | 345 | 345 | 22080 |
| 8x4 or 4x8 unipred | 165 |  | 21120 |
| 16x16 bipred | 529 | 529 | 16928 |
| 8x8 unipred | 225 |  | 14400 |
| 32x16, 16x32, 32x8, 32x24, 8x32, 24x32 bipred | 897 | 897 | 14352 |
| 32x32 bipred | 1521 | 1521 | 12168 |
| 64x32, 32x64, 64x16, 64x48, 16x64, 48x64 bipred | 2769 | 2769 | 11076 |
| 16x8, 8x16, 16x4, 16x12, 4x16, 12x16 unipred | 345 |  | 11040 |
| 64x64 bipred | 5041 | 5041 | 10082 |
| 16x16 unipred | 529 |  | 8464 |
| 32x16, 16x32, 32x8, 32x24, 8x32, 24x32 uipred | 897 |  | 7171 |
| 32x32 uipred | 1521 |  | 6084 |
| 64x32, 32x64, 64x16, 64x48, 16x64, 48x64 uipred | 2769 |  | 5538 |
| 64x64 bipred | 5041 |  | 5041 |

Table 1. The worst case luma reference block size per 64x64 CTU for different PU sizes

In this contribution we propose to set max_refBlksize_per64x64 to be 22080 across levels and for future main tier HEVC range extension profiles. Note that max_refBlksize_per64x64 is independent of chroma format and bit-depth.

# Encoder Memory Bandwidth Control Algorithm

For generating a conforming bitstream encoder control algorithms are needed to impose the proposed constraint on the encoder side. In the experimental results provided in the next section, the following encoder memory bandwidth control algorithm is used.

Fig. 2.  Block diagram of proposed encoder memory bandwidth control algorithm

As shown in Figure 2, the proposed algorithm has the following steps:

- Perform motion estimation for the current  CTU  (i.e. i-th CTU), where i = 0, 1, …, pic_size_in_CTUs -1.

- Compress the current CTU.

- In parallel measure the reference block size consumed by the current CTU, refBlkSize_CTU[i], based on <eq. 2>.

- Accumulate the total consumed reference block size up to the current CTU,

accum_refBlkSize  +=  refBlkSize_CTU[i]

where accum_refBlkSize is set to 0 at the beginning of each picture.

- Compute the delta reference block size that needs to be adjusted in the encoding of the next CTU by steps specified in the table below.

| Step | Remark |
| --- | --- |
| pic_size_in_CTUs = pic_width_in_CTUs * pic_height_in_CTUs | Total number of CTUs in a picture |
| target_refBlkSize = (i+1)*max_refBlkSize_per64x64*N2/1024 + initial_refBlkSize | Target reference block size based on the restriction. max_refBlkSize_per64x64 is maximum luma reference block size for a 64x64 CTU which is set to 22080. 2Nx2N is CTU size, initial_refBlkSize provides control margin, it can be set to e.g. 0.5* max_refBlkSize_per64x64*N2/1024. |
| delta_refBlkSize = accum_refBlkSize - target_refBlkSize; | Compute the delta reference block size (up to the current CTU) relative to the target value. A positive value indicates the consumed reference block so far exceeds the budget. |
| if (delta_refBlkSize > 0 && CTU_num ≥ M) { if (CTU_num ≤ pic_size_in_CTUs– M) delta_refBlkSize_adjusted = delta_refBlkSize/M else delta_refBlkSize_adjusted = delta_refBlkSize/( pic_size_in_CTUs – CTU_num) } else delta_refBlkSize_adjusted = 0; | Compute the reference block size to be adjusted during the encoding of the next CTU. The size of adjusted reference block is smoothed over M CTUs (e.g. M can be set to 4) to avoid jitter and achieve better video quality. |
| delta_refBlkSize_adjusted = delta_refBlkSize_adjusted*1024/N2 | Normalize to 64x64 CTU size |
| If (delta_refBlkSize_adjusted==0) No restriction in the motion estimation of the next CTU else If (delta_refBlkSize_adjusted > 0 && delta_refBlkSize_adjusted<6720) Disable 8x8 bi-pred inter PUs else If (delta_refBlkSize_adjusted > 6720 && delta_refBlkSize_adjusted≤7680) Disable 16x8, 8x16, 16x4, 16x12, 4x16, 12x16 bi-pred inter PUs else If (delta_refBlkSize_adjusted > 7680 && delta_refBlkSize_adjusted≤11872) Disable 16x8, 8x16, 16x4, 16x12, 4x16, 12x16 bi-pred inter PUs, and disable 8x4 and 4x8 uni-pred inter PUs else If (delta_refBlkSize_adjusted > 11872) Disable 16x8, 8x16, 16x4, 16x12, 4x16, 12x16, 16x16 bi-pred inter PUs, and disable 8x4 and 4x8 uni-pred inter PUs | Compute the memory bandwidth control information for the next CTU based on delta_refBlkSize_adjusted. The thresholds (6720, 7680, etc.) here are set according to Table 1. The algorithm has ability of controlling the maximum luma reference block size (i.e. max_refBlkSize_per64x64) down to 14400 per 64x64 CTU. |

- If the current CTU is the last CTU in the picture, terminate the encoding of the current picture. Otherwise, go back to step 1 to process the next CTU. The motion estimation of the next CTU may disable certain types of inter PUs (both regular ad merge/skip inter PUs) based on restrictions specified by the memory bandwidth control information.

# Test Settings and Conditions

The simulations of this document have used HM-12.0+RExt-4.1 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570 64 bits Linux machines of different frequencies, the HEVC range extension common test conditions and reference configurations specified in [3] are followed.

# Experimental Results

The test results are summarized in Table 1. There is no average quality loss observed when reducing the worst case reference block size from 28800 to 22080 per 64x64 region. The run time in the table may not be accurate.

|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |

Table 1. Experimental Results

Tests reveal that in Main tier HEVC range extension setting 1080p@50 10bit 4:2:2 YUV EBUKidsSoccer sequence is the only sequence which exceeds the cap of reference block size of 22080 per 64x64. As shown in Table 2, 3, the proposed encoder memory bandwidth control algorithm can effectively control the memory bandwidth consumption in the pre-defined range (i.e. average 22080 per 64x64 over any picture) without causing quality loss.

|  | Anchor |  |  |  |  |  | Tested |  |  |  |  |  |  | Tested with 22080 per 64x64 cap |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| qp | kbps | Y psnr | U psnr | V psnr | Max avg. memBW per 64x64 | % frames exceeding 22080 per 64x64 | kbps | Y psnr | U psnr | V psnr | Max avg. memBW per 64x64 | % frames exceeding 22080 per 64x64 |  |  |
| 22 | 40880.251 | 36.498 | 40.801 | 40.29 | 25289 | 25.80% | 40844.068 | 36.492 | 40.797 | 40.288 | 22049 | 0.00% |  |  |
| 27 | 11675.75 | 33.888 | 39.508 | 39.266 | 21841 | 0.00% | 11676.202 | 33.888 | 39.508 | 39.266 | 21721 | 0.00% |  |  |
| 32 | 4301.6576 | 31.302 | 38.265 | 38.209 | 17442 | 0.00% | 4301.6576 | 31.302 | 38.265 | 38.209 | 17442 | 0.00% |  |  |
| 37 | 1424.448 | 28.546 | 37.104 | 37.195 | 13515 | 0.00% | 1424.448 | 28.546 | 37.104 | 37.195 | 13515 | 0.00% |  |  |

Table 2. Experimental Results on 1080p@50 10bit 4:2:2 YUV EBUKidsSoccer, RA-MT

|  | Anchor |  |  |  |  |  | Tested |  |  |  |  |  |  | Tested with 22080 per 64x64 cap |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| qp | kbps | Y psnr | U psnr | V psnr | Max avg. memBW per 64x64 | % frames exceeding 22080 per 64x64 | kbps | Y psnr | U psnr | V psnr | Max avg. memBW per 64x64 | % frames exceeding 22080 per 64x64 |  |  |
| 22 | 52461.931 | 37.13 | 40.703 | 40.274 | 25506 | 25.70% | 52436.892 | 37.122 | 40.711 | 40.278 | 22041 | 0.00% |  |  |
| 27 | 11185.527 | 33.603 | 39.272 | 39.115 | 19802 | 0.00% | 11185.527 | 33.603 | 39.272 | 39.115 | 19802 | 0.00% |  |  |
| 32 | 2979.6312 | 30.557 | 38.006 | 38.008 | 15370 | 0.00% | 2979.6312 | 30.557 | 38.006 | 38.008 | 15370 | 0.00% |  |  |
| 37 | 691.576 | 27.764 | 36.88 | 37.004 | 12028 | 0.00% | 691.576 | 27.764 | 36.88 | 37.004 | 12028 | 0.00% |  |  |

Table 3. Experimental Results on 1080p@50 10bit 4:2:2 YUV EBUKidsSoccer, LB-MT

# Conclusions

Motion compensation memory bandwidth has become a worse problem in the HEVC range extension as video fidelity increases. It is recommended to consider the proposed constraint for Main tier HEVC range extension profiles to address the memory bandwidth issue. The proposed constraint can cut the worst case memory bandwidth by about 23% without causing quality degradation.

# References

[1] D. Flynn, J. Sole and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4” JCTVC-N1005_v1, 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013

[2] B. Bross, G. Sullivan and Y.K. Wang, “Editors' proposed corrections to HEVC version 1,” JCTVC-N0041_v1, 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013

[3] D. Flynn, K. Sharman and C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC_N1006, 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013.

# Patent rights declaration(s)

Broadcom Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed Text Changes

In Annex A add the following section

A.4.3 Profile-specific level limits for the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10 and Main 4:4:4 12 profiles

Bitstreams conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10 or Main 4:4:4 12 profile of main tier at a specific level shall obey the following constraints for each bitstream conformance test as specified in Annex Error: Reference source not found:

- The total luma reference block size for motion compensation of any picture shall not exceed 22080* PicWidthInCtbsY* PicHeightInCtbsY* CtbSizeY* CtbSizeY/1024. Where the luma reference block size for an intra prediction unit is defined as 0, and for an inter prediction unit of size nPbW x nPbH is defined as (nPbW+ 7)*( nPbH + 7) if it is a uni-directional prediction unit, and 2*(nPbW+ 7)*( nPbH + 7) if it is a bi-directional prediction unit.

Rename the current subclause A.4.3 to A.4.4