| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0225 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0080 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On inter-layer RPS derivation and sub-layer inter-layer dependency |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Ye-Kui Wang Jianle Chen Vadim Seregin Hendry Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-2042 krapaka@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com 1-858-651-8028 cjianle@qti.qualcomm.com 1-858-651-3481 vseregin@qti.qualcomm.com 82-2-530-6913 hendry@qti.qualcomm.com 1-858-658-5673 martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes two changes for SHVC and MV-HEVC related to inter-layer RPS derivation process and signaling of sub-layer inter-layer prediction dependency. The first change targets to improve the derivation process of NumActiveRefLayerPics by considering the sub-layer inter-layer prediction restrictions indicated by max_tid_il_ref_pics_plus1, so that the pictures belong to sub-layers that are disallowed to be used for inter-layer prediction are not included in the inter-layer reference picture set when all_ref_layers_active_flag is equal to 1. The second change targets to have layer-specific signaling of max_tid_il_ref_pics_plus1, where max_tid_il_ref_pics_plus1 is signalled separately for the layers that use the current layer as a direct reference layer.

# An improvements to inter-layer RPS derivation

MV-HEVC Draft text 5 and SHVC Draft text 3 include the flag all_ref_layers_active_flag signaled in the VPS with the motivation to save slice-level signaling overhead of inter-layer RPS for certain typical scenarios where for each picture, the reference layer pictures of all its direct reference layers are present in the same access unit and are included in the inter-layer reference picture set. Such scenarios include:

- In typical multi-view video coding scenarios, the direct reference layer pictures as specified in video parameter set (VPS) are present in each access unit and are all used for inter-layer prediction and all inserted into at least one reference picture list of the current picture.

- For scalable video coding, it is common for each enhancement layer to have only one direct reference layer and the direct reference layer picture is present in each access unit and used for inter-layer prediction.

The current SHVC design requires that all_ref_layers_active_flag shall be equal to zero when temporal sub-layer inter-layer prediction restriction which is indicated by syntax max_tid_il_ref_pics_plus1 is enabled. When max_tid_il_ref_pics_plus1[ i ] – 1, for any value of i in the range of 0 to vps_max_layers_minus1, inclusive, are less than or equal to the greatest value of TemporalId of all VCL NAL units in the CVSs referring to the VPS (i.e. pictures in at least one sub-layer are not used for inter-layer prediction), the reference layer picture.

However, it would be more favorable to enable the use of all_ref_layers_active_flag equal to 1 even when pictures of some sub-layers are not used for inter-layer prediction, as long as for each picture not belonging to those sub-layers, the reference layer pictures of all direct reference layers of the layer containing the picture are present in the same access unit and are included in the inter-layer reference picture set. If enabled, compared to the current method in MV-HEVC Draft test 5 and SHVC Draft text 3, slice header bits for signaling of the inter-layer RPS for these scenarios can also be avoided.

In the proposed method, when the flag “all_ref_layers_active_flag” is equal to 1 for the current picture, only the reference layer pictures that are allowed to be used for inter-layer prediction are counted to derive the variable NumActiveRefLayerPics, so that the reference layer pictures that belong to sub-layers which are not used for inter-layer prediction (indicated by max_tid_il_ref_pics_plus1) will not be included in the inter-layer reference picture set.

The changes proposed to the MV-HEVC Draft text 5 and SHVC Draft text 3 are highlighted below in yellow or red strikethrough

## Semantics changes of all_ref_layers_active_flag

The semantics of all_ref_layers_active_flag is modified as below:

all_ref_layers_active_flag equal to 1 specifies that for each picture referring to the VPS, the reference layer pictures of that belong to all direct reference layers of the layer containing the picture and that are allowed to be used for inter-layer prediction, as determined by the values of max_tid_il_ref_pics_plus1[ i ], are present in the same access unit as the picture, and are included in the inter-layer reference picture set of the picture. all_ref_layers_active_flag equal to 0 specifies that the above restriction may or may not apply.

## Changes to NumActiveRefLayerPics derivation

The derivation process of NumActiveRefLayerPics is changed as in the highlighted text below:

The variable NumActiveRefLayerPics is derived as follows:

if( nuh_layer_id  = =  0  | |  NumDirectRefLayers[ nuh_layer_id ]  = =  0 )
	NumActiveRefLayerPics = 0
else if( all_ref_layers_active_flag )
	NumActiveRefLayerPics = NumDirectRefLayers[ nuh_layer_id ]
	for( i = 0, NumActiveRefLayerPics = 0; i < NumDirectRefLayers[ nuh_layer_id ]; i++ ) {
		layerIdx = LayerIdxInVps[ RefLayerId[ nuh_layer_id ][ i ] ]
		if( max_tid_il_ref_pics_plus1[ layerIdx ] > TemporalId )
			NumActiveRefLayerPics++
	}

else if( !inter_layer_pred_enabled_flag )
	NumActiveRefLayerPics = 0
else if( max_one_active_ref_layer_flag  | |  NumDirectRefLayers[ nuh_layer_id ]  = = 1 )
	NumActiveRefLayerPics = 1
else
	NumActiveRefLayerPics = num_inter_layer_ref_pics_minus1 + 1

## Bit-saving analysis

The maximum slice header bit-saving that can be achieved is detailed in the Table-1 when max_tid_il_ref_pics_plus1[ i ] – 1, for any value of i in the range of 0 to vps_max_layers_minus1, inclusive, is less than or equal to the greatest value of TemporalId of all VCL NAL units in the CVSs referring to the VPS.

| Number of layers | Slice header bit saving |
| --- | --- |
| 2 | 1 |
| 3 | 3 |
| 4 | 7 |
| 5 | 9 |
| 6 | 16 |
| 7 | 19 |
| 8 | 22 |
| … | … |
| 63 | 373 |
| 64 | 379 |

Table-1: Slice header Bit-saving analysis

Assuming NumActiveRefLayerPics = NumDirectRefLayers[ nuh_layer_id ] – 1 and 
N = Number of layers – 1 = Ceil( Log2( NumDirectRefLayers[ nuh_layer_id ] ) ), the bit saving for each syntax element is shown as a function of N as,

| if( nuh_layer_id > 0 && ! all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { | Bit-Saving |
| --- | --- |
| if( ! pic_skip_flag) |  |
| inter_layer_pred_enabled_flag | 1 bit |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | Ceil(log2(N)) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | Ceil(N – 1 * log2(N)) |
| } |  |
| } |  |

# Flexible sub-layer based inter-layer prediction dependency

MV-HEVC WD5 and SHVC WD3 include the flag “max_tid_il_ref_pics_plus1” in the VPS to provide information at VPS-level on whether pictures belonging to a temporal sub-layer are used for inter-layer prediction by any higher-layer pictures or not. This information can be useful to discard those pictures that belong to a sub-layer that is not used for inter-layer prediction. This provides a trade-off between coding efficiency, memory bandwidth, and computational complexity in HEVC multiple layer extensions.

The current signaling based on max_tid_il_ref_pics_plus1 is too restrictive. It requires that inter-layer prediction for pictures that belong to a temporal sub-layer are either enabled or disabled for pictures of all higher layers as shown in Figure 1. In Figure 1, case (a): max_tid_il_ref_pics_plus1 for Layer ID = 0 is set to 2, that is layer 0 pictures with TemporalId greater than or equal to 2 cannot be used as interlayer reference for any higher layers. Case (b): max_tid_il_ref_pics_plus1 for Layer ID = 0 is set to 7, that is pictures with TemporalId greater than or equal to 7 cannot be used as interlayer reference for any higher layers, since the greatest possible value of TemporalId is 6, thus all pictures, including those with TemporalId = 2, may be used as interlayer reference for higher layers.

Figure 1: Example configurations allowed with current max_tid_il_ref_pics_plus1 design in HEVC extensions

Figure 2 shows an example of a configuration that is not possible to achieve using current signaling process. In Figure 2, case (a): pictures that belong to TemporalId = 2 cannot be used as interlayer reference for Layer ID = 1 but may be used for inter-layer prediction for Layer ID = 2. This configuration provides the flexibility to have lightweight decoding for target Layer ID = 1 and high quality decoding for target Layer ID = 2. Case (b): pictures that belong to TemporalId = 2 cannot be used as interlayer reference for Layer ID = 2 but may be used for inter-layer prediction for Layer ID = 1. This configuration provides the flexibility to have lightweight decoding for target Layer ID = 2 and high quality decoding for target Layer ID = 1.

Figure 2: Example configurations allowed with max_tid_il_ref_pics_plus1 design in HEVC extensions

It is asserted that such trade-off flexibilities may be highly desirable in real time application scenarios.

To achieve this, the modifications are proposed to the semantics of max_tid_ref_present_flag and max_tid_il_ref_pics_plus1 in the MV-HEVC Draft text 5 and SHVC Draft text 3 are highlighted below in yellow.

max_tid_ref_present_flag equal to 1 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ][ j ] is present. max_tid_ref_present_flag equal to 0 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ][ j ] is not present.

max_tid_il_ref_pics_plus1[ i ][ j ] equal to 0 specifies that within the CVS non-IRAP pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ]. max_tid_il_ref_pics_plus1[ i ][ j ] greater than 0 specifies that within the CVS pictures with nuh_layer_id equal to layer_id_in_nuh[ i ] and TemporalId greater than max_tid_il_ref_pics_plus1[ i ][ j ] – 1 are not used as reference for inter-layer prediction for pictures with nuh_layer_id equal to layer_id_in_nuh[ j ]. When not present, max_tid_il_ref_pics_plus1[ i ][ j ] is inferred to be equal to 7.

Following are the proposed changes to syntax table:

| vps_extension( ) { | Descriptor |
| --- | --- |
| …….. |  |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| for( i = 0; i < vps_max_layers_minus1; i++ ) |  |
| for( j = i + 1; j <= vps_max_layers_minus1; j++ ) |  |
| if( direct_dependency_flag[ j ][ i ] ) |  |
| max_tid_il_ref_pics_plus1[ i ][ j ] | u(3) |
| ………. | u(1) |
| vps_vui_present_flag | u(1) |
| } |  |

F.7.4.7.1	General slice segment header semantics

It is a requirement of bitstream conformance that for each value of i in the range of 0 to NumActiveRefLayerPics − 1, inclusive, either of the following two conditions shall be true:

–The value of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ][ LayerIdxInVps[ nuh_layer_id ] is greater than TemporalId.

–The values of max_tid_il_ref_pics_plus1[ LayerIdxInVps[ RefPicLayerId[ i ] ] ][ LayerIdxInVps[ nuh_layer_id ] and TemporalId are both equal to 0 and the picture in the current access unit with nuh_layer_id equal to RefPicLayerId[ i ] is an IRAP picture.

F.8.1.2.1	Marking process for sub-layer non-reference pictures not needed for inter-layer prediction

remainingInterLayerReferencesFlag = 0
		iLidx = LayerIdxInVps[ TargetDecLayerIdList[ i ] ]
		for( j = latestDecIdx + 1; j < numTargetDecLayers; j++ ) {
			jLidx = LayerIdxInVps[ TargetDecLayerIdList[ j ] ]
			if( currTid  <=  ( max_tid_il_ref_pics_plus1[ iLidx ][ jLidx ] –1 ) )
				for( k = 0; k < NumDirectRefLayers[ TargetDecLayerIdList[ j ] ]; k++ )
					if( TargetDecLayerIdList[ i ]  = =  RefLayerId[ TargetDecLayerIdList[ j ] ][ k ] )
						remainingInterLayerReferencesFlag = 1
		}

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).