| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0226 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0081 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On early indication of parallel processing tools in HEVC extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Hendry Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-2042 krapaka@qti.qualcomm.com 82-2-530-6913 hendry@qti.qualcomm.com 1-858-651-8345 yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to indicate the configuration of parallel processing tools (tiles and WPP) across layers in the VPS VUI.

# Introduction and Problem Description

HEVC, MV-HEVC and SHVC support two types of parallel processing tools to make the codec more parallel-friendly, namely tiles and WPP.

The presence of above parallel tools across layers can be derived currently by parsing the active PPS of each layer; however, this is not a straightforward process and introduces delay as the active PPS is known only after the activation process, and finding all the relevant PPSs and parsing them can need quite some parsing efforts. Indicating the presence of above information across layers in the VPS VUI is an much easier way for decoders to know whether or not parallel tools are being used and how are they are being used.

It is asserted that configurations of tiles and WPP across layers affect the pipelining design and implementation of the codec. The pipeline design (allocation of resources across layers) needs to change for different configurations of tiles and WPP, as a result, the memory, cycles and the delay requirements are different. Hence, it would be advantageous to the decoder to know at an earlier point in time (e.g. VPS-level) the configurations of tiles and WPP across layers in the CVS.

When tiles are used across layers for parallelism, loop filters can be applied in two options: cross tile boundaries or not. If loop filter may cross the tile boundary it affects the parallelism across tiles and introduces a delay whereas it may also be argued that loop filter across the tile boundaries helps improve video quality. Hence, it would be advantageous to the decoder to know at an earlier point in time (e.g. VPS-level) whether loop filter cross the tile boundaries or not.

# Proposed Solution

| vps_vui( ){ | Descriptor |
| --- | --- |
| … |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { | u(1) |
| tiles_enabled_vps_flag[ i ] |  |
| if( tiles_enabled_vps_flag[ i ] ) |  |
| loop_filter_across_tiles_disabled_vps_flag[ i ] | u(1) |
| entropy_coding_sync_enabled_vps_flag[ i ] | u(1) |
| } |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < NumDirectRefLayers[ layer_id_in_nuh[ i ] ]; j++ ) { |  |
| if( tiles_enabled_vps_flag[ i ] ) |  |
| tile_boundaries_aligned_flag[ i ][ j ] | u(1) |
| ... |  |

tiles_enabled_vps_flag[ i ] equal to 1 indicates that the value of tiles_enabled_flag is equal to 1 for each PPS that is referred to by at least one picture of the i-th layer specified by the VPS. tiles_enabled_vps_flag[ i ] equal to 0 indicates that the value of tiles_enabled_flag is equal to 0 for each PPS that is referred to by at least one picture of the i-th layer specified by the VPS.

loop_filter_across_tiles_disabled_vps_flag[ i ] equal to 1 indicates that the value of loop_filter_across_tiles_enabled_flag is equal to 0 for each PPS that is referred to by at least one picture of the i-th layer specified by the VPS. loop_filter_across_tiles_enabled_vps_flag[ i ] equal to 0 indicates that the above constraint may or may not apply.

entropy_coding_sync_enabled_vps_flag[ i ] equal to 1 indicates that the value of entropy_coding_sync_enabled_flag is equal to 1 for each PPS that is referred to by at least one picture of the i-th layer specified by the VPS. entropy_coding_sync_enabled_vps_flag[ i ] equal to 0 indicates that the value of entropy_coding_sync_enabled_flag is equal to 0 for each PPS that is referred to by at least one picture of the i-th layer specified by the VPS.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).