| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0227r1 |
| --- | --- |

| Title: | SCE2 : On key pictures in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Jianle Chen Vadim Seregin Ye-Kui Wang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-658-2042 krapaka@qti.qualcomm.com cjianle@qti.qualcomm.com vseregin@qti.qualcomm.com yekuiw@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes a design to enable using the decoded pictures from higher quality layer as reference for lower layer pictures for SNR scalability in SHVC. The document proposes

- Signaling a flag in VPS to specify that immediate higher layer picture is used as reference for lower layer

- The concept of key picture to reduce error drift for base layer decoding

- Reference picture set (RPS) and reference picture list (RPL) initialization process is modified for base layer pictures to include BL reference picture into the RPL and use it as collocated picture for TMVP derivation. This is to ensure that there is no drift introduced to motion information. The included BL reference picture into the RPL is not used for sample prediction.

With respect to SHM 3.0.1 temporal scalability anchor, the proposed method reports an average of −3.1%, −3.3% and −3.5% luma BD-rate reductions (EL + BL) for RA-SNR, LD-B and LD-P SNR scalability cases respectively. It is also reported that over SHM 3.0.1 anchor an average of −2.9%, 1.3% and −0.2% luma BD-rate reductions (EL + BL) is obtained for RA-SNR, LD-B and LD-P SNR scalability cases respectively.

In revision 1 of this proposal, a bitstream restriction is proposed on the included BL reference picture in the RPL.

# Introduction and Problem Statement

Various concepts had been studied for trading off enhancement layer coding efficiency and error drift for packet-based quality scalable coding. Figure 1 below shows two such concepts as an example

Figure 1: Example concepts for trading off enhancement layer coding efficiency and drift for packet-based quality scalable coding

Figure 1 (a) employs two motion compensation loops similar to SHVC. The base layer is not influenced by packet losses in the enhancement layer.

Figure1 (b) allows the flexibility to use higher layer decoded pictures (with high quality) as reference for lower layers pictures. This mechanism improves the coding efficiency compared to the method in Figure 1 (a), however the base layer is influenced by packet losses in the enhancement layer and introduces drift. The drift describes the effect that the motion-compensated prediction loops at encoder and decoder are not synchronized, e.g., because quality refinement packets are discarded from a bit stream. To reduce the drift, “key picture” (marked by the black-framed boxes) are used which don’t use higher layer decoded picture (with high quality) as reference.

Currently, SHVC does not allow the use of higher layer decoded picture (with high quality) as reference for lower layers pictures. This contribution proposes to enable such scheme to achieve better coding efficiency for SNR scalability. The concept of key picture is applied in this proposal to achieve the good trade-off between enhancement layer coding efficiency and base layer error drift.

# Proposed methods

This contribution proposes to allow use of higher layer decoded pictures as reference pictures for lower layer pictures. To allow this flexibility and to keep the design simple, following are proposed

- Signaling a flag in VPS to specify that immediate higher layer picture, when available, is used as reference for lower layer picture.

- The concept of key picture is introduced to reduce error drift for base layer decoding. The key pictures only use the decoded key pictures of the same layer as reference. For simplicity, the pictures with temporal ID equal to 0 are inferred to be a key picture in the current design.

- To ensure that there is no base layer error drift introduced by motion information, the reference picture set (RPS) and reference picture list (RPL) initialization scheme is modified for non-key pictures to include BL reference picture into the RPL. The included BL reference picture into the RPL is not used for sample prediction.

- Higher layer reference pictures for lower layer pictures is allowed only for SNR scalability

Since the proposed method is only applied to SNR scalability, it does not introduce any low level decoding process modification as well as other noticeable complexity. The reference to the higher layer decoded picture can be easily fulfilled by changing the entry of reference picture set.

# Experimental Results

In the proposed method, the picture with temporal ID equal to 0 is inferred to be a key picture, which only uses the decoded key picture of the same layer as reference for inter prediction. Therefore, the proposed method requires to be enabled together with temporal scalability so that the pictures with temporal ID larger than 0 can use the higher layer picture as the references for inter layer prediction.

In this section, the proposed method is experimentally verified for two anchor configurations specified in SCE2.

Anchor configuration 1: SHM 3.0.1 with temporal scalability enabled as in JCTVC-L0322

Anchor configuration 2: SHM 3.0.1 CTC

Table 1: Proposed method Vs. Anchor configuration 1

Table 2: BL only : Proposed method Vs. Anchor configuration 1

Table 3: Proposed method Vs. Anchor configuration 2

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).