| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0228 |
| --- | --- |

| Title: | AHG9: Signaling lossless slices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Yu-Wen Huang, Shawmin Lei No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 {shih-ta.hsiang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the high level syntax modifications to support signaling a lossless representation for an entire slice. As a result, the use of losslessly coding a whole slice can be compactly signaled by a syntax flag. The irrelevant syntax information such as the SAO parameters, quantization parameters, and deblocking filter parameters can be explicitly removed from the bitstreams. It further enables us to exclude any tools and resources that are only utilized in the lossy coding mode when it is indicated the entire slice is losslessly coded. For example, we can reduce the number of the modeling contexts for entropy coding syntax significant_coeff_flag from 44 to 2 for losslessly coding a slice, leading to potential power consumption savings.

# Introduction

The current HEVC RExt draft is capable of achieving a lossless video representation to fully recover the coded source video. It is accomplished by bypassing the transform and quantization steps and skipping deblocking, sample adaptive offset (SAO), and transform coefficient sign hiding. The lossless coding mode can be applied to individual coding units (CUs) and is signaled by cu_transquant_bypass_flag for each CU. The lossless coding mode can be enabled by the syntax transquant_bypass_enabled_flag in the picture parameter set (PPS).

# Proposed method

For lossless video coding applications, it is quite often that an entire picture or slice is coded in a lossless representation. In this case, we then need to code cu_transquant_bypass_flag equal to 1 for each CU using the current HEVC design. This contribution proposes the high level syntax modifications to support signaling a lossless video representation for an entire slice. In the proposed method, a new syntax element cu_transquant_bypass_forced_present_flag is added to the HEVC PPS extension when the PPS syntax element transquant_bypass_enabled_flag is equal to 1. cu_transquant_bypass_forced_present_flag equal to 1 indicates that the syntax element cu_transquant_bypass_forced_flag is present in the independent slice segment header. cu_transquant_bypass_forced_flag equal to 1 indicates that an entire current slice is coded in a lossless representation and cu_transquant_bypass_flag is thus skipped and inferred to be equal to 1 for each CU. It also bypasses coding those syntax elements not being applied to lossless coding such as SAO parameters, deblocking filter parameters, and quantization parameters.

Both cu_transquant_bypass_forced_present_flag and cu_transquant_bypass_forced_flag are inferred to be 0 when not present. When cu_transquant_bypass_flag is not coded, its value is inferred to be 1 if both transquant_bypass_enabled_flag and cu_transquant_bypass_forced_flag are equal to 1, and inferred to be 0 otherwise. The proposed syntax modifications will not make any change to the bitstreams under the lossy coding mode (with transquant_bypass_enabled_flag equal to 0). The proposed syntax modifications to the HEVC RExt draft are provided in the attached document.

# Conclusion and recommendation

This contribution proposes the high level syntax modifications to support signaling a lossless video representation for an entire slice. The proposal provides a simple mechanism to clearly and compactly indicate that an entire slice is coded in a lossless representation. As such, when it is signalled that a slice is losslessly coded, those resources dedicated to lossy coding can be saved and irrelevant syntax information to lossless coding from the bitstream can be removed. We recommend that our proposal be adopted into the next HEVC RExt draft.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).