| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0230 |
| --- | --- |

| Title: | RExt: Chroma Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexis Michael Tourapis David Flynn David Singer | Email: | atourapis@apple.com dflynn@blackberry.com singer@apple.com |
| Source: | Apple Inc. and BlackBerry Ltd. |  |  |

_____________________________

# Abstract

In JCTVC-N0292 and JCTVC-O0044 a method was presented that allows for finer control of the quantization process of chroma information down to the CU level. However, currently the HEVC specification takes in consideration only the PPS level chroma QP offsets, as well as the CU luma QP parameters for the determination of the chroma components deblocking strength for the purposes of in-loop filtering. This contribution proposes to, assuming JCTVC-O0044 is adopted, allow the consideration of the additional CU level quantization information during the deblocking process.

# Introduction

The current HEVC specification takes into consideration only the PPS level chroma QP offsets, in addition to the CU luma QP parameters, for the deblocking of reconstructed chroma samples. In particular, in section 8.7.2.5.5 the index qPi that controls the deblocking process is defined as:

qPi = ((QpQ + Qpp + 1) >> 1) + cQpPicOffset

where QpQ and Qpp are the corresponding luma QP values of the coding units that include the coding blocks containing the chroma samples to be processed, while cQpPicOffset corresponds to the QP offset for the current chroma component that has been signalled in the currently active PPS, i.e. pps_cb_qp_qp_offset or pps_cr_qp_offset. Even though HEVC also permits signalling of additional chroma QP offsets at the slice level, these are not considered during deblocking. This was apparently done in an effort to reduce complexity and memory requirements especially at the decoder. Nevertheless, as a, likely unintentional, side-effect, this limitation also enabled the possibility of separately controlling the deblocking filter strength of the chroma components versus that of the luma (by appropriately allocating the desired overall chroma QP offset between the PPS and slice header signalling).

JCTVC-N0292 and JCTVC-O0044, however, proposed a further adjustment of the chroma QP offset, which now can be performed at the CU level. These adjustments can further enhance coding performance since different chroma characteristics from region to region can be better exploited. One such case, is, for example the encoding of mixed screen/video content. In areas categorized as screen areas, pristine chroma quality may be more desirable, while in video areas higher quantization could be used since masking from luma and due to the characteristics of the signal may be easier. Nevertheless, and although this method resulted in more significant differences between chroma and luma QP values from one region to another, no update to the deblocking process was proposed.

In this contribution it is proposed that the signalled chroma QP offset at the CU level should also, optionally, be considered during the deblocking process, in addition to the chroma QP offset signalled at the PPS. Given the flexibility that the slice header level chroma QP offset provides, in allowing separate control between luma and chroma deblocking, and to retain backwards compatibility with existing profiles, no change in that regard is proposed.

# Proposed Text Changes

### 7.3.6 Slice segment header syntax

#### 7.3.6.1 General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( !dependent_slice_segment_flag ) { |  |
| ... |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( chroma_qp_adjustment_enabled_flag ) { |  |
| slice_chroma_qp_adjustment_enabled_flag | u(1) |
| if (slice_chroma_qp_adjustment_enabled_flag) |  |
| chroma_qp_adjustment_for_deblocking_enabled_flag | u(1) |
| } |  |
| ... |  |
| } |  |
| ... |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

#### 7.4.7.1 General slice segment header semantics

slice_chroma_qp_adjustment_enabled_flag equal to 1 specifies that the cu_chroma_qp_adjustment_flag may be present in the transform unit syntax. cu_chroma_qp_adjustment_enabled_flag equal to 0 specifies that the cu_chroma_qp_adjustment_flag is not present in the transform unit syntax.

When not present, the value of slice_chroma_qp_adjustment_enabled_flag shall be inferred to be equal to 0.

chroma_qp_adjustment_for_deblocking_enabled_flag equal to 1 specifies that the chroma_qp_adjustment[] information shall be considered during deblocking information if these are indexed by a coding unit during decoding. chroma_qp_adjustment_for_deblocking_enabled_flag equal to 0 specifies that no chroma_qp_adjustment information will be considered during deblocking.

When not present, the value of chroma_qp_adjustment_for_deblocking,enabled_flag shall be inferred to be equal to 0.

#### 8.7.2.5.5 Fitering process for chroma block edges

…..

The variable QpC  is determined as specified in Table 8-9 based on the index qPi derived as follows:

qPaDeblock = chroma_qp_adjustment_for_deblocking_enabled_flag ? qPa : 0

qPi = ( ( QpQ  + QpP  + 1 ) >> 1 ) + cQpPicOffset  + qPaDeblock 			 (8-323)

# References

- D. Flynn, N. Nguyen, and D. He, “Rext: Fidelity adaptive coding mode,” JCTVC-N0292, JCT-VC, Jul. 2013.

- D. Flynn et al, “RExt: CU-adaptive chroma QP offsets,” JCTVC-O0044, JCT-VC, Oct. 2013

# Patent rights declaration(s)

Apple Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

BlackBerry Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).