| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0232r1 |
| --- | --- |

| Title: | On intra block copying in RExt |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu, Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA Ohji Nakagami Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com ali.tabatabai@am.sony.com +81-50-3750-2746 ohji.nakagami@jp.sony.com |
| Source: | Sony Electronics Inc. Sony Corporation |  |  |

_____________________________

# Abstract

Intra block copy (IntraBC) is studied from implementation point of view in this contribution. For the clean-up and bug-fix, several modifications are proposed.

- It is proposed to clarify the BvIntra range restriction, especially for the vertical component.

- It is proposed to modify MV coding of IntraBC.

- It is proposed to change the interaction of IntraBC and constrained_intra_pred_flag.

# Comments on BvIntra range

In JCTVC-N1005_v3, BvIntra range is specified as follows. (Red emphasized is by authors)

The variable BvIntra[ x0 ][ y0 ][ compIdx ] specifies the vector used for the intra block copying prediction mode.  The value of BvIntra[ x0 ][ y0 ] shall be in the range of 0 to 128, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 1), MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 2), BvIntra[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

It is asserted that the upper CTU line pixels can be referred by IntraBC according to the current restriction. From implementation point of view, it is a great concern since it implies forcing to run the in-loop filter process frame by frame.

For CTU-based in-loop filtering architecture, the upper CTU line pixels are stored in the DPB, which is usually located on DRAM, after the in-loop filtering is processed. Intermediate pixels like before in-loop filtering are not available for the current CTU. If the intermediate pixels in the upper CTU line are required by IntraBC, significant DRAM bandwidth increase to store and to load them is inevitable. Or, some expensive cache memory is necessary.

On the other hand, in HM-12.0+RExt-4.1 encoder, BvIntra search range is restricted not to access the upper CTU line pixels. The IntraBC performance is measured under the constraint.

Thus, it is proposed to clarify the restriction in the text as follows to align the reference software. Yellow highlighted is to emphasize the constraint not to use the upper CTU line pixels.

The value of BvIntra[ x0 ][ y0 ][ 0 ] shall be in the range of –( 2 * CtbSizeY ) to (CtbSizeY – nCbS).

The value of BvIntra[ x0 ][ y0 ][ 1 ] shall be in the range of −( y0 % CtbSizeY ) to (CtbSizeY – nCbS).

The reference pixels shall be available for the any BvIntra value which satisfies the above restriction.

For the horizontal component, it is recommended to restrict further based on the discussion in terms of coding efficiency.

Note: In order to align BvIntra coding improvement proposal in section 2, following text is suggested since BvIntra is mvd value.

In 8.4.4.2.7	Specification of intra block copying prediction mode,

It is a requirement of bitstream conformance for the input bitstream that the reconstructed vector for the intra block copying prediction mode sutisfis the following restriction.

bvIntra[ 0 ] shall be in the range of –( 2 * CtbSizeY ) to (CtbSizeY – nCbS).

bvIntra[ 1 ] shall be in the range of −(yTbCmp % CtbSizeY) to (CtbSizeY – nCbS).

# Redundancy in MV coding

In RExt4.1, the motion vector of intra block copying mode is coded using the MVD coding scheme directly. Because of the causality of Intra block copying, the motion vector values cannot be arbitrary in the search range. For example, a CU cannot copy a reference block which overlaps with itself. So there are some invalid motion vectors as shown in Figure 1.

Figure 1 Diagram of intra block copying

Those invalid motion vectors lead to redundancy in MV coding of intra block copying mode. To remove the redundancy, the follows solutions are proposed.

## Solution 1

As demonstrated in Figure 1, the absolute values of horizontal (MVx) and vertical (MVy) component of motion vector in intra block copying mode cannot be both less than the size of current CU. In the other words, if MVx is less than CU width(W), |MVy| has to be larger than the height (H) of current CU. |MVy| can be reduced to save bit in coding the value.

- At encoder side, before we encode MV i.e.(MVx, MVy), the following applies

  - If abs(MVx)<W, MVy+=H

- At decoder side, after we parse MV from bitstream, the following applies

  - If abs(MVx)<W, MVy-=H

### Text changes of Solution 1

The proposed text changes are highlighted in yellow based on JCTVC-N1005_v3.doc as follows.

### 8.4.4.2.7 Specification of intra block copying prediction mode

Inputs to this process are:

–	a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable nTbS specifying the transform block size,

–	a variable bvIntra specifying the block copying vector,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The variable bv representing the block vector for prediction in full-sample units is derived as follows:

bv[ 0 ] = bvIntra[ 0 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubWidthC ) − 1 )	(8-1)

bv[ 1 ] = ( abs(bvIntra[ 0 ]) < nTbs ? bvIntra[ 1 ] - nTbs : bvIntra[ 1 ]   ) >> ( ( ( cIdx  = =  0 ) ? 1 : SubHeightC ) − 1 )	(8-2)

The (nTbS)x(nTbS) array of predicted samples samples, with x, y = 0..nTbS − 1, are derived as follows:

–	The reference sample location ( xRefCmp, yRefCmp ) is specified by:

( xRefCmp, yRefCmp ) = ( xTbCmp + x + bv[ 0 ], yTbCmp + y + bv[ 1 ] )	(8-3)

- Each sample at the location ( xRefCmp, yRefCmp ) is assigned to predSamples[ x ][ y ].

## Solution 2

Alternatively, the condition can be based on MVy and modification is applied to MVx for saving bits.

- At encoder side, before we encode MV i.e.(MVx, MVy), the following applies

  - If abs(MVy)<H, MVx+=W

- At decoder side, after we parse MV from bitstream, the following applies

  - If abs(MVy)<H, MVy-=W

### Text changes of Solution 2

The proposed text changes are highlighted in yellow based on JCTVC-N1005_v3.doc as follows.

### 8.4.4.2.7 Specification of intra block copying prediction mode

Inputs to this process are:

–	a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable nTbS specifying the transform block size,

–	a variable bvIntra specifying the block copying vector,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The variable bv representing the block vector for prediction in full-sample units is derived as follows:

bv[ 0 ] = ( abs(bvIntra[ 1 ]) < nTbs ? bvIntra[ 0 ] - nTbs : bvIntra[ 0 ]   )  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubWidthC ) − 1 )	(8-4)

bv[ 1 ] = bvIntra[ 1 ]  >> ( ( ( cIdx  = =  0 ) ? 1 : SubHeightC ) − 1 )	(8-5)

The (nTbS)x(nTbS) array of predicted samples samples, with x, y = 0..nTbS − 1, are derived as follows:

–	The reference sample location ( xRefCmp, yRefCmp ) is specified by:

( xRefCmp, yRefCmp ) = ( xTbCmp + x + bv[ 0 ], yTbCmp + y + bv[ 1 ] )	(8-6)

- Each sample at the location ( xRefCmp, yRefCmp ) is assigned to predSamples[ x ][ y ].

## Solution 3

MVD coding scheme assumes a symmetric distribution of syntax elements, while motion vectors of intra block copying mode demonstrate a shifted peak at MVx to the negative region based on RExt4.1 as shown in Figure 2. It is proposed to simply shift the MVx without conditions.

Figure 2 Example of statistics for MV in intra block copying

### Text changes of Solution 3

The proposed text changes are highlighted in yellow based on JCTVC-N1005_v3.doc as follows.

### 8.4.4.2.7 Specification of intra block copying prediction mode

Inputs to this process are:

–	a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable nTbS specifying the transform block size,

–	a variable bvIntra specifying the block copying vector,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The variable bv representing the block vector for prediction in full-sample units is derived as follows:

bv[ 0 ] = ( bvIntra[ 0 ] - nTbs )  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubWidthC ) − 1 )	(8-7)

bv[ 1 ] = bvIntra[ 1 ]  >> ( ( ( cIdx  = =  0 ) ? 1 : SubHeightC ) − 1 )	(8-8)

The (nTbS)x(nTbS) array of predicted samples samples, with x, y = 0..nTbS − 1, are derived as follows:

–	The reference sample location ( xRefCmp, yRefCmp ) is specified by:

( xRefCmp, yRefCmp ) = ( xTbCmp + x + bv[ 0 ], yTbCmp + y + bv[ 1 ] )	(8-9)

- Each sample at the location ( xRefCmp, yRefCmp ) is assigned to predSamples[ x ][ y ].

## Simulation results

The proposed solutions are implemented based on RExt4.1. Simulations are conducted following the test conditions used for RCE3[1]. Encoding and decoding time is not available because simulations are run in a heterogeneous cluster.

Summary of simulation results are listed as follows. Detailed results are available in accompanying excel files.

### Results of solution 1

Table 1 Results summary of lossy case for Solution 1

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.2% | -0.2% | -0.2% | -0.2% | -0.1% | 0.0% | -0.2% | -0.1% | 0.0% |
| Animation YUV 444 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -0.5% | -0.5% | -0.6% | -0.5% | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% |
| SC(444) YUV Optional | -0.6% | -0.6% | -0.6% | -0.4% | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -0.5% | -0.6% | -0.6% | -0.5% | -0.4% | -0.5% |  |  |  |
| SC(444) YUV Optional | -0.9% | -0.8% | -0.9% | -0.5% | -0.5% | -0.6% |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.8% | -0.4% | 0.0% | -0.4% | -0.2% |  |  |  |
| Class B | 0.0% | -0.3% | -0.3% | 0.0% | -0.1% | -0.1% |  |  |  |
| SC RGB 444 | 0.3% | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.1% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% |  |  |  |
| Animation YUV 444 | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | 0.8% | 0.7% | 0.6% | 0.2% | 0.2% | 0.1% |  |  |  |
| SC(444) YUV Optional | -1.8% | -1.6% | -1.7% | -0.2% | 0.0% | -0.3% |  |  |  |

Table 2 Results summary of lossless case for Solution 1

### Results of solution 2

Table 3 Results summary of lossy case for Solution 2

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.2% | -0.2% | -0.2% | -0.2% | -0.1% | 0.0% | -0.2% | -0.1% | 0.0% |
| Animation YUV 444 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -0.5% | -0.5% | -0.6% | -0.5% | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% |
| SC(444) YUV Optional | -0.6% | -0.6% | -0.6% | -0.4% | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -0.5% | -0.6% | -0.6% | -0.5% | -0.4% | -0.5% |  |  |  |
| SC(444) YUV Optional | -0.9% | -0.8% | -0.9% | -0.5% | -0.5% | -0.6% |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.8% | -0.4% | 0.0% | -0.4% | -0.2% |  |  |  |
| Class B | 0.0% | -0.3% | -0.3% | 0.0% | -0.1% | -0.1% |  |  |  |
| SC RGB 444 | 0.3% | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.1% | -0.2% | -0.3% | -0.1% | -0.1% | -0.1% |  |  |  |
| Animation YUV 444 | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | 0.8% | 0.7% | 0.6% | 0.2% | 0.2% | 0.1% |  |  |  |
| SC(444) YUV Optional | -1.8% | -1.6% | -1.7% | -0.2% | 0.0% | -0.3% |  |  |  |

Table 4 Results summary of lossless case for Solution 2

### Results of solution 3

Table 5 Results summary of lossy case for Solution 3

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% | -0.3% |
| Animation RGB 444 | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.7% | -0.6% | -0.7% | -0.6% | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% |
| Animation YUV 444 | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -0.2% | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% | -0.1% | -0.1% |
| SC(444) YUV Optional | -0.6% | -0.6% | -0.5% | -0.4% | -0.5% | -0.4% | -0.2% | -0.4% | -0.3% |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -0.3% | -0.4% | -0.3% | -0.3% | -0.3% |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -0.2% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.5% | -0.5% | -0.4% | -0.3% | -0.3% | -0.3% |  |  |  |
| Animation YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | 0.0% |  |  |  |
| SC(444) YUV Optional | -0.9% | -0.6% | -0.9% | -0.7% | -0.4% | -0.6% |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.5% | -0.9% | -0.2% | -0.3% | -0.5% |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | 0.2% | 0.6% | 0.4% | 0.2% | 0.7% | 0.5% |  |  |  |
| Animation RGB 444 | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.5% | -0.4% | -0.5% | -0.3% | -0.1% | -0.2% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.2% |  |  |  |
| SC(444) GBR Optional | 0.3% | 0.4% | 0.3% | 0.8% | 1.0% | 0.7% |  |  |  |
| SC(444) YUV Optional | -0.9% | -0.5% | -0.8% | -0.4% | 0.3% | -0.2% |  |  |  |

Table 6 Results summary of lossless case for Solution 3

# Interaction with constrained_intra_pred_flag

constrained_intra_pred_flag in HEVC version 1 restricts the references to non-Intra coded CU for the purpose of error resilience. Because of the adoption of intra block copying mode, the references to CU with intra block copying mode is allowed in the current RExt text(JCTVC-N1005_v3.doc). However, this may break the original intent for error resilience of constrained_intra_pred_flag. For example, in a P/B slice, current CU has a neighboring CU (“A” in Figure 3) which is intra block copying mode and may refer to an inter mode CU in “A”’s search range. Thus, due to the reference chain from current CU to “A”, and “A” to its references, errors may propagate from lost temporal picture to “A” and eventually to current CU.

Figure 3 Intra block copying and constrained_intra_pred_flag

To align with the original intent of constrained_intra_pred_flag, it is proposed the following solutions for P/B slices.

- Intra block copying motion vector shall not point where the reference pixels are derived from Intra block copying or Inter mode. (encoder solution)

- Reference pixels are unavailable if they are derived from Intra block copying or Inter mode. (decoder solution)

## Text changes (encoder solution)

The proposed text changes are highlighted in yellow based on JCTVC-N1005_v3.doc as follows.

#### 7.4.9.9Motion vector difference semantics

The variable BvIntra[ x0 ][ y0 ][ compIdx ] specifies the vector used for the intra block copying prediction mode.  The value of BvIntra[ x0 ][ y0 ] shall be in the range of 0 to 128, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 1), MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 2), BvIntra[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

Note: An encoder should avoid Intra block copying mode with motion vectors BvIntra[ x0 ][ y0 ][ compIdx ] that point where the reference pixels are derived from CTB with Intra block copying or Inter mode.

## Text changes (decoder solution)

The proposed text changes are highlighted in yellow based on JCTVC-N1005_v3.doc as follows.

##### 8.4.4.2.1 General intra sample prediction

Inputs to this process are:

–	a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable predModeIntra specifying the intra prediction mode,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The nTbS * 4 + 1 neighbouring samples p[ x ][ y ] that are constructed samples prior to the deblocking filter process, with x = −1, y = −1..nTbS * 2 − 1 and x = 0..nTbS * 2 − 1, y = −1, are derived as follows:

–	The neighbouring location ( xNbCmp, yNbCmp ) is specified by:

( xNbCmp, yNbCmp ) = ( xTbCmp + x, yTbCmp + y )	(8-10)

- The current luma location ( xTbY, yTbY ) and the neighbouring luma location ( xNbY, yNbY ) are derived as follows:

( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTbCmp, yTbCmp ) : ( xTbCmp * SubWidthC, yTbCmp * SubHeightC )	(8-11)

( xNbY, yNbY ) = ( cIdx  = =  0 ) ? ( xNbCmp, yNbCmp ) : ( xNbCmp * SubWidthC, yNbCmp * SubHeightC )	(8-12)

- The availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ) and the neighbouring luma location ( xNbY, yNbY ) as inputs, and the output is assigned to availableN.

–	Each sample p[ x ][ y ] is derived as follows:

- If one or more of the following conditions are true, the sample p[ x ][ y ] is marked as "not available for intra prediction":

  - The variable availableN is equal to FALSE.

  - CuPredMode[ xNbY ][ yNbY ] is not equal to MODE_INTRA and constrained_intra_pred_flag is equal to 1.

  - Slice_type is equal to P and intra_bc_flag[ xNbY ][ yNbY ] is equal to 1 and constrained_intra_pred_flag is equal to 1.

  - Slice_type is equal to B and intra_bc_flag[ xNbY ][ yNbY ] is equal to 1 and constrained_intra_pred_flag is equal to 1.

- Otherwise, the sample p[ x ][ y ] is marked as "available for intra prediction" and the sample at the location ( xNbCmp, yNbCmp ) is assigned to p[ x ][ y ].

# Conclusion

Several modifications for clean-up and bug-fix are proposed in this contribution. Coding efficiency and functionality are enhanced. It is recommended to adopt those changes to working draft of RExt.

# Reference

- A. Saxena, D. Kwon, M. Naccari, C. Pang,” HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques”, JCTVC-N1123, Vienna, Austria, July 2013.

# Patent rights declaration(s)

Sony Corporation and Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).