| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0233r1 |
| --- | --- |

| Title: | Motion Compensation Interpolation with 16 Bit Intermediate Buffer for HEVC Range Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu, Jianle Chen, Woo-Shik Kim, Marta Karczewicz, Joel Sole, Liwei Guo 5775 Morehouse Drive San Diego, CA, USA 92121 | Tel: Email: | 1-858-651-7749 wpu@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

HEVC version 1 describes 8 bit and 10 bit codec. The bit precision of internal intermediate data is controlled within 16 bit to improve memory bandwidth utilization and reduce computational complexity. For HEVC Range Extension, up to 16 bit video is supported. In current HEVC Range Extension design, the intermediate buffer of motion compensation interpolation module reaches up to 20 bit to retain high intermediate data accuracy. In this proposal, a method is proposed to keep the motion interpolation module’s internal data within 16 bit without sacrificing the coding performance.

# Introduction

According to the current specification of HEVC Range Extension [1], the 2D separable luma interpolation process is specified as follows.

Figure 1 – Integer samples (shaded blocks with upper-case letters) and fractional sample positions (un-shaded blocks with lower-case letters) for quarter sample luma interpolation.

In Figure 1, the positions labeled with upper-case letters Ai, j within shaded blocks represent luma samples at full-sample locations in the reference picture. These samples are used for generating the predicted luma sample value at the fractional position.

In section 8.5.3.3.2 in [1], the variables shift1, shift2, and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6, and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

Detailed operation for interpolation is included in Appendix. Generally speaking, the interpolation process is first performed horizontally and then vertically. For each direction, 8-tap filters are used to derive sample values at fractional pixel positions. To reduce the internal data’s dynamic range, after horizontal filtering, the output data is right shifted (shift1) bits before starting the vertical filtering. Similarly, after vertical filtering, a right shift of (shift2) bits is applied.

In above process, the variable shift1 is the right shift bits after horizontal (first step) interpolation process and shift2 is the right shift bits of horizontal (second step) interpolation process. When the input video bit depth is equal to or less than 12 bit, the intermediate data is within the 16 bit range. If the input is high bit depth video (e.g. 14 or 16 bit), the interpolated samples can be out of a 16 bit dynamic range. For example, in Appendix (8-203), the sum of the positive filter coefficients is 4 + 40 + 40 + 4 = 88 (7 bit). If the input is 16 bit, shift1 is equal to 4. The largest value b0,0 can reach is:

(((1 << 16) – 1) * 88) >> 4 = (57FFA)16 19 bit

As there is negative filter coefficients in (8-203), b0,0 can also be negative. Therefore, 20 bits are required to represent each interpolated data. Such high intermediate data’s bit depth can affect the codec’s throughput significantly.

# Proposed Method

In the proposed method, the clipping operation and right shift are used together to control the intermediate data range of the interpolation filter within 16 bit. The 16 bit clipping operation is defined as:

Clip16( x ) = Clip3( 0, 65535, x )

For high bit depth (≥14 bit) input, the variable shift1 is set equal to BitDepth − 10, the variable shift2 is set equal to 6, and the variable shift3 is set equal to 16 − BitDepth. If input bit depth is less than 14 bit, specifications in Section 8.5.3.3.3.2 [1] is applied. The clipping operation is applied on the intermediate output of horizontal interpolation and vertical interpolation filter when the input bit depth is larger than 12 bit. For example, for Appendix (8-202), the proposed method can be expressed as:

a0,0 = Clip16 ( ( −A−3,0 + 4 * A−2,0 − 10 * A−1,0 + 58 * A0,0 +	 17 * A1,0 − 5 * A2,0 + A3,0 )  >>  shift1)       	(8-1’)

The same operation is applied to (8-203) to (8-216) and also the chroma interpolation process.

By applying the proposed method, all the intermediate data can be kept within the dynamic range of 16 bit integer when the input or internal bit-depth is up to 16 bit, which can facilitate implementation and reduce complexity.

# Simulation Results

## Test Condition

The proposed method is implemented on top of HM12.0_RExt4.1. The test condition in this proposal is defined as follows:

- ‘GBR’ ordering using the 1080p variants of the following five SVT sequences (16-bit precision, 50fps). Use frame count equivalent to 100.

- CrowdRun

- ParkJoy

- DucksTakeOff

- IntoTree

- OldTownCross

- For each sequence, internal bit-depths to be tested are: 14 and 16-bits.

- QPs according to internal bit-depth as follows:

  - 16-bit: −28, −31, −34, −37

  - 14-bit: −16, −19, −22, −25

- MAX_TRANSFORM_DYNAMIC_RANGE equal to max(15, bitDepth + 6)

- ENTROPY_CODING_DYNAMIC_RANGE equal to max(15, bitDepth + 6)

- As per AHG5/8 conditions, testing with transform skip enabled for 4x4 blocks only

- Testing with ‘extended precision processing’ enabled with the HM forward transform.

- Software settings:

  - #define RExt__HIGH_BIT_DEPTH_SUPPORT  1

  - Apply “--ExtendedPrecision=1”

  - #define RExt__HIGH_PRECISION_FORWARD_TRANSFORM  0

- RDOQ/RDOQTS enabled

Other settings as per the ‘encoder_lowdelay _rext.cfg’ configuration file (i.e. ‘low delay B’) and ‘encoder_randomaccess_rext.cfg’ configuration file (i.e. ‘random access’).

  - Test Results

The test results are summarized in Tables I and II. Compared with HEVC Range Extension with high precision processing flag enabled, the proposed method shows no noticeable coding efficiency loss, while memory access and computational complexity are significantly reduced.

Table I. 16 bit motion interpolation (proposed) v.s. HM RExt 4.1 Anchor

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 14-bit | 0.0% | 0.0% | 0.0% |
| 16-bit | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Low Delay B |  |  |
|  | Y | U | V |
| 14-bit | 0.0% | 0.0% | 0.0% |
| 16-bit | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 102% |  |  |

In another set of experiment, the high precision processing flag is also enabled. But within the motion interpolation module, high precision processing is disabled. i.e. the interpolation process specified in HEVC version 1 [2] is used. The variable shift1 is set equal to BitDepthY − 8, the variable shift2 is set equal to 6, and the variable shift3 is set equal to 14 − BitDepthY.

# Conclusion

In this contribution, the method to support high bit depth coding with 16 bit intermediate buffer for motion interpolation is proposed. Compared with current HEVC Range Extension with high precision processing flag enabled, no performance degradation was observed while memory access and computation complexity is greatly reduced. Compared with HEVC Range Extension with high precision processing flag on but changing the motion interpolation to HEVC version 1 scheme, on average 1.4% and 0.9% BD-rate reduction for 16 bit RA and LDB were observed.

Table II. 16 bit motion interpolation (proposed) v.s. HM RExt 4.1 Anchor (HEVCv1 Interpolation)

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 14-bit | 0.0% | 0.0% | 0.0% |
| 16-bit | -1.2% | -1.5% | -1.5% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Low Delay B |  |  |
|  | Y | U | V |
| 14-bit | 0.0% | 0.0% | 0.0% |
| 16-bit | -0.7% | -1.0% | -1.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 104% |  |  |

# Reference

- D. Flynn, J. Sole, T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4,” JCTVC-N1005_v3, August 2013.

- “High efficiency video coding,” ITU-T Recommendation Edition 1.0, 2013-04-13.

# Appendix

HEVC Range Extension’s current motion interpolation is specified as follows:

In Figure 1, given the luma samples Ai, j at full-sample locations ( xAi, j, yAi, j ), the luma samples a0,0 to r0,0 at fractional sample positions are derived as follows:

–	The samples labelled a0,0, b0,0, c0,0, d0,0, h0,0, and n0,0 are derived by applying an 8-tap filter to the nearest integer position samples as follows:

a0,0 = ( −A−3,0 + 4 * A−2,0 − 10 * A−1,0 + 58 * A0,0 +	 17 * A1,0 − 5 * A2,0 + A3,0 )  >>  shift1	(8-2)

b0,0 = ( −A−3,0 + 4 * A−2,0 − 11 * A−1,0 + 40 * A0,0 +	 40 * A1,0 − 11 * A2,0 + 4 * A3,0 − A4,0 )  >>  shift1	(8-3)

c0,0 = ( A−2,0 − 5 * A−1,0 + 17 * A0,0 +	 58 * A1,0 − 10 * A2,0 + 4 * A3,0 − A4,0 )  >>  shift1	(8-4)

d0,0 = ( −A0,−3 + 4 * A0,−2 − 10 * A0,−1 + 58 * A0,0 +	 17 * A0,1 − 5 * A0,2 + A0,3 )  >>  shift1	(8-5)

h0,0 = ( −A0,−3 + 4 * A0,−2 − 11 * A0,−1 + 40 * A0,0 +	 40 * A0,1 − 11 * A0,2 + 4 * A0,3 − A0,4 )  >>  shift1	(8-6)

n0,0 = ( A0,−2 − 5 * A0,−1 + 17 * A0,0 +	 58 * A0,1 − 10 * A0,2 + 4 * A0,3 − A0,4 )  >>  shift1	(8-7)

–	The samples labelled e0,0, i0,0, p0,0, f0,0, j0,0, q0,0, g0,0, k0,0, and r0,0 are derived by applying an 8-tap filter to the samples a0,i, b0,i and c0,i with i = −3..4 in the vertical direction as follows:

e0,0 = ( −a0,−3 + 4 * a0,−2 − 10 * a0,−1 + 58 * a0,0 +	 17 * a0,1 − 5 * a0,2 + a0,3 )  >>  shift2	(8-8)

i0,0 = ( −a0,−3 + 4 * a0,−2 − 11 * a0,−1 + 40 * a0,0 +	 40 * a0,1 − 11 * a0,2 + 4 * a0,3 − a0,4 )  >>  shift2	(8-9)

p0,0 = ( a0,−2 − 5 * a0,−1 + 17 * a0,0 +	 58 * a0,1 − 10 * a0,2 + 4 * a0,3 − a0,4 )  >>  shift2	(8-10)

f0,0 = ( −b0,−3 + 4 * b0,−2 − 10 * b0,−1 + 58 * b0,0 +	 17 * b0,1 − 5 * b0,2 + b0,3 )  >>  shift2	(8-11)

j0,0 = ( −b0,−3 + 4 * b0,−2 − 11 * b0,−1 + 40 * b0,0 +	 40 * b0,1 − 11 * b0,2 + 4 * b0,3 − b0,4 )  >>  shift2	(8-12)

q0,0 = ( b0,−2 − 5 * b0,−1 + 17 * b0,0 +	 58 * b0,1 − 10 * b0,2 + 4 * b0,3 − b0,4 )  >>  shift2	(8-13)

g0,0 = ( −c0,−3 + 4 * c0,−2 − 10 * c0,−1 + 58 * c0,0 +	 17 * c0,1 − 5 * c0,2 + c0,3 )  >>  shift2	(8-14)

k0,0 = ( −c0,−3 + 4 * c0,−2 − 11 * c0,−1 + 40 * c0,0 +	 40 * c0,1 − 11 * c0,2 + 4 * c0,3 − c0,4 )  >>  shift2	(8-15)

r0,0 = ( c0,−2 − 5 * c0,−1 + 17 * c0,0 +	 58 * c0,1 − 10 * c0,2 + 4 * c0,3 − c0,4 )  >>  shift2	(8-16)

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).