| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, Switzerland, CN, Oct. 23 – Nov. 2, 2013 | Document: JCTVC-O0234 |
| --- | --- |

| Title: | AHG20: Multilayer HRD Management |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |  |
| Purpose: | Information and Proposal |  |  |  |  |
| Author(s) or Contact(s): | Sam Narasimhan and Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 | Tel: Email: | 1-858-404-3985 sam.narasimhan@arrisi.com ajay.luthra@arrisi.com |  |  |
|  | S. Hattori, K. Sato and T. Suzuki Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan |  | +81 50 3750 2741 shinobu.hattori@jp.sony.com Kazushi.Sato@jp.sony.com teruhiko.suzuki@jp.sony.com |  |  |
|  | Ali Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA |  | 1-408-352-4715 ali.tabatabai@am.sony.com |  |  |
|  | Arris, Sony Corp., and Sony Electronics Inc. |  |  |  |  |

_____________________________

# Abstract

Layer based buffer flow management (CPB bit rate, CPB size, CPB management, DPB management) for a multilayer operation is being investigated. The key issues of this investigation include:

- Layer based CPB buffer flow model

- Syntax and semantics for signaling layer based HRD management

- Scope of buffering period and picture timing SEI messages for cross layer timing and synchronization.

- System level STD operation and issues to support both layer based CPB management multi-layer HRD model

# Introduction

Multi-layer representation of a bitstream to allow temporal, spatial and view enhancements is an inherent characteristic of scalability models as practiced, in HEVC, SHVC and MV-HEVC. In addition, a hybrid scalable model is also introduced allowing the transmission of AVC base layer stream to an AVC base layer decoder while an HEVC enhancement layer is transmitted to an HEVC decoder, separately. Traditional HRD buffer management approaches identify HRD parameters and number of HRD models on the whole i.e., for combined layers as opposed to on a per layer basis. This contribution highlights system level/video level complexities with the traditional HRD buffer management approach and provides potential solutions for layer based buffer management (including HRD management).   .

# Layer Based CPB Buffer Flow Model

Figure 1 shows an example of a multilayer STD buffer flow model from a system operation perspective . As shown, it consists of a PID based demuxing to map a multi-layered trasnport stream into its constituent layers (2 layers in this example). Each layer is then fed into TB & MB buffers to generate elementary streams as inputs to the CPB buffers. Figure 2 shows a generalization of HRD buffer flow model when dealing with more than one layer based on layered CPB and sub-DPB concept introduced in [1].

+

DEMUX

PI Filt.

DEMUX

TB0+MB0 (BL)

TB1+MB1 (EL)

CPB0

CPB1

DPB1

DPB0

Figure 1

CPB1

CPBn

CPB0

Sub_DPB0

Arrival

Times

Removal

Times

Sub_DPB1

Sub_DPBn

Output

Times

Figure 2

HRD buffer management as shown above consists of both CPB as well as DPB buffer management. This proposal describes two methods by which management of layered based CPB is performed.

# Syntax and Semantics of Layer based CPB Management

In this section, two methods are proposed for layer based buffer management. Section 3.1 proposes an extension to VPS to signal layer specific HRD parameters using the hrd_parameters() construct. Section 3.2 proposes a new SEI to signal layer specific CPB parameters such as maximum bit rate and buffer size.

- 3.1 VPS extension to signal layer specific HRD parameters

Following the notion of sub-DPBs in [1] the notion of sub-CPB buffer model is introduced in this proposal by associating each sub-CPB with a given layer. Considering the above buffering flow model an approach is proposed which is based, namely, on the current definitions of layer sets and output layer sets as specified in [2] and also modification of the proposal made in [3].

First a new syntax structure “sub_cpb_parameters_table()” is introduced in vps_extension() syntax structure.

| vps_extension( ) { |  |
| --- | --- |
| … |  |
| sub_cpb_parameters_table( ) |  |
| … |  |
| } |  |

Accordingly, sub_cpb_parameters_table() syntax structure is shown, below:

| sub_cpb_parameters_table () { | Descriptor |
| --- | --- |
| … |  |
| for( i=1; i <= vps_num_layer_sets_minus1; i++) { |  |
| sub_cpb_info_present_flag[ i ] | u(1) |
| if(sub_cpb_info_present_flag[ i ]) { |  |
| for(k=0; k < vps_num_hrd_parameters && i != hrd_layer_set_idx[k] ; k++) |  |
| for( j=1; j <= vps_max_layer_id; j++) { |  |
| if( layer_id_included_flag[ i ][ j ] ) { |  |
| hrd_parameters( cprm_present_flag[ k ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

sub_cpb_info_present_flag[ i ]: sub_cpb_info_present_flag[ i ] = 1 indicates CPB parameters for each layer in the layer set i are given. sub_cpb_info_present_flag[ i ] = 0 indicates CPB parameters for each layer are not present.

3.2 SEI signaling for layer specific CPB parameters

A modified SEI message based on reference [3] is provided below:

| multilayer_cpb_info( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| active_vps_id | 5 | u(4) |
| for( i = 1; i <= vps_max_layer_id; i++ ) { |  |  |
| layer_cpb_info_present_flag[i] | 5 | u(1) |
| if(layer_cpb_info_present_flag[i]) { |  |  |
| layer_max_bitrate_value[ i ] layer_average_bitrate_value[i] | 5 5 | u(16) u(16) |
| layer_max_cpb_size_value[i] | 5 | u(16) |
| } |  |  |
| } |  |  |
| } |  |  |

active_vps_id: identifies an active VPS that contains the layer information. The value of active_vps_id shall be equal to the value of video_parameter_set_id of the active VPS for the VCL NAL units of the access unit containing the SEI message.

layer_cpb_info_present_flag[i]: specifies whether the CPB parameter information about layer index I is included.

layer_max_bit_rate_value[ i ]: specifies the maximum bit rate value of layer index i.

layer_avg_bit_rate_value[ i ]: specifies average bit rate value of layer index i.

layer_max_cpb_size_value[i]: specifies maximum cpb size value of layer index i.

Note : This SEI may be included for a new layer [i] as long as it meets the maximum layer values signaled in the VPS and SPS. This SEI may also be extended to signal a new layer set that this new layer[i] will be part of as well as the profile/level parameters for the new layer set.

# Scope of Buffering Period and Picture Timing SEI Messages

For BP & PT SEI messages two possible solutions are envisioned: First, because of the strict output timing requirements it is proposed to have a single removal time and accordingly a single PT SEI message to be associated with all the layers. BP SEI messages can be however associated with each layer for sub_CPB buffer management. A second possibility is to have both BP and PT SEI messages to be associated with each layer.

# Conclusion

This proposal has investigated some of the issues related to multilayer HRD management both at the systems and video coding layer and has provided methods that show how to address these issues. It is recommended that these methods to be discussed in the meeting for potential adoption.

# References

- A. Ramasubramonian, Y. Chen, Y. Wang, M. Hannuksela and S. Deshpande, “MV-HEVC/SHVC HLS: On DPB Operations“, JCTVC-N0093r2, Vienna, AT, August 2013

- J. Chen, J. Boyce, Y. Ye and M. Hannuksela (Eds.), “SHVC Working Draft 2“, JCTVC-M1008_v3, Incheon, KR, April. 2013

- S. Narasimhan, A. Luthra, K. Sato and A. Tabatabai, “Consideration of buffer management issues HEVC scalability Operations“, JCTVC-N0049r1, Vienna, AT, August 2013

# Patent rights declaration(s)

Arris may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Group and Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IECw patent statement and licensing declaration form).