| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0235r1 M31017 |
| --- | --- |

| Title: | High Precision Weighted Prediction for HEVC Range Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Pu, Jianle Chen, Marta Karczewicz, Woo-Shik Kim, Joel Sole, Liwei Guo 5775 Morehouse Drive San Diego, CA, USA 92121 | Tel: Email: | 1-858-651-7749 wpu@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

For HEVC Range Extension, up to 16 bit video is supported. However, in the current HEVC Range Extension working draft, weighted prediction parameters (i.e. weight and offset) are both restricted to 8 bit precision, which may affect coding efficiency due to the lack of enough precision. A high precision weighted prediction is proposed, which extends the precision of the offset parameter. Simulation results show for fading high bit depth sequences, the high precision weighted prediction can achieve average BD-rate reduction of 1.3% and 2.8% for RA and LD-B, respectively, with reportedly almost no computation complexity increment.

# Introduction

According to the current specification of HEVC Range Extension [1], the weighted prediction offset parameters are derived as follows:

If cIdx is equal to 0 for luma samples, the following applies:

o0 = luma_offset_l0[ refIdxL0 ] * ( 1  <<  ( bitDepth − 8 ) )                                   	(8-1)

o1 = luma_offset_l1[ refIdxL1 ] * ( 1  <<  ( bitDepth − 8 ) )                                   	(8-2)

Otherwise (cIdx is not equal to 0 for chroma samples), the following applies:

o0 = ChromaOffsetL0[ refIdxL0 ][ cIdx − 1 ] * ( 1  <<  ( bitDepth − 8 ) )                        	(8-3)

o1 = ChromaOffsetL1[ refIdxL1 ][ cIdx − 1 ] * ( 1  <<  ( bitDepth − 8 ) )                        	(8-4)

For 16-bit input video, the minimum offset is

1<< (16−8)=256

which is not precise enough to characterize fine global illumination changes.

# Proposed Method

In the proposed method, for high bit depth input (i.e. 14 and 16 bit) the offsets (o0 and o1) have the same precision as the input in order to provide enough precision for the weighted prediction process. The proposed weighted prediction process is specified as:

If cIdx is equal to 0 for luma samples, the following applies:

o0 = luma_offset_l0[ refIdxL0 ]	                                                          	(8-5’)

o1 = luma_offset_l1[ refIdxL1 ]	                                                          	(8-6’)

Otherwise (cIdx is not equal to 0 for chroma samples), the following applies:

o0 = ChromaOffsetL0[ refIdxL0 ][ cIdx − 1 ]	                                               	(8-7’)

o1 = ChromaOffsetL1[ refIdxL1 ][ cIdx − 1 ]	                                               	(8-8’)

# The related syntax elements are updated as:

luma_offset_l0[ i ] is the additive offset applied to the luma prediction value for list 0 prediction using RefPicList0[ i ]. If extended_precision_processing_flag equal to 0, the value of luma_offset_l0[ i ] shall be in the range of −128 to 127, inclusive. If extended_precision_processing_flag equal to 1, the value of luma_offset_l0[ i ] shall be in the range of –(1 << (BitDepthY – 1) to (1 << (BitDepthY – 1) – 1, inclusive. When luma_weight_l0_flag[ i ] is equal to 0, luma_offset_l0[ i ] is inferred as equal to 0.

delta_chroma_offset_l0[ i ][ j ] is the difference of the additive offset applied to the chroma prediction values for list 0 prediction using RefPicList0[ i ] with j equal to 0 for Cb and j equal to 1 for Cr. The variable ChromaOffsetL0[ i ][ j ] is derived as follows:

If extended_precision_processing_flag equal to 0,

ChromaOffsetL0[ i ][ j ] = Clip3( −128, 127, ( delta_chroma_offset_l0[ i ][ j ] −                      	(7-9)
				( ( 128 * ChromaWeightL0[ i ][ j ] )  >>  ChromaLog2WeightDenom ) + 128 ) )

The value of delta_chroma_offset_l0[ i ][ j ] shall be in the range of −512 to 511, inclusive. When chroma_weight_l0_flag[ i ] is equal to 0, ChromaOffsetL0[ i ][ j ] is inferred to be equal to 0.

If extended_precision_processing_flag equal to 1,

ChromaOffsetL0[i][j] = Clip3( –(1 << (BitDepthC – 1), (1 << (BitDepthC – 1) –1, (delta_chroma_offset_l0[i][j] -                 
			(((1 << (BitDepthC – 1)) * ChromaWeightL0[i][j] ) >>  ChromaLog2WeightDenom ) + (1 << (BitDepthC – 1)))					                                                                     (7-10’)

The value of delta_chroma_offset_l0[ i ][ j ] shall be in the range of −(1 << (BitDepthC + 1) to (1 << (BitDepthC + 1) −1, inclusive. When chroma_weight_l0_flag[ i ] is equal to 0, ChromaOffsetL0[ i ][ j ] is inferred to be equal to 0.

# Simulation Results

## Test Condition

The test condition in this proposal is defined as follows. The test condition is based on RCE2 [3]. As RCE2 only tests Intra cases only, the below test condition is updated to test Inter cases.

- ‘GBR’ ordering using the 1080p variants of the following five SVT sequences (16 bit precision, 50fps).

- CrowdRun

- ParkJoy

- DucksTakeOff

- IntoTree

- OldTownCross

- For each sequence, internal bit-depths to be tested are: 14 and 16-bits.

- QPs according to internal bit-depth as follows:

- 16-bit: −28, −31, −34, −37

- 14-bit: −16, −19, −22, −25

- MAX_TRANSFORM_DYNAMIC_RANGE equal to max(15, bitDepth + 6)

- ENTROPY_CODING_DYNAMIC_RANGE equal to max(15, bitDepth + 6)

- As per AHG5/8 conditions, testing with transform skip enabled for 4x4 blocks only

- Testing with ‘extended precision processing’ enabled with the HM forward transform.

- Software settings:

  - #define RExt__HIGH_BIT_DEPTH_SUPPORT  1

  - Apply “--ExtendedPrecision=1”

  - #define RExt__HIGH_PRECISION_FORWARD_TRANSFORM  0

- RDOQ/RDOQTS enabled

Other settings as per the ‘encoder_lowdelay _rext.cfg’ configuration file (i.e. ‘low delay B’) and ‘encoder_randomaccess_rext.cfg’ configuration file (i.e. ‘random access’).

## Fading Sequence Generation

As the 16 bit video sequence’s dynamic range is large, the fading sequence is generated using an exponential fading model, i.e.,

F'(x,y)=k⋅F(x,y)+(1−k)⋅65535

where F(x, y) is the pixel value at position (x, y) in the original sequence and F’(x, y) is the generated fading sequence pixel at (x, y). The scaling factor k is defined as:

k=e−0.08×POC

To suppress the noise in input video sequences, a low passed filter [1 4 6 4 1]/16 is applied to the original video sequence before generating the fading sequences, as it was suggested in [2].

## Test Results

The test results are illustrated in Figure 1. Compared with HEVC Range Extension with weighted prediction enabled, the proposed high precision weighted prediction shows 1.3% and 2.8% coding gain for RA and LDB respectively, without noticeable complexity differences.

Table I. High Precision Weighted Prediction (proposed) vs. EM RExt 4.1 Anchor (WP enabled)

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 14-bit | -1.8% | -1.2% | -1.6% |
| 16-bit | -1.3% | -1.0% | -1.1% |
| Enc Time[%] | 89% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Low Delay B |  |  |
|  | Y | U | V |
| 14-bit | -3.6% | -2.7% | -3.4% |
| 16-bit | -2.7% | -2.0% | -2.3% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 92% |  |  |

# Conclusion

In this contribution, a method to increase weighted prediction parameters precision is proposed. Compared with current HEVC Range Extension with weighted prediction enabled, average 1.3% (RA) and 2.8% (LDB) BD-rate reduction for high bit depth sequences were observed.

# Reference

- D. Flynn, J. Sole, T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4,” JCTVC-N1005_v3, August 2013.

- K. Sharman, N. Saunders, and J. Gamei, “AHG 5 and 18: Internal Precision for High Bit Depths,” JCTVC-N0188, August 2013.

- C. Rosewarne, J. Sole, K. Sharman, and S.-H Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialisation and update methods,” JCTVC-N1122, August 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).