| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0239 |
| --- | --- |

| Title: | RCE2: Results of Test D1 on Rice Parameter Initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Liwei Guo Joel Sole Rajan Joshi | Tel: Email: | 1-858-845-7532 liweig@qti.qualcomm.com |
|  | Karl Sharman Nick Saunders James Gamei |  | karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Qualcomm Incorporated, Sony Europe Ltd. |  |  |

_____________________________

# Abstract

This contribution reports the results of RCE2 test D1, which is the combined test of JCTVC-N0181 and JCTVC-N0189.

# Introduction

In HEVC, the binarization of the syntax element coeff_abs_level_remaining depends on the Rice parameter. The parameter (cRiceParam) is set to 0 at the beginning of each 4×4 sub-block of transform coefficients. In the 14th JCTVC meeting, both JCTVC-N0181 [1] and JCTVC-N0189 [2] presented methods to initialize the Rice parameter based on previously coded coefficients. This contribution presents the combined method of JCTVC-N0181 and JCTVC-N0189 and reports the results.

# Technical Details

In the proposed scheme, the 4×4 sub-blocks are divided into different categories (“sbType”). For each sub-block, the initial Rice parameter is derived based on previously coded sub-blocks in the same category. The categorization is based on whether the block is a transform-skip block (“isTSFlag”). A PPS flag (“useSeparateStatsFlag”) is introduced to enable further separation of the sub-blocks based on whether they belong to the first color component or not (“isLumaFlag”)

sbType = (useSeparateStatsFlag? 2*isLumaFlag:0) + isTSFlag;

Stats statCoeff  are maintained for each sub-block type (sbType) depending on the absolute coefficient value (uiLevel):

if (uiLevel >=3*(1<<(statCoeff /4)))            statCoeff ++;

else if ((2* uiLevel)<(1<<( statCoeff/4)))           statCoeff --;

This variable is updated at most once per 4×4 sub-block using the value of the first coded coeff_abs_level_remaining of the sub-block. The entries of statCoeff are 8-bit and they are reset to 0 at the beginning of the slice (like the CABAC context variables).

The value of statCoeff is used to initialize the Rice parameter at the beginning of each 4×4 sub-block as

cRiceParam = Min(maxRicePara, statCoeff/4).

# Experimental results

The proposed method is implemented on HM_12.0_RExt4.1. AHG8 (lossless/lossy) and AHG18 (lossy) test conditions specified in [3] are used. Decoding time is measured on a homogenous cluster.

The following settings of the proposed scheme are tested in this contribution.

## Setting 1

In this test, the maximum rice parameter (MAX_RICE_PARAM) is set to be 7 for AHG8 test and 16 for AHG18 test. Extra results for AHG8 test with  MAX_RICE_PARAM = 4 can be found in uploaded Excel files. The useSeparateStatsFlag is set to be 1 for YUV sequences and 0 for RGB sequences. Extra results for always (for both YUV and RGB) setting useSeparateStatsFlag = 1 can be found in the uploaded Excel files.

Table 1 Results of Setting 1 for AHG8 lossless coding

| Avg Bit-Rate Saving | AI-Main | RA-Main | LB-Main |
| --- | --- | --- | --- |
| Class F | 1.5% | 0.7% | 0.4% |
| Class B | 0.1% | 0.0% | 0.0% |
| SC RGB 444 | 9.2% | 7.5% | 6.6% |
| Animation RGB 444 | 0.3% | 0.0% | 0.0% |
| SC YUV 444 | 5.6% | 4.5% | 4.1% |
| Animation YUV 444 | 0.3% | 0.1% | 0.1% |
| RangeExt | 3.6% | 2.7% | 2.7% |
| SC GBR 444 Optional | 21.2% | 20.7% | 21.1% |
| SC YUV 444 Optional | 15.7% | 15.2% | 14.5% |
| Enc Time[%] | 103% | 103% | 102% |
| Dec Time[%] | 98% | 101% | 99% |

Table 2 Results of Setting 1 for AHG8 lossy coding

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.5% | -0.6% | -0.6% | -0.9% | -0.9% | -0.9% | -1.3% | -1.3% | -1.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.5% | -1.6% | -1.6% | -2.9% | -2.8% | -2.8% | -4.4% | -4.6% | -4.6% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.6% | -0.7% | -0.7% | -1.3% | -1.2% | -1.3% | -2.2% | -2.2% | -2.2% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -6.0% | -5.8% | -5.7% | -9.5% | -9.2% | -9.6% | -13.0% | -13.6% | -13.7% |
| SC(444) YUV Optional | -1.7% | -1.9% | -1.8% | -3.7% | -3.6% | -3.4% | -6.4% | -6.4% | -6.9% |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.4% | -0.4% | -0.6% | -0.6% | -0.6% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.3% | -1.3% | -1.3% | -2.4% | -2.4% | -2.4% |  |  |  |
| Animation RGB 444 | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.6% | -0.7% | -0.7% | -1.0% | -1.0% | -1.1% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -5.7% | -5.5% | -5.3% | -9.0% | -8.9% | -9.2% |  |  |  |
| SC(444) YUV Optional | -1.7% | -1.8% | -1.8% | -3.3% | -3.2% | -3.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.1% | -0.3% | -0.3% | -0.5% | -0.5% |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.7% | -0.8% | -0.7% | -1.5% | -1.5% | -1.5% |  |  |  |
| Animation RGB 444 | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC YUV 444 | -0.7% | -0.8% | -0.9% | -1.0% | -0.9% | -1.1% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.2% |  |  |  |
| SC(444) GBR Optional | -5.6% | -5.8% | -5.4% | -9.1% | -9.4% | -9.6% |  |  |  |
| SC(444) YUV Optional | -1.8% | -1.7% | -1.7% | -2.6% | -2.6% | -2.8% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 3 Results of Setting 1 for AHG18 lossy coding - 6 bit HM forward transform

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G | B | R |
| 12-bit | -7.1% | -6.8% | -6.8% |
| 14-bit | -18.7% | -18.4% | -18.3% |
| 16-bit | -28.6% | -28.5% | -28.5% |
| Enc Time[%] | 86% |  |  |
| Dec Time[%] | 84% |  |  |

Table 4 Results of Setting 1 for AHG18 lossy coding – high precision forward transform

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G | B | R |
| 12-bit | -6.9% | -7.1% | -7.1% |
| 14-bit | -18.2% | -18.1% | -18.1% |
| 16-bit | -27.9% | -27.7% | -27.7% |
| Enc Time[%] | 85% |  |  |
| Dec Time[%] | 84% |  |  |

## Setting 2

In this test, MAX_RICE_PARAM is set to equal to MAX_TR_DYNAMIC_RANGE. The Rice parameter update will never be triggered once uiGoRiceParam reaches MAX_TR_DYNAMIC_RANGE, and thus the clipping in the update process can be removed,

i.e., the update process

uiGoRiceParam = min<UInt>(uiGoRiceParam+1, MAX_RICE_PARAM);

can be simplified as

uiGoRiceParam++;

Table 5 Results of Setting 2 for AHG8 lossless coding

| Avg Bit-Rate Saving | AI-Main | RA-Main | LB-Main |
| --- | --- | --- | --- |
| Class F | 1.5% | 0.7% | 0.4% |
| Class B | 0.1% | 0.0% | 0.0% |
| SC RGB 444 | 9.2% | 7.5% | 6.6% |
| Animation RGB 444 | 0.3% | 0.0% | 0.0% |
| SC YUV 444 | 5.6% | 4.5% | 4.1% |
| Animation YUV 444 | 0.3% | 0.1% | 0.1% |
| RangeExt | 3.6% | 2.7% | 2.7% |
| SC GBR 444 Optional | 21.2% | 20.7% | 21.1% |
| SC YUV 444 Optional | 15.7% | 15.2% | 14.5% |
| Enc Time[%] | 100% | 99% | 96% |
| Dec Time[%] | 98% | 99% | 98% |

Table 6 Results of Setting 2 for AHG8 lossy coding

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.5% | -0.6% | -0.6% | -0.9% | -0.9% | -0.9% | -1.3% | -1.3% | -1.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.5% | -1.6% | -1.6% | -2.9% | -2.8% | -2.8% | -4.4% | -4.6% | -4.6% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.6% | -0.7% | -0.7% | -1.3% | -1.2% | -1.3% | -2.2% | -2.2% | -2.2% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -6.0% | -5.8% | -5.7% | -9.5% | -9.2% | -9.6% | -12.9% | -13.6% | -13.7% |
| SC(444) YUV Optional | -1.7% | -1.9% | -1.8% | -3.7% | -3.6% | -3.4% | -6.4% | -6.4% | -6.9% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.4% | -0.4% | -0.6% | -0.6% | -0.6% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.3% | -1.3% | -1.3% | -2.4% | -2.4% | -2.4% |  |  |  |
| Animation RGB 444 | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.6% | -0.7% | -0.7% | -1.0% | -1.0% | -1.1% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -5.7% | -5.5% | -5.3% | -9.0% | -8.9% | -9.2% |  |  |  |
| SC(444) YUV Optional | -1.7% | -1.8% | -1.8% | -3.3% | -3.2% | -3.2% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.1% | -0.1% | -0.3% | -0.3% | -0.5% | -0.5% |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.7% | -0.8% | -0.7% | -1.5% | -1.5% | -1.5% |  |  |  |
| Animation RGB 444 | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC YUV 444 | -0.7% | -0.8% | -0.9% | -1.0% | -0.9% | -1.1% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.0% |  |  |  |
| SC(444) GBR Optional | -5.6% | -5.8% | -5.4% | -9.1% | -9.4% | -9.6% |  |  |  |
| SC(444) YUV Optional | -1.8% | -1.7% | -1.7% | -2.6% | -2.6% | -2.8% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |

Table 7 Results of Setting 2 for AHG18 lossy coding - 6 bit HM forward transform

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -7.1% | -6.8% | -6.8% |
| 14-bit | -18.7% | -18.4% | -18.3% |
| 16-bit | -28.6% | -28.5% | -28.5% |
| Enc Time[%] | 88% |  |  |
| Dec Time[%] | 92% |  |  |

Table 8 Results of Setting 2 for AHG18 lossy coding - high precision forward transform

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -6.9% | -7.1% | -7.1% |
| 14-bit | -18.2% | -18.1% | -18.1% |
| 16-bit | -27.9% | -27.7% | -27.7% |
| Enc Time[%] | 92% |  |  |
| Dec Time[%] | 90% |  |  |

## Setting 3

In the previous two settings, the initial value of the Rice parameter is set to 0 for RDOQ. In this setting, a value derived using previously coded coefficients is used as the initial Rice parameter in RDOQ for a TU. The setting of MAX_RICE_PARAM is the same as Setting 2 (MAX_RICE_PARAM = MAX_TR_DYNAMIC_RANGE).

Table 9 Results of Setting 3 for AHG8 lossless coding

| Avg Bit-Rate Saving | AI-Main | RA-Main | LB-Main |
| --- | --- | --- | --- |
| Class F | 1.5% | 0.7% | 0.4% |
| Class B | 0.1% | 0.0% | 0.0% |
| SC RGB 444 | 9.2% | 7.5% | 6.6% |
| Animation RGB 444 | 0.3% | 0.0% | 0.0% |
| SC YUV 444 | 5.6% | 4.5% | 4.1% |
| Animation YUV 444 | 0.3% | 0.1% | 0.1% |
| RangeExt | 3.6% | 2.7% | 2.7% |
| SC GBR 444 Optional | 21.2% | 20.7% | 21.1% |
| SC YUV 444 Optional | 15.7% | 15.2% | 14.5% |
| Enc Time[%] | 100% | 100% | 100% |
| Dec Time[%] | 98% | 98% | 99% |

Table 10 Results of Setting 2 for AHG8 lossy coding

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.5% | -0.6% | -0.6% | -0.9% | -1.0% | -1.0% | -1.3% | -1.4% | -1.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.6% | -1.6% | -1.6% | -3.0% | -2.8% | -2.9% | -4.4% | -4.6% | -4.6% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.6% | -0.6% | -0.7% | -1.3% | -1.2% | -1.3% | -2.2% | -2.3% | -2.2% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -6.1% | -5.8% | -5.7% | -9.5% | -9.2% | -9.6% | -13.0% | -13.6% | -13.7% |
| SC(444) YUV Optional | -1.8% | -1.9% | -1.8% | -3.8% | -3.6% | -3.5% | -6.5% | -6.4% | -7.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.4% | -0.4% | -0.6% | -0.6% | -0.6% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -1.4% | -1.4% | -1.4% | -2.5% | -2.4% | -2.4% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.6% | -0.6% | -0.7% | -1.0% | -1.0% | -1.1% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | -5.8% | -5.6% | -5.4% | -9.2% | -9.0% | -9.3% |  |  |  |
| SC(444) YUV Optional | -1.9% | -1.9% | -1.8% | -3.6% | -3.5% | -3.3% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.2% | -0.3% | -0.4% | -0.3% | -0.3% | -0.5% |  |  |  |
| Class B | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% | 0.1% |  |  |  |
| SC RGB 444 | -1.0% | -1.1% | -1.1% | -2.0% | -2.0% | -2.0% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.2% | -0.2% | -0.1% | -0.5% | -0.4% | -0.4% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -5.3% | -5.3% | -5.2% | -9.2% | -9.3% | -9.7% |  |  |  |
| SC(444) YUV Optional | -2.1% | -2.1% | -2.0% | -3.3% | -3.2% | -3.3% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |

Table 11 Results of Setting 3 for AHG18 lossy coding - 6 bit HM forward transform

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -7.2% | -6.8% | -6.9% |
| 14-bit | -18.7% | -18.4% | -18.3% |
| 16-bit | -28.6% | -28.5% | -28.5% |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 88% |  |  |

Table 12 Results of Setting 3 for AHG18 lossy coding - high precision forward transform

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -7.0% | -7.1% | -7.1% |
| 14-bit | -18.2% | -18.1% | -18.1% |
| 16-bit | -27.9% | -27.7% | -27.7% |
| Enc Time[%] | 91% |  |  |
| Dec Time[%] | 88% |  |  |

# Conclusions

This contribution presents the combination test D1 of RCE2, which is the combination of JCTVC-N0181 and JCTVC-N0189. The tested method collects the statistics of already coded coefficients (at most once per sub-block) and calculates a value for initializing Rice parameter for coding the next sub-block. Coding efficiency improvements have been observed in all the test cases.

# Draft texts

Please find draft texts in the attached documents.

# References

- M. Karczewicz, J. Sole, and R. Joshi, “Non-RCE2: Rice Parameter Initialization”, JCTVC-N0181, Vienna, Austria, Jul. 2013.

- K. Sharman, N. Saunders, and J. Gamei, “AHG 5 and 18: Entropy Coding Compression Efficiency for High Bit Depths”, JCTVC-N0189, Vienna, Austria, Jul. 2013.

- C. Rosewarne, J. Sole, K. Sharman, and S.-H Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialization and update methods”, JCTVC-N1122, Vienna, Austria, Jul. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).