| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0243 |
| --- | --- |

| Title: | Cross-check report of AHG10/AHG12: On SHM Common Test Conditions (JCTVC-O0097) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ohji Nakagami | Tel: Email: | +81-50-3750-2746 ohji.nakagami@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document is a cross-check report of JCTVC-O0097. It is reported that the x-check result is identical to the result provided by the proponent.

# Cross check

In JCTVC-O0097 [1], it is proposed to disable the macro FINAL_RPL_CHANGE_N0082 by default.

As a background, the macro was introduced for SHM3.0 encoder to reduce RPLM bit in SHM CTC RA condition on top of the modified RPL initialization process, which was proposed in JCTVC-N0082 [2] and JCTVC-N0316 [3].

It is pointed in [1] that the macro introduces the additional encoder control complexity, and increases the number of lines in SHM3.0 encoder source code. On the other hand, it is pointed that gains by the macro is trivial. For the clarification, the following settings are compared.

Default setting in SHM-3.0.1:

FINAL_RPL_CHANGE_N0082   1

Tested setting in SHM-3.0.1:

FINAL_RPL_CHANGE_N0082   0

The same settings are tested. It is reported that the cross check result is identical to [1].

The bitrate difference in RA, which condition is affected by the macro, is shown in Table1. It is noted that the higher precision is used to emphasize the difference is not significant. The complete result is attached in the document.

Table1 RA result with higher precision

|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  | RA HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.006% | -0.009% | 0.006% |  |  |  | -0.008% | -0.003% | -0.014% |
| Class B | -0.003% | 0.003% | 0.019% | 0.007% | 0.008% | -0.009% | -0.001% | -0.032% | -0.045% |
| Overall (Test vs Ref) | -0.004% | 0.000% | 0.015% | 0.007% | 0.008% | -0.009% | -0.003% | -0.024% | -0.036% |
| Overall (Test vs single layer) | 19.002% | 33.101% | 31.836% | 16.197% | 28.869% | 29.149% | 14.405% | 32.113% | 34.138% |
| Overall (Ref vs single layer) | 19.008% | 33.108% | 31.821% | 16.189% | 28.856% | 29.162% | 14.409% | 32.149% | 34.188% |
| EL only (Test vs Ref) | 0.006% | 0.009% | 0.024% | 0.024% | 0.026% | 0.005% | 0.002% | -0.022% | -0.034% |

# Reference

- JCTVC-O0097, “AHG10/AHG12: On SHM Common Test Conditions”

- JCTVC-N0082, “MV-HEVC/SHVC HLS: On initialization process of reference picture lists for HEVC extensions”

- JCTVC-N0316, “MV-HEVC/SHVC HLS: Initial inter-layer reference picture list construction”

- JCTVC-N1009, “Common SHM test conditions and software reference configurations”

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).