| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0244 |
| --- | --- |

| Title: | RExt: Signaling Motion Vector Range for Intra Block Copying |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo, Chao Pang, Jianle Chen, Joel Sole, Rajan Joshi, Marta Karczewicz, Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-845-7532 liweig@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes to add a new VUI syntax element to indicate a restriction on the range of the horizontal component of the motion vector for Intra Block Copying. It is asserted that such signaling can help the codec better allocate memory and optimize pipelining.

# Introduction

In the 14th JCT-VC meeting, Intra Block Copying (IntraBC) [1] was adopted into the HEVC Range Extensions draft specification. In this contribution, it is proposed to signal the range of the displacement between the prediction block and the current block (denoted as motion vector). The prediction blocks consist of reconstructed pixels before loop-filtering. Loop-filtering of these pixels has to be delayed until they are no longer in the IntraBC motion vector range of any future block. It is asserted that signaling IntraBC motion vector range can help the codec better allocate memory and optimize pipelining.

# Proposed method

In this method, it is proposed that a new syntax element be added to VUI parameters: log2_max_mv_length _intrabc_horizontal.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| … |  |
| bitstream_restriction_flag | u(1) |
| if( bitstream_restriction_flag ) { |  |
| … |  |
| log2_max_mv_length_horizontal | ue(v) |
| log2_max_mv_length_vertical | ue(v) |
| log2_max_mv_length_intrabc_horizontal | ue(v) |
| … |  |
| } |  |
| } |  |

log2_max_mv_length_intrabc_horizontal indicates the maximum absolute value of a decoded horizontal motion vector component for any coding unit Cui with intra_bc_flag equal to1, in units of luma samples, for all pictures in the coded video sequence. A value of n asserts that no value of a horizontal motion vector component of CUi is less than −2n − xCU, in units of luma samples, where xCU is the absolute value of the horizontal distance between the left boundary of CUi and the left boundary of the CTB containg CUi, in units of luma samples.

# References

[1] C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs”, JCTVC-N0256, Vienna, AT, August 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).