| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0247 WG11 Doc: m31030 |
| --- | --- |

| Title: | RCE2: Test B1. Rice parameter signaling for transform-skip blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or | Seung-Hwan Kim Misra Kiran Andrew Segall | Email: | kimse@sharplabs.com misrak @sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution presents sub-test B.1 in RCE2, where the Rice parameter (cRiceParam) is explicitly signalled for each coefficient sub-block while leaving the existing cRiceParam update process. The proposed method is applied for two cases (1) transform-skip and transform-bypass blocks and (2) all blocks.

# Proposed solution

In the proposed method, rice parameter (cRiceParam) is signalled for each transform-skip block and the existing rice-parameter update process is removed. cRiceParam is signalled from the given set {0, 2, 4, 6}. The binarization for the cRiceParm is shown.

Table 1 TU binarization for cRiceParam

| Rice-parameter | Bin string |
| --- | --- |
| 0 | 1 |
| 2 | 01 |
| 4 | 001 |
| 6 | 000 |

Table 2 TU binarization for cRiceParam (higher bit-depth, BD>=12)

| Rice-parameter | Bin string |
| --- | --- |
| 0 | 1 |
| 4 | 01 |
| 8 | 001 |
| 12 | 000 |

# Experiment Results

Table 3 and Table  provide results for RCE2 Test B1 (transform-skip and transform-bypass blocks only) under AHG8/RCE2/3 lossy and lossless conditions, respectively (as defined in [3] and [4]). Table 5 and Table 6 provide results for RCE2 Test B1 (transform-skip and transform-bypass blocks only) under AHG18 conditions, with the HM forward transform and the high precision forward transform respectively.

Table 3. RCE2 Test B1 results under RCE2/3 lossy conditions (transform-skip and transform-bypass blocks only).

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.1% | -0.1% | -0.1% | -0.3% | -0.2% | -0.2% | -0.6% | -0.5% | -0.6% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.6% | -0.6% | -0.6% | -1.6% | -1.5% | -1.5% | -2.9% | -3.1% | -3.1% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | -0.4% | -0.3% | -0.3% | -0.9% | -1.0% | -0.9% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -3.5% | -3.2% | -3.2% | -6.2% | -5.9% | -6.2% | -9.5% | -10.1% | -10.1% |
| SC(444) YUV Optional | -0.4% | -0.6% | -0.5% | -1.6% | -1.5% | -1.5% | -4.0% | -3.9% | -4.4% |
| Enc Time[%] | 102% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| SC RGB 444 | -0.5% | -0.5% | -0.6% | -1.3% | -1.2% | -1.3% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -3.2% | -3.1% | -2.9% | -5.8% | -5.6% | -5.7% |  |  |  |
| SC(444) YUV Optional | -0.5% | -0.7% | -0.6% | -1.3% | -1.3% | -1.3% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.1% | -0.5% | -0.2% | 0.1% | -0.2% | -0.1% |  |  |  |
| Class B | 0.0% | -0.1% | -0.3% | 0.0% | -0.1% | -0.1% |  |  |  |
| SC RGB 444 | 0.1% | 0.1% | 0.0% | -0.8% | -0.7% | -0.8% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.1% | -0.2% | -0.2% | -0.4% | -0.3% | -0.4% |  |  |  |
| Animation YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | -2.3% | -2.4% | -2.0% | -5.4% | -5.5% | -5.7% |  |  |  |
| SC(444) YUV Optional | -1.2% | -1.1% | -1.1% | -1.7% | -1.7% | -1.9% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  |  |  |  |

Table 4. RCE2 Test B1 results under RCE2/3 lossless conditions (transform-skip and transform-bypass blocks only).

|  | AI Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 4.57 | 4.63 | 5.57 | 5.64 | 2.27 | 2.27 | 11.14 | 11.24 | 1.2% | 1.3% | 0.3% | 2.5% |
| Class B | 2.24 | 2.24 | 2.26 | 2.26 | 2.08 | 2.08 | 2.44 | 2.44 | -0.1% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | 7.88 | 8.34 | 9.5 | 10.44 | 5.23 | 5.43 | 14.73 | 17.30 | 5.5% | 7.3% | 1.3% | 14.8% |
| Animation RGB 444 | 2.48 | 2.48 | 2.5 | 2.52 | 2.15 | 2.15 | 3.05 | 3.05 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 11.09 | 11.44 | 13.2 | 13.91 | 7.87 | 7.98 | 19.32 | 21.57 | 3.0% | 4.1% | 0.5% | 10.4% |
| Animation YUV 444 | 3.00 | 3.00 | 3.2 | 3.16 | 2.57 | 2.57 | 3.93 | 3.93 | 0.1% | 0.0% | -0.1% | 0.3% |
| RangeExt | 1.92 | 1.97 | 2.38 | 2.42 | 1.46 | 1.52 | 4.37 | 4.41 | 2.4% | 2.1% | 0.5% | 4.0% |
| SC GBR 444 Optional | 20.30 | 23.29 | 24.82 | 29.53 | 9.69 | 10.64 | 36.10 | 42.93 | 12.9% | 14.3% | 9.0% | 18.1% |
| SC YUV 444 Optional | 31.85 | 36.41 | 34.39 | 39.84 | 22.30 | 24.57 | 51.97 | 61.43 | 12.5% | 12.8% | 9.2% | 15.4% |
| Enc Time[%] |  |  | 103% |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  | 101% |  |  |  |  |  |  |  |  |  |

|  | RA Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.66 | 8.67 | 36.22 | 36.67 | 3.01 | 3.01 | 90.48 | 91.95 | 0.1% | 0.6% | -0.1% | 1.6% |
| Class B | 2.60 | 2.60 | 2.60 | 2.60 | 2.57 | 2.57 | 2.64 | 2.64 | -0.1% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | 53.71 | 55.05 | 117.01 | 128.49 | 19.48 | 19.71 | 225.68 | 256.76 | 2.4% | 5.8% | 0.8% | 14.8% |
| Animation RGB 444 | 3.58 | 3.58 | 3.60 | 3.59 | 3.43 | 3.43 | 3.70 | 3.70 | -0.1% | -0.1% | -0.1% | -0.1% |
| SC YUV 444 | 73.79 | 74.65 | 157.08 | 165.69 | 24.42 | 24.52 | 287.13 | 320.46 | 1.2% | 3.3% | 0.3% | 10.4% |
| Animation YUV 444 | 3.8 | 3.84 | 4.48 | 4.48 | 2.75 | 2.75 | 5.70 | 5.69 | 0.0% | -0.1% | -0.1% | 0.0% |
| RangeExt | 2.1 | 2.1 | 2.5 | 2.5 | 1.5 | 1.58 | 4.41 | 4.44 | 1.5% | 1.3% | 0.0% | 3.1% |
| SC GBR 444 Optional | 42.0 | 45.8 | 254.1 | 305.2 | 11.5 | 12.39 | 395.52 | 481.75 | 8.3% | 13.7% | 7.6% | 17.9% |
| SC YUV 444 Optional | 101.1 | 110.6 | 314.9 | 369.7 | 30.5 | 32.88 | 531.53 | 633.65 | 8.6% | 12.3% | 7.4% | 16.1% |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |

|  | LB Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.9 | 8.9 | 57.8 | 58.1 | 3.0 | 3.0 | 169.5 | 170.5 | 0.0% | 0.2% | -0.1% | 0.6% |
| Class B | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | -0.1% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | 62.2 | 63.2 | 355.6 | 404.0 | 20.5 | 20.7 | 1618.0 | 1894.5 | 1.6% | 5.2% | 0.6% | 14.6% |
| Animation RGB 444 | 3.6 | 3.6 | 3.6 | 3.6 | 3.4 | 3.4 | 3.7 | 3.7 | -0.1% | -0.1% | -0.1% | -0.1% |
| SC YUV 444 | 85.1 | 85.7 | 469.0 | 508.5 | 25.7 | 25.7 | 2105.0 | 2348.5 | 0.6% | 2.8% | 0.3% | 10.4% |
| Animation YUV 444 | 4 | 3.9 | 4.5 | 4.5 | 2.8 | 2.8 | 5.8 | 5.8 | -0.1% | -0.1% | -0.1% | 0.0% |
| RangeExt | 2.1 | 2.1 | 2.5 | 2.5 | 1.5 | 1.6 | 4.4 | 4.4 | 1.5% | 1.3% | -0.1% | 3.1% |
| SC GBR 444 Optional | 45 | 48.8 | 1010.8 | 1227.0 | 11.6 | 12.5 | 2253.7 | 2774.3 | 7.6% | 13.5% | 7.4% | 18.8% |
| SC YUV 444 Optional | 117.1 | 126.8 | 1139.4 | 1311.6 | 31.0 | 33.5 | 2208.6 | 2474.0 | 7.7% | 11.8% | 7.2% | 17.4% |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |

Table 5. RCE2 Test B1 simulation results under AHG18 conditions with the HM forward transform (transform-skip and transform-bypass blocks only).

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -4.9% | -4.4% | -4.4% |
| 14-bit | -16.9% | -16.5% | -16.4% |
| 16-bit | -27.8% | -27.6% | -27.6% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

Table 6. RCE2 Test B1 simulation results under AHG18 conditions with the high precision forward transform (transform-skip and transform-bypass blocks only).

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -4.0% | -3.8% | -3.8% |
| 14-bit | -15.1% | -14.6% | -14.6% |
| 16-bit | -25.8% | -25.2% | -25.2% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Table 7 and Table 8 provide results for RCE2 Test B1 (All blocks) under AHG8/RCE2/3 lossy and lossless conditions, respectively (as defined in [3] and [4]). Table 9 and Table 10 provide results for RCE2 Test B1 (All blocks) under AHG18 conditions, with the HM forward transform and the high precision forward transform respectively.

Table 7. RCE2 Test B1 results under RCE2/3 lossy conditions. (All blocks)

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | -0.2% | -0.4% | -0.4% | -0.4% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| SC RGB 444 | -0.5% | -0.5% | -0.5% | -1.5% | -1.4% | -1.4% | -2.8% | -3.0% | -3.0% |
| Animation RGB 444 | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| SC YUV 444 | 0.1% | 0.1% | 0.0% | -0.3% | -0.2% | -0.2% | -0.8% | -0.8% | -0.8% |
| Animation YUV 444 | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| RangeExt | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| SC(444) GBR Optional | -3.5% | -3.3% | -3.2% | -6.2% | -5.9% | -6.2% | -9.5% | -10.2% | -10.2% |
| SC(444) YUV Optional | -0.3% | -0.5% | -0.5% | -1.6% | -1.4% | -1.4% | -3.9% | -3.8% | -4.3% |
| Enc Time[%] | 102% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.1% | 0.2% | 0.2% | 0.0% | 0.1% | 0.1% |  |  |  |
| Class B | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC RGB 444 | -0.4% | -0.5% | -0.5% | -1.2% | -1.2% | -1.2% |  |  |  |
| Animation RGB 444 | 0.2% | 0.2% | 0.2% | 0.1% | 0.1% | 0.2% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Animation YUV 444 | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% |  |  |  |
| RangeExt | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% |  |  |  |
| SC(444) GBR Optional | -3.3% | -3.1% | -3.0% | -6.0% | -5.8% | -5.9% |  |  |  |
| SC(444) YUV Optional | -0.3% | -0.4% | -0.4% | -1.4% | -1.4% | -1.3% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.3% | -0.6% | -0.2% | 0.2% | -0.1% | 0.0% |  |  |  |
| Class B | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% |  |  |  |
| SC RGB 444 | -0.1% | -0.2% | -0.2% | -0.8% | -0.8% | -0.8% |  |  |  |
| Animation RGB 444 | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% | 0.2% |  |  |  |
| SC YUV 444 | 0.1% | 0.0% | 0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Animation YUV 444 | 0.2% | 0.1% | 0.3% | 0.2% | 0.1% | 0.2% |  |  |  |
| RangeExt | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC(444) GBR Optional | -2.4% | -2.5% | -2.2% | -6.2% | -6.4% | -6.7% |  |  |  |
| SC(444) YUV Optional | -1.5% | -1.5% | -1.4% | -2.1% | -2.1% | -2.3% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  |  |  |  |

Table 8. RCE2 Test B1 results under RCE2/3 lossless conditions. (All blocks)

|  | AI Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 4.57 | 4.63 | 5.57 | 5.64 | 2.27 | 2.27 | 11.14 | 11.24 | 1.2% | 1.3% | 0.3% | 2.5% |
| Class B | 2.24 | 2.24 | 2.26 | 2.26 | 2.08 | 2.08 | 2.44 | 2.44 | -0.1% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | 7.88 | 8.34 | 9.5 | 10.44 | 5.23 | 5.43 | 14.73 | 17.30 | 5.5% | 7.3% | 1.3% | 14.8% |
| Animation RGB 444 | 2.48 | 2.48 | 2.5 | 2.52 | 2.15 | 2.15 | 3.05 | 3.05 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 11.09 | 11.44 | 13.2 | 13.91 | 7.87 | 7.98 | 19.32 | 21.57 | 3.0% | 4.1% | 0.5% | 10.4% |
| Animation YUV 444 | 3.00 | 3.00 | 3.2 | 3.16 | 2.57 | 2.57 | 3.93 | 3.93 | 0.1% | 0.0% | -0.1% | 0.3% |
| RangeExt | 1.92 | 1.97 | 2.38 | 2.42 | 1.46 | 1.52 | 4.37 | 4.41 | 2.4% | 2.1% | 0.5% | 4.0% |
| SC GBR 444 Optional | 20.30 | 23.29 | 24.82 | 29.53 | 9.69 | 10.64 | 36.10 | 42.93 | 12.9% | 14.3% | 9.0% | 18.1% |
| SC YUV 444 Optional | 31.85 | 36.41 | 34.39 | 39.84 | 22.30 | 24.57 | 51.97 | 61.43 | 12.5% | 12.8% | 9.2% | 15.4% |
| Enc Time[%] |  |  | 103% |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  | 101% |  |  |  |  |  |  |  |  |  |

|  | RA Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.66 | 8.67 | 36.22 | 36.67 | 3.01 | 3.01 | 90.48 | 91.95 | 0.1% | 0.6% | -0.1% | 1.6% |
| Class B | 2.60 | 2.60 | 2.60 | 2.60 | 2.57 | 2.57 | 2.64 | 2.64 | -0.1% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | 53.71 | 55.05 | 117.01 | 128.49 | 19.48 | 19.71 | 225.68 | 256.76 | 2.4% | 5.8% | 0.8% | 14.8% |
| Animation RGB 444 | 3.58 | 3.58 | 3.60 | 3.59 | 3.43 | 3.43 | 3.70 | 3.70 | -0.1% | -0.1% | -0.1% | -0.1% |
| SC YUV 444 | 73.79 | 74.65 | 157.08 | 165.69 | 24.42 | 24.52 | 287.13 | 320.46 | 1.2% | 3.3% | 0.3% | 10.4% |
| Animation YUV 444 | 3.8 | 3.84 | 4.48 | 4.48 | 2.75 | 2.75 | 5.70 | 5.69 | 0.0% | -0.1% | -0.1% | 0.0% |
| RangeExt | 2.1 | 2.1 | 2.5 | 2.5 | 1.5 | 1.58 | 4.41 | 4.44 | 1.5% | 1.3% | 0.0% | 3.1% |
| SC GBR 444 Optional | 42.0 | 45.8 | 254.1 | 305.2 | 11.5 | 12.39 | 395.52 | 481.75 | 8.3% | 13.7% | 7.6% | 17.9% |
| SC YUV 444 Optional | 101.1 | 110.6 | 314.9 | 369.7 | 30.5 | 32.88 | 531.53 | 633.65 | 8.6% | 12.3% | 7.4% | 16.1% |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |

|  | LB Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.9 | 8.9 | 57.8 | 58.1 | 3.0 | 3.0 | 169.5 | 170.5 | 0.0% | 0.2% | -0.1% | 0.6% |
| Class B | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | -0.1% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | 62.2 | 63.2 | 355.6 | 404.0 | 20.5 | 20.7 | 1618.0 | 1894.5 | 1.6% | 5.2% | 0.6% | 14.6% |
| Animation RGB 444 | 3.6 | 3.6 | 3.6 | 3.6 | 3.4 | 3.4 | 3.7 | 3.7 | -0.1% | -0.1% | -0.1% | -0.1% |
| SC YUV 444 | 85.1 | 85.7 | 469.0 | 508.5 | 25.7 | 25.7 | 2105.0 | 2348.5 | 0.6% | 2.8% | 0.3% | 10.4% |
| Animation YUV 444 | 4 | 3.9 | 4.5 | 4.5 | 2.8 | 2.8 | 5.8 | 5.8 | -0.1% | -0.1% | -0.1% | 0.0% |
| RangeExt | 2.1 | 2.1 | 2.5 | 2.5 | 1.5 | 1.6 | 4.4 | 4.4 | 1.5% | 1.3% | -0.1% | 3.1% |
| SC GBR 444 Optional | 45 | 48.8 | 1010.8 | 1227.0 | 11.6 | 12.5 | 2253.7 | 2774.3 | 7.6% | 13.5% | 7.4% | 18.8% |
| SC YUV 444 Optional | 117.1 | 126.8 | 1139.4 | 1311.6 | 31.0 | 33.5 | 2208.6 | 2474.0 | 7.7% | 11.8% | 7.2% | 17.4% |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |

Table 9. RCE2 Test B1 simulation results under AHG18 conditions with the HM forward transform. (All blocks)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -6.4% | -6.0% | -6.0% |
| 14-bit | -17.8% | -17.4% | -17.4% |
| 16-bit | -27.9% | -27.7% | -27.8% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

Table 10. RCE2 Test B1 simulation results under AHG18 conditions with the high precision forward transform. (All blocks)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -6.1% | -6.3% | -6.3% |
| 14-bit | -17.3% | -17.3% | -17.3% |
| 16-bit | -27.3% | -27.0% | -27.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

# Conclusion

This contribution presents an approach to the Rice parameter adaptation whereby the rice parameter initial value is signaled for transform skip and transquant bypass blocks, while leaving the update mechanism unchanged.

# References

- S.H. Kim, A. Segall, “Non-RCE1/Non-RCE2: Simplified level coding for Transform skip and lossless coding”, JCTVC-M0304, Incheon, KR, April 2013.

- S.H. Kim, A. Segall, “RCE2 Test A.1: Rice parameter signaling for transform-skip blocks”, JCTVC-N0264, Vienna, Austria, July 2013.

- Chris Rosewarne, Joel Sole, Karl Sharman, S.-H. Kim “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialisation and update methods”, Joint Collaborative Team on Video Coding, JCTVC-N1122, Vienna, AT, 25 July – 2 Aug. 2013.

- A. Saxena, D. Kwon, M. Naccari, C. Pang, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques,” JCTVC-N1123, 14th JCT-VC Meeting, Vienna, AT, July 2013.

# Text Changes

# - Changes to residual coding syntax and semantics

#### 7.3.8.11		Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize = = 2 ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| last_sig_coeff_x_prefix | ae(v) |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| lastScanPos = 16 |  |
| lastSubBlock = ( 1 << ( log2TrafoSize − 2 ) ) * ( 1 << ( log2TrafoSize − 2 ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = 16 |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ lastSubBlock ][ 0 ] |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ lastSubBlock ][ 1 ] |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 0 ] |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| for( n = ( i = = lastSubBlock ) ? lastScanPos − 1 : 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| } |  |
| firstSigScanPos = 16 |  |
| lastSigScanPos = −1 |  |
| numGreater1Flag = 0 |  |
| lastGreater1ScanPos = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| if( numGreater1Flag < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numGreater1Flag++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && lastGreater1ScanPos = = −1 ) |  |
| lastGreater1ScanPos = n |  |
| } |  |
| if( lastSigScanPos = = −1 ) |  |
| lastSigScanPos = n |  |
| firstSigScanPos = n |  |
| } |  |
| } |  |
| } |  |
| signHidden = ( lastSigScanPos − firstSigScanPos > 3 && !cu_transquant_bypass_flag ) |  |
| if( lastGreater1ScanPos != −1 ) |  |
| coeff_abs_level_greater2_flag[ lastGreater1ScanPos ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] && ( !sign_data_hiding_enabled_flag ¦ ¦ !signHidden ¦ ¦ ( n != firstSigScanPos ) ) ) |  |
| coeff_sign_flag[ n ] |  |
| } | ae(v) |
| numSigCoeff = 0 |  |
| sumAbsLevel = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| if(transform_skip_flag[ x0 ][ y0 ][ cIdx ] ¦¦ cu_transquant_bypass_flag ) { |  |
| cRiceParam |  |
| } |  |
| else { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = lastGreater1ScanPos) ? 3 : 2 ) : 1 ) ) |  |
| } |  |
| coeff_abs_level_remaining[ n ] |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_enabled_flag && signHidden ) { | ae(v) |
| sumAbsLevel += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( ( n = = firstSigScanPos ) && ( ( sumAbsLevel % 2 ) = = 1 ) ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| } |  |
| numSigCoeff++ |  |
| } |  |
| } |  |
| } |  |
| } |  |

#### 9.3.3.9 Binarization process for RiceParam

Input to this process is a request for a binarization for the syntax element RiceParam.

Output of this process is the binarization of the syntax element.

The binarization for the syntax element RiceParam is specified in Table 9-X.

Table 9-X TU binarization for cRiceParam (for Method 1)

| RiceParam | Bin string |
| --- | --- |
| 0 | 1 |
| 1 | 01 |
| 2 | 001 |
| 3 | 0001 |
| 4 | 0000 |

Table 9-X TU binarization for cRiceParam (for Method 2)

| RiceParam | Bin string |
| --- | --- |
| 0 | 1 |
| 2 | 01 |
| 4 | 00 |

#### 9.3.3.10 Binarization process for coeff_abs_level_remaining

Input to this process is a request for a binarization for the syntax element coeff_abs_level_remaining[ n ], the current sub-block scan index i, and baseLevel.

Output of this process is the binarization of the syntax element.

- If this process is invoked for the first time for the current sub-block scan index i, cLastRiceParam are set from the decoded information when the following condition is satisfied

if((transform_skip_flag[ x0 ][ y0 ][ cIdx ] || cu_transquant_bypass_flag) )

where transform_skip_flag[ x0 ][ y0 ][ cIdx ] indicates the transform skip flag for the current sub-block.

- Otherwise the variables cLastAbsLevel and cLastRiceParam are derived as follows:

- If this process is invoked for the first time for the current sub-block scan index i, cLastAbsLevel and cLastRiceParam are set equal to 0.

- Otherwise (this process is not invoked for the first time for the current sub-block scan index i), cLastAbsLevel and cLastRiceParam are set equal to the values of cAbsLevel and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element coeff_abs_level_remaining as specified in this subclause.

The variable cAbsLevel is set equal to baseLevel + coeff_abs_level_remaining[ n ].

The variable cRiceParam is derived from cLastAbsLevel and cLastRiceParam as:

cRiceParam = Min( cLastRiceParam + ( cLastAbsLevel > ( 3 * ( 1  <<  cLastRiceParam ) ) ? 1 : 0 ), 4 )	(9-1)

# - Changes required to the initialization process for context variables

The following changes to Table 9-1 are required for the introduction of the inter_rdpcm_flag and inter_rdpcm_dir_flag binary flags. Moreover, one initialization table (Table 9X) is added.

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| sao( ) | sao_merge_left_flag sao_merge_up_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | sao_type_idx_luma sao_type_idx_chroma | Error: Reference source not found | 0 | 1 | 2 |
| coding_quadtree( ) | split_cu_flag[ ][ ] | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 | 1..4 | 5..8 |
|  | prev_intra_luma_pred_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | rqt_root_cbf | Error: Reference source not found |  | 0 | 1 |
| prediction_unit( ) | merge_flag[ ][ ] | Error: Reference source not found |  | 0 | 1 |
|  | merge_idx[ ][ ] | Error: Reference source not found |  | 0 | 1 |
|  | inter_pred_idc[ ][ ] | Error: Reference source not found |  | 0..4 | 5..9 |
|  | ref_idx_l0[ ][ ], ref_idx_l1[ ][ ] | Error: Reference source not found |  | 0..1 | 2..3 |
|  | mvp_l0_flag[ ][ ], mvp_l1_flag[ ][ ] | Error: Reference source not found |  | 0 | 1 |
| transform_tree( ) | split_transform_flag[ ][ ][ ] | Error: Reference source not found | 0..2 | 3..5 | 6..8 |
|  | cbf_luma[ ][ ][ ] | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
|  | cbf_cb[ ][ ][ ], cbf_cr[ ][ ][ ] | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | Error: Reference source not found |  | 0 | 2 |
|  | abs_mvd_greater1_flag[ ] | Error: Reference source not found |  | 1 | 3 |
| transform_unit( ) | cu_qp_delta_abs | Error: Reference source not found | 0..1 | 2..3 | 4..5 |
| residual_coding( ) | transform_skip_flag[ ][ ][ 0 ] | Table 9- | 0 | 1 | 2 |
|  | transform_skip_flag[ ][ ][ 1 ] transform_skip_flag[ ][ ][ 2 ] | Table 9- | 3 | 4 | 5 |
|  | cRiceParam_prefix | Table 9X | 0 | 1 | 2 |
|  | last_sig_coeff_x_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..53 |
|  | last_sig_coeff_y_prefix | Error: Reference source not found | 0..17 | 18..35 | 36..53 |
|  | coded_sub_block_flag[ ][ ] | Error: Reference source not found | 0..3 | 4..7 | 8..11 |
|  | sig_coeff_flag[ ][ ] | Error: Reference source not found | 0..41 | 42..83 | 84..125 |
|  | coeff_abs_level_greater1_flag[ ] | Error: Reference source not found | 0..23 | 24..47 | 48..71 |
|  | coeff_abs_level_greater2_flag[ ] | Error: Reference source not found | 0..5 | 6..11 | 12..17 |

Table 9-X – Values of initValue for ctxIdx of cRiceParam_prefix

| Initialization variable | ctxIdx of cRiceParam_prefix |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 153 | 153 | 153 |

# Changes required to the binarization process

Table 9-2 is changed to define the binarization process for the inter_rdpcm_flag and inter_rdpcm_dir_flag flags.

| Table 9-2 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| slice_segment_data( ) | end_of_slice_segment_flag | FL | cMax = 1 |
|  | end_of_sub_stream_one_bit | FL | cMax = 1 |
| sao( ) | sao_merge_left_flag | FL | cMax = 1 |
|  | sao_merge_up_flag | FL | cMax = 1 |
|  | sao_type_idx_luma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_type_idx_chroma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_offset_abs[ ][ ][ ][ ] | TR | cMax = ( 1 << ( Min( bitDepth, 10 ) − 5 ) ) − 1, cRiceParam = 0 |
|  | sao_offset_sign[ ][ ][ ][ ] | FL | cMax = 1 |
|  | sao_band_position[ ][ ][ ] | FL | cMax = 31 |
|  | sao_eo_class_luma | FL | cMax = 3 |
|  | sao_eo_class_chroma | FL | cMax = 3 |
| coding_quadtree( ) | split_cu_flag[ ][ ] | FL | cMax = 1 |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | part_mode | Error: Reference source not found | ( xCb, yCb ) = ( x0, y0), log2CbSize |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | prev_intra_luma_pred_flag[ ][ ] | FL | cMax = 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |
|  | intra_chroma_pred_mode[ ][ ] |  | - |
|  | rqt_root_cbf | FL | cMax = 1 |
| prediction_unit( ) | merge_flag[ ][ ] | FL | cMax = 1 |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |
|  | inter_pred_idc[ x0 ][ y0 ] | Error: Reference source not found | nPbW, nPbH |
|  | ref_idx_l0[ ][ ] | TR | cMax = num_ref_idx_l0_active_minus1, cRiceParam = 0 |
|  | mvp_l0_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l1[ ][ ] | TR | cMax = num_ref_idx_l1_active_minus1, cRiceParam = 0 |
|  | mvp_l1_flag[ ][ ] | FL | cMax = 1 |
| transform_tree( ) | split_transform_flag[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_luma[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_cb[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_cr[ ][ ][ ] | FL | cMax = 1 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |
| transform_unit( ) | cu_qp_delta_abs | Error: Reference source not found | - |
|  | cu_qp_delta_sign_flag | FL | cMax = 1 |
| residual_coding( ) | transform_skip_flag[ ][ ][ ] | FL | cMax = 1 |
|  | c_riceParam_prefix | FL | cMax = 1 |
|  | c_riceParam_suffix | FL | cMax = 1 |
|  | last_sig_coeff_x_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_y_prefix | TR | cMax = ( log2TrafoSize << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_x_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_x_prefix >> 1 ) − 1 ) − 1 ) |
|  | last_sig_coeff_y_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_y_prefix >> 1 ) − 1 ) − 1 ) |
|  | coded_sub_block_flag[ ][ ] | FL | cMax = 1 |
|  | sig_coeff_flag[ ][ ] | FL | cMax = 1 |
|  | coeff_abs_level_greater1_flag[ ] | FL | cMax = 1 |
|  | coeff_abs_level_greater2_flag[ ] | FL | cMax = 1 |
|  | coeff_abs_level_remaining[ ] |  | current sub-block scan index i, baseLevel |
|  | coeff_sign_flag[ ] | FL | cMax = 1 |

# Changes required to the derivation of context index variables and bypass

The following rows are added to Table 9-3 for the inter_rdpcm_flag and inter_rdpcm_dir_flag flags.

| Table 9-3 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_slice_segment_flag | terminate | na | na | na | na | na |
| end_of_sub_stream_one_bit | terminate | na | na | na | na | na |
| sao_merge_left_flag | 0 | na | na | na | na | na |
| sao_merge_up_flag | 0 | na | na | na | na | na |
| sao_type_idx_luma | 0 | bypass | na | na | na | na |
| sao_type_idx_chroma | 0 | bypass | na | na | na | na |
| sao_offset_abs[ ][ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| sao_offset_sign[ ][ ][ ][ ] | bypass | na | na | na | na | na |
| sao_band_position[ ][ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| sao_eo_class_luma | bypass | bypass | bypass | na | na | na |
| sao_eo_class_chroma | bypass | bypass | bypass | na | na | na |
| split_cu_flag[ ][ ] | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| cu_transquant_bypass_flag | 0 | na | na | na | na | na |
| cu_skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| pred_mode_flag | 0 | na | na | na | na | na |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | na | na |
| part_mode log2CbSize > MinCbLog2SizeY | 0 | 1 | 3 | bypass | na | na |
| pcm_flag[ ][ ] | terminate | na | na | na | na | na |
| prev_intra_luma_pred_flag[ ][ ] | 0 | na | na | na | na | na |
| mpm_idx[ ][ ] | bypass | bypass | na | na | na | na |
| rem_intra_luma_pred_mode[ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| intra_chroma_pred_mode[ ][ ] | 0 | bypass | bypass | na | na | na |
| rqt_root_cbf | 0 | na | na | na | na | na |
| merge_flag[ ][ ] | 0 | na | na | na | na | na |
| merge_idx[ ][ ] | 0 | bypass | bypass | bypass | na | na |
| inter_pred_idc[ x0 ][ y0 ] | ( nPbW + nPbH ) != 12 ? CtDepth[ x0 ][ y0 ] : 4 | 4 | na | na | na | na |
| ref_idx_l0[ ][ ] | 0 | 1 | bypass | bypass | bypass | bypass |
| ref_idx_l1[ ][ ] | 0 | 1 | bypass | bypass | bypass | bypass |
| mvp_l0_flag[ ][ ] | 0 | na | na | na | na | na |
| mvp_l1_flag[ ][ ] | 0 | na | na | na | na | na |
| split_transform_flag[ ][ ][ ] | 5 − log2TrafoSize | na | na | na | na | na |
| cbf_cb[ ][ ][ ] | trafoDepth | na | na | na | na | na |
| cbf_cr[ ][ ][ ] | trafoDepth | na | na | na | na | na |
| cbf_luma[ ][ ][ ] | trafoDepth = = 0 ? 1 : 0 | na | na | na | na | na |
| abs_mvd_greater0_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_greater1_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_minus2[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| mvd_sign_flag[ ] | bypass | na | na | na | na | na |
| cu_qp_delta_abs | 0 | 1 | 1 | 1 | 1 | bypass |
| cu_qp_delta_sign_flag | bypass | na | na | na | na | na |
| transform_skip_flag[ ][ ][ ] | 0 | na | na | na | na | na |
| last_sig_coeff_x_prefix | 0..17 (subclause Error: Reference source not found) |  |  |  |  |  |
| last_sig_coeff_y_prefix | 0..17 (subclause Error: Reference source not found) |  |  |  |  |  |
| last_sig_coeff_x_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| last_sig_coeff_y_suffix | bypass | bypass | bypass | bypass | bypass | bypass |
| coded_sub_block_flag[ ][ ] | 0..3 (subclause Error: Reference source not found) | na | na | na | na | na |
| sig_coeff_flag[ ][ ] | 0..41 (subclause Error: Reference source not found) | na | na | na | na | na |
| coeff_abs_level_greater1_flag[ ] | 0..23 (subclause Error: Reference source not found) | na | na | na | na | na |
| coeff_abs_level_greater2_flag[ ] | 0..5 (subclause Error: Reference source not found) | na | na | na | na | na |
| coeff_abs_level_remaining[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| coeff_sign_flag[ ] | bypass | na | na | na | na | na |
| Rice_param_prefix | 0 | na | na | na | na | na |
| Rice_param_suffix | bypass | na | na | na | na | na |

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).