| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0249 |
| --- | --- |

| Title: | On frame packing arrangement SEI message for 4:4:4 content in 4:2:0 bitstreams |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Döne Bugdayci, Kemal Ugur, Miska M. Hannuksela | Email: | ext-done.bugdayci@nokia.com kemal.ugur@nokia.com miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution provides a comparison between coding of 4:4:4 content in 4:2:0 bitstreams using the frame packing arrangement SEI message described in JCTVC-N0270 and single layer coding of 4:4:4 content under common test conditions for range extension and screen content sequences. Experimental results for main tier show that the 4:4:4 frame packing arrangement requires on average (335%, 1725%, 3110%) additional bitrate for Y,U,V components, respectively, for range extension sequences and (159%, 518%, 500%) additional bitrate for Y,U,V components, respectively, for screen content sequences.

# Introduction

JCTVC-N0270 provides a method to use the frame packing arrangement SEI message to compress 4:4:4 content using a 4:2:0 codec by packing one 4:4:4 frame information into two 4:2:0 frames e.g. in top-bottom arrangement. This method enables the distribution of 4:4:4 content to HEVC version 1 decoders where decoding of only 4:2:0 bitstreams are possible. Our aim is to measure the performance of this method under the common test conditions of HMRExt and compare it to the case where 4:4:4 content is coded using HMRExt-4.1 codec.

# Experimental Results

## Test set-up

In order to test the SEI method of JCTVC-N0270, frame packing and unpacking operations were conducted using the software provided in [1]. 10-bit support was added to the software to be able to generate results with 10-bit sequences. We tested only direct frame packing and unpacking methods (methods 2 and 4 of the software). Direct frame packing method applies a pre-filtering for anti-aliasing while creating the main 4:2:0 view to be placed in the upper part of the top-bottom arrangement. Lower part contains the auxiliary 4:2:0 view created using the remaining chroma information of the original 4:4:4 frame. Unpacking operation is the reverse of packing to extract the 4:4:4 frame from the packed frame with top-bottom arrangement. The SEI method results were generated using HM12.0. It should be noted that, if the same QP is used for both luma and chroma components, then the chroma PSNR of the test is not matched with the single layer 4:4:4 coded sequence and makes the BD-rate comparisons unreliable. In order to bring the chroma quality of the frame packing test closer to the single layer test, the main and auxiliary view chroma components were coded with a chroma QP offset using the maximum possible value -12. Auxiliary view luma QP was adjusted to be equal to chroma QP, since that part also contains chroma information. This QP arrangement is shown below in Figure 1 for luma QP of 32.

Luma component of main view

Luma component of auxiliary view

Chroma components of main view

Chroma components of auxiliary view

QP = 32

QP = 20

QP = 20

QP = 20

QP = 20

QP = 20

Figure 1. Illustration of QP arrangement for testing the 4:4:4 frame packing arrangement

Single layer 4:4:4 results were generated using HMRExt-4.1 software with the common test conditions.

## Objective results

Table 1 & 2 provide the summary of the comparison for range extension and screen content sequences, respectively. More details about the performance comparison are provided in the attached excel sheet.

Table 1 Single layer 4:4:4 vs SEI, range extension sequences

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| YCbCr 4:4:4 | 181.5% | 418.7% | 622.8% | 160.7% | 345.3% | 388.1% | 121.7% | 219.8% | 325.2% |
|  | Random Access Main-tier |  |  | Random Access High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:4:4 | 335.7% | 2178.2% | 2808.8% | 320.6% | 1477.1% | 2218.1% |  |  |  |
|  | Low delay B Main-tier |  |  | Low delay B High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| YCbCr 4:4:4 | 489.7% | 2579.4% | 5900.5% | 366.9% | 2404.7% | 2510.8% |  |  |  |

Table 2 Single layer 4:4:4 vs SEI, screen content sequences

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| SC YUV 444 | 145.7% | 207.7% | 218.6% | 143.0% | 192.0% | 204.1% | 139.3% | 179.8% | 189.6% |
| Animation YUV 444 | 145.9% | 453.0% | 425.5% | 134.1% | 359.6% | 346.1% | 125.4% | 308.5% | 304.3% |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| SC YUV 444 | 145.5% | 229.9% | 245.4% | 146.2% | 217.9% | 236.8% |  |  |  |
| Animation YUV 444 | 165.8% | 842.6% | 790.4% | 169.3% | 767.2% | 719.9% |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| SC YUV 444 | 148.9% | 251.9% | 266.4% | 151.5% | 240.1% | 263.0% |  |  |  |
| Animation YUV 444 | 200.4% | 1121.3% | 1057.9% | 197.2% | 902.6% | 836.0% |  |  |  |

## Subjective results

Because of the packing and unpacking operations, some artifacts become visible in the unpacked 4:4:4 sequence. To illustrate this, partial images from range extension and screen content sequences coded with all intra configuration and QP=32 are provided for visual inspection. More results are provided in the attached document.

Figure 2. Frame packing artifacts for the Viking sequence

Figure 3. Frame packing artifacts for the Robot sequence

Figure 4. Frame packing artifacts for the Kimono sequence

# References

- Zhang et. al, “Updated proposal with software for frame packing arrangement SEI message for 4:4:4 content in 4:2:0 bitstreams”, JCTVC-N0270, 14th Meeting: Vienna, AT, 25 July – 2 August 2013.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).