| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0251 |
| --- | --- |

| Title: | SHVC HLS: On adaptive resolution change based on single_layer_for_non_irap_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Ying Chen Ye-Kui Wang Adarsh K. Ramasubramonian 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | vseregin@qti.qualcomm.com cheny@qti.qualcomm.com yekuiw@qti.qualcomm.com aramasub@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In SHVC, resolution change can be enabled by setting single_layer_for_non_irap_flag in the VPS equal to 1. This proposal discusses some issues related setting single_layer_for_non_irap_flag equal to 1, including some DPB related issues and other issues. In order to resolve the DPB related issues, it is proposed that, when a resolution switching occurs, all reference pictures in the DPB as marked as "unused for reference" and other decoded pictures with PicOutputFlag equal to 0 are removed from the DPB. For other issues discussed, some bitstream constraints are proposed.

# Resolution change

The semantics of single_layer_for_non_irap_flag is as follows:

single_layer_for_non_irap_flag equal to 1 indicates either that all the VCL NAL units of an access unit have the same nuh_layer_id value or that two nuh_layer_id values are used by the VCL NAL units of an access unit and the picture with the greater nuh_layer_id value is an IRAP picture. single_layer_for_non_irap_flag equal to 0 indicates that nuh_layer_id values may or may not be constrained beyond constraints specified in other parts of this Recommendation | International Standard.

This allows having adaptive resolution switching, for example from a smaller picture resolution to a higher one as shown on the figure below, wherein the picture BL3 is an IRAP picture, and other access units in the figure all contain only one picture, in either the lower layer or the higher layer.

Fig.1 Resolution change

# DPB related issues

With the current design, when resolution switching happens, the reference pictures of the layer from which the resolution was switched (BL in the above example) are still stored in the DPB, but the pictures in the switching-from layer are often no longer needed for inter prediction after the coding of the following access units containing only pictures of the switched-to layer. It is therefore proposed that all reference pictures, including the lower-layer picture of a switching point access unit of the switching-from layer stored in the DPB are marked as "unused for reference". Additionally, for all pictures of the switching-from layer that are not to be output or have been outputted are removed from the DPB. Thus it provides better DPB management and memory usage.

For the example on Fig. 1, when the resolution switching happened in the access unit with POC equal to 3, after decoding of BL3 (base layer picture with POC equal to 3), earlier decoded pictures BL0, BL1, BL2 that might be stored in DPB are no longer necessary for decoding of the BL pictures and can be marked as "unused for reference", and those that are not to be output or have been output, are removed from the DPB. After decoding of EL3 (enhancement layer picture with POC equal to 3), the decoded picture BL3 can be removed from the DPB. Alternatively, all these marking and removal are performed after decoding of EL3.

Alternative to marking of all pictures in the switching-from layer as "unused for reference" and potentially remove all of them from the DPB, it is further possible to keep at least one picture of the switching-from layer in the DPB if it belongs to the lower layer. Let’s name these pictures as "waiting pictures". In this case, when the next resolution switching happens, i.e., from the higher layer to the lower layer, the pictures following the switching point access units in decoding order and belonging to the lower layer may use the waiting pictures for inter prediction.

Fig. 2. Keeping a reference picture for the base layer.

For this scenario, it can be more efficient from compression point of view to keep at least one picture for temporal reference in the base layer, shown as BL3 in Fig. 2, and other base layer pictures can be marked as "unused for reference" as described above. In this case, when switching is done from high to low resolution (or just from higher layer for lower layer), this kept picture can be used for inter prediction shown with red arrow.

We propose to enable both approaches. The proposed changes are provided below.

Changes are marked with yellow colour.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| single_layer_for_non_irap_flag | u(1) |
| if( single_layer_for_non_irap_flag ) |  |
| keep_base_layer_picture_flag | u(1) |
| … |  |
| } |  |

keep_base_layer_picture_flag equal to 1 specifies that at least one picture from the lower layer picture is marked as "used for reference" for future reference after the switching to a higher layer. keep_base_layer_picture_flag equal to 0 specifies all pictures are marked as "unused for reference" after a layer switching. When not present, the value of keep_base_layer_picture_flag is inferred to be equal to 0.

- F.8.1 General decoding process

The specifications in subclause 8.1 apply with following additions.

When the current picture has nuh_layer_id greater than 0, the following applies.

–	Depending on the value of separate_colour_plane_flag, the decoding process is structured as follows:

–	If separate_colour_plane_flag is equal to 0, the following decoding process is invoked a single time with the current picture being the output.

–	Otherwise (separate_colour_plane_flag is equal to 1), the following decoding process is invoked three times. Inputs to the decoding process are all NAL units of the coded picture with identical value of colour_plane_id. The decoding process of NAL units with a particular value of colour_plane_id is specified as if only a CVS with monochrome colour format with that particular value of colour_plane_id would be present in the bitstream. The output of each of the three decoding processes is assigned to one of the 3 sample arrays of the current picture, with the NAL units with colour_plane_id equal to 0, 1 and 2 being assigned to SL, SCb, and SCr, respectively.

NOTE – The variable ChromaArrayType is derived as equal to 0 when separate_colour_plane_flag is equal to 1 and chroma_format_idc is equal to 3. In the decoding process, the value of this variable is evaluated resulting in operations identical to that of monochrome pictures (when chroma_format_idc is equal to 0).

–	The decoding process operates as follows for the current picture CurrPic.

–	For the decoding of the slice segment header of the first slice, in decoding order, of the current picture, the decoding process for starting the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

–	If ViewScalExtLayerFlag[ nuh_layer_id ] is equal to 1, the decoding process for a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

–	Otherwise, when DependencyId[ nuh_layer_id] is greater than 0, the decoding process for a coded picture with nuh_layer_id greater than 0 specified in subclause H.8.1.1 is invoked.

–	After all slices of the current picture have been decoded, the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

- When single_layer_for_non_irap_flag is equal to 1, the following applies:

  - The variable switchingFlag is set to 0.

  - When the current picture is an IRAP picture and there is a picture in the same access unit with a lower value of nuh_layer_id than the current picture, the following applies. The nuh_layer_id values of these two pictures are denoted as layerIdA and layerIdB with layerIdB greater than layerIdA, switchingFlag is set to 1, the variable layerIdSwitch is set as layerIdA, the variable keepPicFlag is set equal to keep_base_layer_picture_flag.

  - When there is only one picture within the current access unit and its nuh_layer_id value is less than the nuh_layer_id value of the picture in the previous access unit, switchingFlag is set to 1, layerIdSwitch is set to the nuh_layer_id value of the picture in the previous access unit, keepPicFlag is set equal to 0.

  - When switchingFlag is equal to 1, the following applies in the order listed:

    - When keepPicFlag is equal to 1, the picture in the same access unit as that of the current picture is marked as “used for reference”.

    - All other reference pictures with nuh_layer_id equal to layerIdSwitch in the DPB are marked as "unused for reference", and other decoded pictures with PicOutputFlag equal to 0 are removed from the DPB. Each of those pictures remaining in the DPB, except for the current picture and, when keepPicFlag is equal to 1, the lower layer picture in the same access unit as the current picture, is removed from the DPB immediately after it is outputted.

# On continuous AUs with two pictures

The current semantics of single_layer_for_non_irap_flag equal to 1 allows for two or more AUs that are continuous in decoding order and that have enhancement layer picture in each of these AUs as IRAP pictures, or even all AUs in the bitstream contain two pictures with IRAP pictures at the enhancement layer, and yet the two values of nuh_layer_id of one AU can be both different from that of the next AU in decoding order.

Such strange behaviors should not be allowed to achieve the coding efficiency for the switching point introduced by the technique supported by this flag. It is therefore proposed to require that, when single_layer_for_non_irap_flag is equal to 1, there shall not be two AUs continuous in decoding order that both contain two pictures.

# On target output layers

To fit the design of adaptive resolution change using single_layer_for_non_irap_flag is equal to 1 in SHVC, the number of target output layers is set equal to number of layers present in the bitstream, and each layer in the bitstream is a target output layer, though at most one picture at any access unit is output, and this is the picture of the highest layer.

Furthermore, the following restriction is proposed:

When single_layer_for_non_irap_flag is equal to 1, in each access unit that has two pictures (of two layers), the one in the lower layer shall have pic_output_flag in each slice segment header equal to 0.

# On layer jumping

The current semantics of single_layer_for_non_irap_flag equal to 1 allows for such jumping of layers. At access unit (AU) n, there is only the base layer picture (nuh_layer_id equal to 0), and in the next AU in decoding order, there are two pictures with nuh_layer_id equal to 1 and 2, respectively, or with nuh_layer_id equal to any other two values of i and j greater than 0 and less than 63.

Such layer jumping should not be allowed, because of e.g., the incremental burden of number of layers to be maintained in the decoded picture buffer (as the number is proportional to the number of such “jumping AUs”), and the possibility of keeping a lot of frames being marked as “used for reference” in the lower layers.

It is therefore proposed to require that, when single_layer_for_non_irap_flag is equal to 1, in each AU containing two pictures of two layers, one of the two layers shall be the same as the layer of the picture contained in the previous AU that contains only one picture.

# On signalling of derivation of picture order count

To enable correct values of picture order count for pictures, one of the following constraints is applied when single_layer_for_non_irap_flag is equal to 1:

- For each access unit that contains two pictures, that the previous access unit in decoding order has only one picture, and that the greater value of nuh_layer_id among all VCL NAL units in the access unit is greater than the nuh_layer_id of the VCL NAL units of the picture in the previous access unit in decoding order (i.e. this access unit is the first access unit where an up-switching occurs), the contained higher-layer picture shall be an IDR picture or a BLA picture. This way, the derivation of the POC value of the higher-layer picture and subsequent pictures in decoding order in the same layer does not rely on information from previous access units.

  - If such a constraint is not required, the derivation of the POC of the IRAP picture in the switching point will be problematic, since there is no previous POC anchor picture for such an IRAP picture. More specifically, missing of POC MSB makes it impossible to derive the POC value of such a picture, unless POC MSB can be somehow predicted from the base layer. However, such cross-layer behavior is usually not desirable.

- For each access unit that contains two pictures, both of the pictures shall have poc_reset_flag equal to 1 in the slice segment headers, or the derivation of the POC value of both of the pictures is made independent from information from previous access units by any other means.

# On bitstream extraction

Typical SHVC bitstreams are backward compatible to HEVC version 1. However, when a bitstream with single_layer_for_non_irap_flag is equal to 1 is fed into a bitstream extractor with a target layer being the base layer, the resulted bitstream is problematic: for a significant long period of time, there are no the pictures in the extracted bitstream being present, which provides unacceptable user experience.

Therefore bitstream extraction should be disallowed for such a bitstream with single_layer_for_non_irap_flag equal to 1, which can be realized by adding the following restriction (because only those output layer set specified by the VPS are required to be conforming sub-bitstreams, if extracted by the sub-bitstream extraction process):

When single_layer_for_non_irap_flag is equal to 0, there shall be only one layer set and only one output layer set specified in the VPS, and the output layer set contains all layers in the bitstream and all the layers are target output layers.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).