| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0253 |
| --- | --- |

| Title: | On constraints of the Scalable Main profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Kemal Ugur Nokia Research Center | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution includes two aspects related to the specification of the Scalable Main profile:

- Profiles are indicated in the VPS extension syntax for each output layer set, not for the entire bitstream. Consequently, the constraints for profiles, such as the Scalable Main profile, are proposed to be written to concern output layer sets.

- It is asserted that the constraint on avc_base_layer_flag appears to be true not only for the Scalable Main profile but to any bitstreams following the SHVC or MV-HEVC decoding process, because no mapping process of AVC motion fields has been specified. It is therefore proposed to move the constraint to Annex F.

# Background

The Scalable Main profile is currently specified as follows:

- H.11.1.2	Scalable Main profile

Bitstreams conforming to the scalable main profile shall obey the following constraints:

–	The picture resampling process of picture sample values as specified in subclause Error: Reference source not found shall not be invoked more than once for decoding of each particular picture and the resampling process of picture motion field as specified in subclause Error: Reference source not found shall not be invoked more than once for decoding of each particular picture. When both picture sample values and picture motion field resampling processes are invoked for decoding of a particular picture, they shall be applied to the same reference layer picture.

–	When DependencyId[ i ] is equal to 1, ScalabilityId[ LayerIdxInVps [ i ]][ smIdx ] shall be equal to 0 for any smIdx value from 0 to 15, inclusive, that is not equal to 2, for any coded picture with nuh_layer_id equal to i.

–	When avc_base_layer_flag equal to 1, it is a requirement of bitstream conformance that MotionPredEnabledFlag[ iNuhLId ][ 0 ] shall be equal to 0 for iNuhLId equal to any value of nuh_layer_id present in the bitstream.

# Proposed specification text

- F.7.4.3.1.1.	Video parameter set extension semantics

…

When avc_base_layer_flag equal to 1, it is a requirement of bitstream conformance that MotionPredEnabledFlag[ iNuhLId ][ 0 ] shall be equal to 0 for iNuhLId equal to any value of nuh_layer_id present in the bitstream.

- H.11.1.2	Scalable Main profile

Output layer setsBitstreams conforming to the scalable main profile shall obey the following constraints with layerSetIdx being the layer set for an output layer set conforming to the Scalable Main profile:

–	For the pictures with nuh_layer_id included in the layer set layerSetIdx, the picture resampling process of picture sample values as specified in subclause Error: Reference source not found shall not be invoked more than once for decoding of each particular picture and the resampling process of picture motion field as specified in subclause Error: Reference source not found shall not be invoked more than once for decoding of each particular picture. When both picture sample values and picture motion field resampling processes are invoked for decoding of a particular picture, they shall be applied to the same reference layer picture.

–	–	When DependencyId[ i ] is equal to 1, ScalabilityId[ LayerIdxInVps [ i ]][ smIdx ] shall be equal to 0 for any smIdx value from 0 to 15, inclusive, that is not equal to 2, for any coded picture with nuh_layer_id equal to i.

–	ScalabilityId[ j ][ smIdx ] shall be equal to 0 for any smIdx value not equal to 2 and for any value of j for which layer_id_included_flag[ layerSetIdx ][ j ] is equal to 1.

–	When avc_base_layer_flag equal to 1, it is a requirement of bitstream conformance that MotionPredEnabledFlag[ iNuhLId ][ 0 ] shall be equal to 0 for iNuhLId equal to any value of nuh_layer_id present in the bitstream.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).