| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0255 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0085 |
| --- | --- |

| Title: | SHVC / MV-HEVC HLS: On motion and inter-layer constrained tile set SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Krishna Rapaka Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | hendry@qti.qualcomm.com krapaka@qti.qualcomm.com yekuiw@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution discusses some issues of the current design of the motion constrained tile set (MCTS) SEI message and the inter-layer constrained tile set (ILCTS) SEI message, and proposes three bitstream restrictions to be incorporated into the semantics. The proposed restrictions are as follows:

- For both the MCTS and the ILCTS SEI messages, any two tiles in each tile set, shall either be neighboring to each other or the region between them belong to other tiles in the same tile set.

- For both the MCTS and the ILCTS SEI messages, when there is more than one rectangular region in a tile set, the rectangular regions shall be signalled in increasing order of top_left_tile_index[ i ][ j ].

- The ILCTS SEI message shall not be present when extended scalability is used.

# Problem Description

HEVC Range Extensions includes the motion constrained tile sets (MCTS) SEI message to define tile set in which a coding tree block belongs to a tile set shall only have motion prediction by referencing to area within the same tile set. Furthermore, SHVC, MV-HEVC, and 3D-HEVC include the inter-layer constrained tile sets (ILCTS) SEI message to define tile set in which inter-layer prediction is constrained to refer to corresponding particular tile sets only.

The current designs of the MCTS and the ILCTS SEI messages have three shortcomings as described below.

The first shortcoming is that it is possible to have a tile set that contains tiles that are not neighbors. The following figure illustrates two examples of tile set configuration where non-neighboring tiles are grouped into a tile set. Configuration in which non-neighboring tiles are grouped into a tile set is not common in practical applications and would cause additional difficulty for implementations.

Figure 1 – Examples of non-neighboring tiles in a tile set.

The second shortcoming is that it is possible to have many signaling variation for tile sets that is geometrically identical. It would be more natural and more desirable if the order of rectangular areas within a tile set is governed so that we have the same signaling for the same geometry shape. The following figure illustrates show order variation of signalling rectangular regions in a tile set.

Figure 2 – Example of variation of order in signalling rectangular regions in a tile set.

The third shortcoming arises when extended spatial scalability is used. In the case of extended spatial scalability a region in the enhancement layer may not have corresponding/collocated region in the reference layer as shown in Figure 3. In the current design of theILCTS SEI message, the syntax element “il_num_sets_in_message_minus1” or the number of tiles needs to be same for all the pictures in the same access units. Furthermore MV-HEVC WD5 and SHVC WD3 apply the constraint that the ILCTS SEI message shall not be present unless every PPS that is active for a layer has tile_boundaries_aligned_flag equal to 1 or fulfills the conditions that would be indicated by tile_boundaries_aligned_flag being equal to 1.

Figure 3: Example extended spatial scalability configuration across layers

However, for extended spatial scalability scenario, it is possible to have more tiles/tile sets in the enhancement layer than the base layer and still fulfill the conditions that would be indicated by tile_boundaries_aligned_flag being equal to 1.

# Proposed solutions

To overcome the above shortcomings, the following three restrictions are proposed:

- For both the MCTS and the ILCTS SEI messages, any two tiles in each tile set, shall either be neighboring to each other or the region between them belong to other tiles in the same tile set.

- For both the MCTS and the ILCTS SEI messages, when there is more than one rectangular region in a tile set, the rectangular regions shall be signalled in increasing order of top_left_tile_index[ i ][ j ].

- The ILCTS SEI message shall not be present when extended scalability is used.

The text changes are provided below.

# Changes to text specification

## Restriction on rectangular region in a tile set

The following restriction is proposed to overcome the first shortcoming described in Section 3. The proposed restriction is highlighted in yellow color.

Proposed constraint for the MCTS SEI message

top_left_tile_index[ i ][ j ] and bottom_right_tile_index[ i ][ j ] identify the tile position of the top-left tile and the tile position of the bottom-right tile in a rectangular region of the i-th identified motion-constrained tile set, respectively, in tile raster scan order. When j is greater than 0, the rectangular region that is defined by top_left_tile_index[ i ][ j ] and bottom_right_tile_index[ i ][ j ] shall contain one or more tiles that are directly adjacent to one or more tiles in the rectangular region defined by top_left_tile_index[ i ][ k ] and bottom_right_tile_index[ i ][ k ], where k ranges from 0 to j – 1, inclusive.

Proposed constraint for the ILCTS SEI message

il_top_left_tile_index[ i ][ j ] and il_bottom_right_tile_index[ i ][ j ] identify the tile position of the top-left tile and the tile position of the bottom-right tile in a rectangular region of the i-th identified inter-layer constrained tile set, respectively, in tile raster scan order. When j is greater than 0, the rectangular region that is defined by il_top_left_tile_index[ i ][ j ] and il_bottom_right_tile_index[ i ][ j ] shall contain one or more tiles that are directly adjacent to one or more tiles  in the rectangular region defined by il_top_left_tile_index[ i ][ k ] and il_bottom_right_tile_index[ i ][ k ], where k ranges from 0 to j – 1, inclusive.

## Restriction on the value of top_left_tile_index[ i ][ j ]

The following restriction is proposed to overcome the second shortcoming described in Section 3. The proposed restriction is highlighted in yellow color.

Proposed constraint for the MCTS SEI message

top_left_tile_index[ i ][ j ] and bottom_right_tile_index[ i ][ j ] identify the tile position of the top-left tile and the tile position of the bottom-right tile in a rectangular region of the i-th identified motion-constrained tile set, respectively, in tile raster scan order. For any m and n in the range of 0 to num_tile_rects_in_set_minus1[ i ], inclusive, where m is less than n, the value of top_left_tile_index[ i ][ m ] shall be less than the value of top_left_tile_index[ i ][ n ].

Proposed constraint for the ILCTS SEI message

il_top_left_tile_index[ i ][ j ] and il_bottom_right_tile_index[ i ][ j ] identify the tile position of the top-left tile and the tile position of the bottom-right tile in a rectangular region of the i-th identified inter-layer constrained tile set, respectively, in tile raster scan order. For any m and n in the range of 0 to il_num_tile_rects_in_set_minus1[ i ], inclusive, where m is less than n, the value of il_top_left_tile_index[ i ][ m ] shall be less than the value of il_top_left_tile_index[ i ][ n ].

## Restriction on inter-layer constrained tile sets

Proposed changes regarding to the contrstraints on the presence of the ILCTS SEI message are as follows, wherein texts highlighted in yellow are additions, and striketroughs are removals:

The inter-layer constrained tile sets SEI message shall not be present for a layer when tiles_enabled_flag is equal to 0 for any PPS that is active for a in the layer.

The inter-layer constrained tile sets SEI message shall not be present for a layer unless every PPS that is active for athe layer has tile_boundaries_aligned_flag equal to 1 or fulfills the conditions that would be indicated by tile_boundaries_aligned_flag being equal to 1.

The inter-layer constrained tile sets SEI message shall not be present unless for any SPS that is active for a layer, the values of scaled_ref_layer_left_offset[ i ], scaled_ref_layer_top_offset[ i ], scaled_ref_layer_right_offset[ i ], scaled_ref_layer_bottom_offset[ i ] for all values of i are all equal to 0.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).