| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0259_r1 |
| --- | --- |

| Title: | AHG18: On Supporting Higher Bit Depth With Transform Skip Only Changes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Seung-Hwan Kim 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com kimse@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

It is asserted that to support higher bit depth the current HEVC range extension draft requires transforms capable of processing a larger data range when compared to HEVC version 1. It is proposed that higher bit depth be supported by changing the transform skip processing path only. It is asserted that the proposed design is desirable since it does not require a change to the core transform processing capabilities. The performance of the proposed design is evaluated using AHG18 anchors and the average luma BD rate change is found to be 0.6% (12-bit), 1.1% (14-bit), 0.6% (16-bit).

Revision 1 of the document contains experimental results for AHG18 test conditions. It also includes specification text changes reflecting the use of 16-bit clip after the first 1-D inverse transform (which is the same as in HEVC version 1).

# Introduction

The current HEVC range extension draft extends the clipping range at the output of the dequantizer to Max(16, (BitDepth+7)) bits when extended_precision_processing_flag is equal to 1. This increase in clipping range results in dequantized coefficient values greater than 16-bits being passed to the inverse transform process. The increased input range requires a change to the core transform processing capabilities when compared with HEVC version 1.

This contribution proposes a design where higher bit depth is supported by changing the transform skip processing path only. Figure 1 illustrates the proposed changes.

Figure 1: Illustration identifying the proposed design

In Figure 1, the variable BD represents sample bit depth, M represents the Log2 of block size. The specification text changes in [1] corresponding to the proposed modification are listed in section 6.

# Experimental Results

The proposed approach is integrated in HM-12.0-RExt-4.1 software and evaluated using AHG18 lossy test conditions for higher bit depth. The summary of results listed below. The detailed results can be found in the attached excel document.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | 0.6% | 0.9% | 0.9% |
| 14-bit | 1.1% | 2.3% | 2.6% |
| 16-bit | 0.6% | 0.4% | 0.4% |

Note, the HM-12.0-RExt-4.1 software which is used as anchor above differs from [1] as described in bug-report #1185.

- Conclusion

The contribution proposes a design where higher bit depth is supported by changing the transform skip processing path only. The performance of the proposed design is evaluated using AHG18 anchors and the average luma BD rate change with AHG18 anchors is found to be 0.6% (12-bit), 1.1% (14-bit), 0.6% (16-bit).

# References

[1] David Flynn, Joel Sole, Teruhiko Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4,” JCTVC-N1005, 14th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Vienna, AT, July – 2 Aug. 2013.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Specification text change

The specification text change for “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4” is highlighted in yellow.

- 8.6.3	Scaling process for transform coefficients

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable nTbS specifying the size of the current transform block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable qP specifying the quantization parameter.

Output of this process is the (nTbS)x(nTbS) array d of scaled transform coefficients with elements d[ x ][ y ].

The variables bdShift, coeffMin and coeffMax are derived as follows:

–	If cIdx is equal to 0,

bdShift = BitDepthY + Log2( nTbS ) − 5		(8-1)

coeffMin = CoeffMinY		(8-2)

coeffMax = CoeffMaxY		(8-3)

–	Otherwise,

bdShift = BitDepthC + Log2( nTbS ) − 5		(8-4)

coeffMin = CoeffMinC		(8-5)

coeffMax = CoeffMaxC		(8-6)

  - If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0,

coeffMin = −32768

coeffMax = 32767

  - If cIdx is equal to 0,

bdShift = BitDepthY + Log2( nTbS ) – 5	(8-xxx)

  - Otherwise,

bdShift = BitDepthC + Log2( nTbS ) − 5	(8-xxx)

  - Otherwise, (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1)

  - If cIdx is equal to 0,

bdShift = BitDepthY + Log2( nTbS ) − 5 – Max( BitDepthY + Log2( nTbS ) − 15, 0 )	(8-xxx)

coeffMin = −( 1  <<  ( extended_precision_processing_flag ? Max( 15, BitDepthY ) : 15 ) )	(8-xxx)

coeffMax = ( 1  <<  ( extended_precision_processing_flag ? Max( 15, BitDepthY ) : 15 ) ) − 1	(8-xxx)

  - Otherwise,

bdShift = BitDepthC + Log2( nTbS ) − 5– Max( BitDepthC + Log2( nTbS ) − 15, 0 )	(8-xxx)

coeffMin = −( 1  <<  ( extended_precision_processing_flag ? Max( 15, BitDepthC ) : 15 ) )	(8-xxx)

coeffMax = ( 1  <<  ( extended_precision_processing_flag ? Max( 15, BitDepthC ) : 15 ) ) − 1	(8-xxx)

The list levelScale[ ] is specified as levelScale[ k ] = { 40, 45, 51, 57, 64, 72 } with k = 0..5.

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1, the following applies:

–	The scaling factor m[ x ][ y ] is derived as follows:

–	If scaling_list_enabled_flag is equal to 0,

m[ x ][ y ] = 16		(8-282)

–	Otherwise (scaling_list_enabled_flag is equal to 1),

m[ x ][ y ] = ScalingFactor[ sizeId ][ matrixId ][ x ][ y ]	(8-283)

Where sizeId is specified in Error: Reference source not found for the size of the quantization matrix equal to (nTbS)x(nTbS) and matrixId is specified in Error: Reference source not found for sizeId, CuPredMode[ xTbY ][ yTbY ], and cIdx, respectively.

–	The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( coeffMin, coeffMax, ( ( TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ][ x ][ y ] * m[ x ][ y ] * 
			levelScale[ qP%6 ]  <<  (qP / 6 ) ) + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift )	(8-248)

### 8.6.4	Transformation process for scaled transform coefficients

- 8.6.4.1	General

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable nTbS specifying the size of the current transform block,

–	a variable cIdx specifying the colour component of the current block,

–	an (nTbS)x(nTbS) array d of scaled transform coefficients with elements d[ x ][ y ].

Output of this process is the (nTbS)x(nTbS) array r of residual samples with elements r[ x ][ y ].

The variables coeffMin and coeffMax are derived as follows:

–	If cIdx is equal to 0,

coeffMin = CoeffMinY		(8-7)

coeffMax = CoeffMaxY		(8-8)

–	Otherwise,

coeffMin = CoeffMinC		(8-9)

coeffMax = CoeffMaxC		(8-10)

Depending on the values of CuPredMode[ xTbY ][ yTbY ], nTbS, and cIdx, the variable trType is derived as follows:

–	If CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA, nTbS is equal to 4, and cIdx is equal to 0, trType is set equal to 1.

–	Otherwise, trType is set equal to 0.

The (nTbS)x(nTbS) array r of residual samples is derived as follows:

- Each (vertical) column of scaled transform coefficients d[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 is transformed to e[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 by invoking the one-dimensional transformation process as specified in subclause 8.6.4.2 for each column x = 0..nTbS − 1 with the size of the transform block nTbS, the list d[ x ][ y ] with y = 0..nTbS − 1, and the transform type variable trType as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbS − 1.

- The intermediate sample values g[ x ][ y ] with  x = 0..nTbS − 1, y = 0..nTbS − 1  are derived as follows:

g[ x ][ y ] = Clip3( coeffMin -32768, coeffMax32767, ( e[ x ][ y ] + 64 )  >>  7 )	(8-xxx)

- Each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 is transformed to r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 by invoking the one-dimensional transformation process as specified in subclause 8.6.4.2 for each row y = 0..nTbS − 1 with the size of the transform block nTbS, the list g[ x ][ y ] with x = 0..nTbS − 1, and the transform type variable trType as inputs, and the output is the list r[ x ][ y ] with x = 0..nTbS − 1.