| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0260 |
| --- | --- |

| Title: | Inter Prediction Signalling and Picture Marking |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Sachin Deshpande 5750 NW Pacific Rim Blvd Camas, WA 98607, USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com sdeshpande@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

In this document a slice segment header flag is signalled to indicate if a picture is used for inter prediction in the decoding process of subsequent decoded pictures in the current layer. Also picture marking process is added to the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0 utilizing the signalled flag. It is asserted that in certain coding structures DPB memory could be reduced using proposed approach.

# Introduction

This document proposes following:

- A slice segment header flag is signalled to indicate if a picture is used for inter prediction in the decoding process of subsequent decoded pictures in the current layer

- Picture marking process is added to the decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0.

It is asserted that in certain coding structures DPB memory could be reduced using proposed approach.  For example in a mixed frame rate case where a layer has higher frame rate than its reference layer and a DPB is shared between the layers then the DPB memory could be reduced using the proposed approach.

# Proposal

Proposed changes to JCTVC-N1008 [1] are highlighted in yellow below:

F.7.4.3.1.1	Video parameter set extension semantics

…

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with index j may be a direct reference layer for the layer with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

The variables NumDirectRefLayers[ i ], and RefLayerId[ i ][ j ] SamplePredEnabledFlag[ i ][ j ], MotionPredEnabledFlag[ i ][ j ], and DirectRefLayerIdx[ i ][ j ], and UsedForInterLyrRefFlag[ i ][ j ] are derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	iNuhLId = layer_id_in_nuh[ i ]
	NumDirectRefLayers[ iNuhLId ] = 0
	for( j = 0; j < i; j++ ) {
		if( direct_dependency_flag[ i ][ j ] ) {
			RefLayerId[ iNuhLId ][ NumDirectRefLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ]
			SamplePredEnabledFlag[ iNuhLId ][ j ] = ( ( direct_dependency_type[ i ][ j ] + 1 ) & 1 )
			MotionPredEnabledFlag[ iNuhLId ][ j ] = ( ( ( direct_dependency_type[ i ][ j ] + 1 ) & 2 ) >> 1 )
			DirectRefLayerIdx[ iNuhLid ][ layer_id_in_nuh[ j ] ] = NumDirectRefLayers[ iNuhLId ] – 1			}
		for( k = i+1, UsedForInterLyrRefFlag[ iNuhLId ][ j ] = 0; k <= vps_max_layers_minus1 && ! UsedForInterLyrRefFlag[ iNuhLId ][ j ]; k++ ) 
			if (direct_dependency_flag[ k ][ j ])
				UsedForInterLyrRefFlag[ iNuhLId ][ j ] = 1
	}
}

…

- F.7.3.6	Slice segment header syntax

- F.7.3.6.1	General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !dependent_slice_segment_flag ) { |  |
| … |  |
| if( nuh_layer_id > 0 && ! all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( nuh_layer_id > 0 ) |  |
| unused_for_pred_in_cur_layer_flag | u(1) |
| … |  |
| } |  |
| … |  |
| } |  |

- F.7.4.7	Slice segment header semantics

- F.7.4.7.1	General slice segment header semantics

…

unused_for_pred_in_cur_layer_flag equal to 1 specifies that the current picture with nuh_layer_id equal to currLayerId is not used as a reference picture for inter prediction in the decoding process of subsequent pictures in decoding order with nuh_layer_id equal to currLayerId. unused_for_pred_in_cur_layer_flag equal to 0 specifies that the current picture with nuh_layer_id equal to currLayerId may be used as a reference picture for inter prediction in the decoding process of subsequent pictures in decoding order with nuh_layer_id equal to currLayerId. When not present, the value of unused_for_pred_in_cur_layer_flag is inferred to be equal to 0.

The variable UnusedForInterPredFlag is derived as follows:

UnusedForInterPredFlag[ nuh_layer_id ] = unused_for_pred_in_cur_layer_flag.

It is a requirement of bitstream conformance that the value of unused_for_pred_in_cur_layer_flag shall be the same for all slices of a coded picture.

…

- F.8.1.2	Decoding process for ending the decoding of a coded picture with nuh_layer_id greater than 0

PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, if LayerInitialisedFlag[ nuh_layer_id ] is equal to 0, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

The following applies:

–	If discardable_flag is equal to 1, the decoded picture is marked as "unused for reference".

–	Otherwise, the decoded picture is marked as "used for short-term reference".

When TemporalId is equal to HighestTid, the marking process for sub-layer non-reference pictures not needed for inter-layer prediction specified in subclause F.8.1.2.1 is invoked with latestDecLayerId equal to nuh_layer_id as input.

FirstPicInLayerDecodedFlag[ nuh_layer_id ] is set equal to 1.

For i in the range of 0 to LayerIdxInVps[ nuh_layer_id ] − 1, inclusive, the following applies

–	Let bPic be the picture in the current access unit with nuh_layer_id equal to layer_id_in_nuh[ i ].

–	When LayerInitialisedFlag[ layer_id_in_nuh[ i ] ] is equal to 1, FirstPicInLayerDecodedFlag[ layer_id_in_nuh[ i ] ] is equal to 1, UsedForInterLyrRefFlag[ nuh_layer_id ][ i ] is equal to 0, UnusedForInterPredFlag[ layer_id_in_nuh[ i ] ] is equal to 1, the decoded picture bPic is marked as "unused for reference"

# References

[1] Jianle Chen, Jill Boyce, Yan Ye, Miska M. Hannuksela, “High efficiency video coding (HEVC) scalable extension draft 3,” JCTVC-N1008_v3, 14th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Vienna, AT, July – 2 Aug. 2013.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).