| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0261 |
| --- | --- |

| Title: | SCE4: Cross-check of test 5.3 color prediction with 3D LUT (10bit base case) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or | Jie Zhao Seung-Hwan Kim | Email: | jzhao@sharplabs.com kimse@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This document reports the cross-check results of SCE4 test 5.3 “test case 2”-10-bit base on Color Gamut and Bit-Depth Scalability proposed in JCTVC-O159 [1] by Technicolor. JCTVC-O159 proposed inter layer prediction based on the use of a 3D Look-Up Table (LUT). We tested the case of using 10-bit base layer, and confirmed that the SCE4 test 5.3 software matches with the document and the RD results match those provided by the proponent.

# Introduction

In Color Gamut Scalability, Color Predictor module is to predict the EL color samples from the collocated BL color samples. JCTVC-O159 stated that the determination of this color transfer function is not straightforward, and they used 3D color Look-Up Tables (LUT) to perform inter-layer prediction. The 3D LUTs used in their simulations were trained offline, and provided as extern files to the encoder. The 3D LUT is signaled to decoder using octree based description.

# Experimental results

We used the source code provided by proponent, and modified the encoder config file to use the corresponding LUT file. The experimental setup corresponds to the SCE4 description [2].

We tested the “test case 2” – 10 bit base only as requested. Table 1 shows the summary RD performance of the tested method compared with the SHM-3.0.1-CE4 anchor in SCE4 common test conditions. The BD rate results matched with those provided by proponent. Below are the detailed results. Since our cluster is heterogeneous, we did not verify coding time.

Table 1. Summary result of SCE4 test 5.3 10 bit base

|  | AI HEVC 2x 10-bit base |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A+ | -12.2% | -9.6% | -14.9% |
| Overall (Test vs Ref) | -12.2% | -9.6% | -14.9% |
| Overall (Test vs single layer) | 11.2% | 16.8% | 9.8% |
| Overall (Ref vs single layer) | 26.8% | 29.4% | 28.9% |
| EL only (Test vs Ref) | -22.3% | -19.6% | -24.5% |
| Overall (Test EL+BL vs single EL+BL) | -27.1% | -23.5% | -28.5% |
|  | RA HEVC 2x 10-bit base |  |  |
|  | Y | U | V |
| Class A+ | -8.5% | -3.4% | -10.1% |
| Overall (Test vs Ref) | -8.5% | -3.4% | -10.1% |
| Overall (Test vs single layer) | 20.9% | 31.0% | 19.3% |
| Overall (Ref vs single layer) | 32.3% | 35.5% | 33.0% |
| EL only (Test vs Ref) | -15.2% | -9.7% | -16.2% |
| Overall (Test EL+BL vs single EL+BL) | -19.4% | -12.1% | -19.8% |

# Conclusions

It is confirmed that all RD results generated for the cross-check match the results provided by the proponents on the SCE4 participant e-mail list and also in JCTVC-O0159. Software matches with the document.

# References

- P. Bordes, P. Andrivon, F. Hiron, P. Salmon, R. Boitard, “SCE4: Results on 5.3-test1 and 5.3-test2”, JCTVC-O0159, Geneva, October 2013.

[2] A. Segall, P. Bordes, C. Auyeung, X. Li, E. Alshina, A. Duenas, “Description of Core Experiment SCE4: Color Gamut and Bit-Depth Scalability”, JCTVC-N1104, 14th Meeting: Vienna, Austria, AT, July, 2013.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).