| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0262 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0086 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: decoding operation in the case of missing reference pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution depends on the capability of specifying alternative output layers (JCTVC-O0153/JCT3V-F0058).

The reference picture set design enables decoders to detect unintentional losses of reference pictures and the layer-wise start-up process enables decoders to increment the number of decoded layers in a step-wise manner. It is proposed that if a missing reference picture in a reference picture set for an enhancement-layer picture is detected and an alternative output layer exists for that enhancement layer, the following applies:

- The layer-wise start-up process is initiated for the enhancement layer.

- Pictures of an alternative output layer are output until the enhancement layer contains an IRAP picture.

It is asserted that the proposed decoder operation provides tolerance to picture losses and careless bitstream manipulation e.g. in middle-boxes.

# Overview of the Proposal

The reference picture set design enables decoders to detect unintentional losses of reference pictures. If a reference picture is lost in an enhancement layer, a decoder can continue decoding of a lower layer and switch up to decoding the enhancement layer the next time it contains an IRAP picture. The specification of alternative output layers as proposed in JCTVC-O0153/JCT3V-F0058 enables decoders to determine, when the output of the lower layer is considered valid.

It is proposed that if a missing reference picture in a reference picture set for an enhancement-layer picture is detected and an alternative output layer exists for that enhancement layer, the following applies:

- The layer-wise start-up process is initiated for the enhancement layer.

- Pictures of an alternative output layer are output until the enhancement layer contains an IRAP picture.

The above-proposed impacts are proposed to be realized in the specification text as follows:

- LayerInitialisedFlag is set equal to 0 for the enhancement layer. This causes OutputFlag to be set to 0 for the enhancement layer until the next IRAP picture in the enhancement layer and the use of an alternative output layer for picture output.

- FirstPicInLayerDecodedFlag is set equal to 0 for the picture referring to a missing reference picture. This causes the generation of unavailable reference pictures to be invoked.

If a missing reference picture in a reference picture set for an enhancement-layer picture is detected and no alternative output layer exists for that enhancement layer, a bitstream conformance violation is concluded. Hence, in MV-HEVC for which no alternative output layers are specified, no change in the decoder operation is caused.

The detailed proposal is included in the change marks of the accompanying specification text document.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).