| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0263 |
| --- | --- |

| Title: | Non-RCE1: Inter colour-component residual coefficients prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

This contribution proposes the inter colour-component residual coefficient prediction for HEVC Range Extensions. In the range extensions’ core experiments 1 (RCE1), inter-colour component residual prediction is studied. This proposal presents an improvement of RCE1 Experiment 2. In the proposed method, inter component prediction is performed before inverse transforms. Compared to Experiment 2, dependent point in a decoding pipeline becomes forward. No syntax change is required to the Experiment 2. For common test conditions of RGB sequences, the average BD-rates of the method are 16.5%, 13.0%, and 12.3% for AI, RA, and LDB main tiers, respectively. For YUV sequences, the average BD-rates are 1.2%, 0.4%, and 0.2% for AI, RA, LDB main tiers, respectively.

# Introduction

In JCTVC-N0266 [1], an inter colour component residual prediction method was proposed as follows.

rC'(x,y)=ΔrC'(x,y)+α×rL'(x,y)>>3	(1)

where
rL' denotes the reconstructed residual sample of the luma component,
ΔrC denotes the predicted signal using inter-color prediction,
ΔrC'denotes the reconstructed signal after coding and decoding ΔrC, and
rC' denotes the reconstructed chroma residual.

This method shows good coding gain in both RGB and YCbCr test sequences. A dependency of inter colour-component is remained in the signal processing stage for a decoder pipeline.

# Technical descriptions

## Prediction method

In this proposal, the chroma residual is predicted not in signal domain but in transformed domain. The difference to the method in JCTVC-N0266 is a stage of inter colour-component processing. Figure 1, 2, and 3 show the difference of in-loop residual prediction (ILP) / in-loop residual coefficient prediction (ILCP) stages among HM_RExt, JCTVC-N0266, and the proposed method.

Figure 1 Decoding process of HEVC

Figure 2 Decoding process of HEVC with ILP

Figure 3 Decoding process of HEVC with the proposed ILCP

When a transform type like DCT / DST / TS is different between luma and chroma (G and B/R), the proposed method cannot be applied to such transform unit. Therefore, we propose three options.

## Proposal 1

Proposal 1 means no syntax change to JCTVC-N0266. The stage of the inter colour-component process becomes forward to inverse transform in a decoder pipeline.

When the transform type is different, the proposed method is disabled by scaling parameter. This is one of the encoder optimization.

## Proposal 2

Proposal 2 means that redundant syntax is reduced. ILCP is disabled for 4x4 TU because the case of different transform type occurs only in 4x4 TU. Small parsing dependency is required.

## Proposal 3

Proposal 3 means that different transform type is resolved by a further change of decoding process. DST is utilized for both luma and chroma 4x4 TU. No syntax is changed to JCTVC-N0266.

# Test results

The method implemented on top of HM12.0_RExt4.1 [2] and RCE1 Experiment 2 [3]. The experiments are performed using the common test conditions defined for RCE1 [4].

Table 1, 2, and 3 are the summary results of the proposed methods.

Table 1 Summary results by the proposal 1 compared with HEVC Range Extensions 4.1 anchor

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -16.5% | -15.3% | -15.9% | -12.6% | -12.5% | -12.4% | -9.6% | -9.7% | -9.4% |
| YCbCr 4:4:4 | -1.2% | -5.8% | -5.6% | -1.4% | -3.6% | -5.0% | -1.5% | -2.6% | -3.5% |
| Overall | -9.4% | -10.9% | -11.1% | -7.4% | -8.3% | -9.0% | -5.8% | -6.4% | -6.7% |
|  | -9.4% | -10.9% | -11.2% | -7.4% | -8.4% | -9.1% | -5.8% | -6.5% | -6.4% |
| Enc Time[%] | 118% |  |  | 119% |  |  | 120% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  | 98% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -13.0% | -11.0% | -12.5% | -10.2% | -8.1% | -9.6% |  |  |  |
| YCbCr 4:4:4 | -0.4% | -7.4% | -6.2% | -0.8% | -5.3% | -6.5% |  |  |  |
| Overall | -7.1% | -9.3% | -9.6% | -5.8% | -6.8% | -8.2% |  |  |  |
|  | -7.1% | -9.6% | -9.4% | -5.9% | -8.6% | -8.4% |  |  |  |
| Enc Time[%] | 112% |  |  | 115% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -12.3% | -8.5% | -9.8% | -9.5% | -6.3% | -7.5% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -5.5% | -5.4% | -0.6% | -3.7% | -5.9% |  |  |  |
| Overall | -6.7% | -7.1% | -7.7% | -5.3% | -5.1% | -6.8% |  |  |  |
|  | -6.6% | -6.4% | -7.4% | -5.5% | -5.8% | -6.7% |  |  |  |
| Enc Time[%] | 112% |  |  | 115% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |  |  |  |

Table 2 Summary results by the proposal 2 compared with HEVC Range Extensions 4.1 anchor

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -16.7% | -15.5% | -16.0% | -12.8% | -12.6% | -12.5% | -9.7% | -9.8% | -9.5% |
| YCbCr 4:4:4 | -1.4% | -6.1% | -5.8% | -1.6% | -3.8% | -5.3% | -1.7% | -2.7% | -3.7% |
| Overall | -9.6% | -11.1% | -11.3% | -7.6% | -8.5% | -9.1% | -6.0% | -6.5% | -6.8% |
|  | -9.6% | -11.1% | -11.4% | -7.6% | -8.6% | -9.3% | -5.9% | -6.6% | -6.5% |
| Enc Time[%] | 118% |  |  | 118% |  |  | 119% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 98% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -13.1% | -11.0% | -12.6% | -10.3% | -8.1% | -9.7% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -7.7% | -6.4% | -0.8% | -5.5% | -6.7% |  |  |  |
| Overall | -7.2% | -9.5% | -9.7% | -5.9% | -6.9% | -8.3% |  |  |  |
|  | -7.2% | -9.7% | -9.6% | -6.0% | -8.9% | -8.5% |  |  |  |
| Enc Time[%] | 113% |  |  | 116% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -12.4% | -8.5% | -9.8% | -9.6% | -6.3% | -7.5% |  |  |  |
| YCbCr 4:4:4 | -0.1% | -5.6% | -5.4% | -0.5% | -3.8% | -5.9% |  |  |  |
| Overall | -6.7% | -7.1% | -7.8% | -5.3% | -5.1% | -6.8% |  |  |  |
|  | -6.6% | -6.2% | -7.4% | -5.5% | -5.8% | -6.7% |  |  |  |
| Enc Time[%] | 112% |  |  | 116% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |  |  |  |

Table 3 Summary results by the proposal 3 compared with HEVC Range Extensions 4.1 anchor

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -17.9% | -16.4% | -17.0% | -14.1% | -13.4% | -13.6% | -10.8% | -10.4% | -10.4% |
| YCbCr 4:4:4 | -1.4% | -6.3% | -6.3% | -1.8% | -4.2% | -5.8% | -2.0% | -3.2% | -4.1% |
| Overall | -10.2% | -11.6% | -12.0% | -8.3% | -9.1% | -9.9% | -6.7% | -7.0% | -7.5% |
|  | -10.2% | -11.7% | -12.2% | -8.4% | -9.2% | -10.1% | -6.7% | -7.1% | -7.2% |
| Enc Time[%] | 113% |  |  | 112% |  |  | 112% |  |  |
| Dec Time[%] | 95% |  |  | 95% |  |  | 95% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -13.9% | -11.4% | -13.1% | -10.9% | -8.4% | -10.1% |  |  |  |
| YCbCr 4:4:4 | -0.5% | -7.7% | -6.7% | -0.8% | -5.7% | -6.9% |  |  |  |
| Overall | -7.6% | -9.7% | -10.1% | -6.2% | -7.1% | -8.6% |  |  |  |
|  | -7.6% | -10.1% | -10.0% | -6.3% | -9.2% | -8.9% |  |  |  |
| Enc Time[%] | 109% |  |  | 111% |  |  |  |  |  |
| Dec Time[%] | 91% |  |  | 92% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -12.6% | -8.6% | -10.0% | -9.7% | -6.4% | -7.6% |  |  |  |
| YCbCr 4:4:4 | -0.1% | -5.5% | -5.4% | -0.6% | -3.8% | -5.9% |  |  |  |
| Overall | -6.8% | -7.1% | -7.8% | -5.4% | -5.2% | -6.8% |  |  |  |
|  | -6.7% | -6.1% | -7.5% | -5.6% | -5.9% | -6.8% |  |  |  |
| Enc Time[%] | 108% |  |  | 111% |  |  |  |  |  |
| Dec Time[%] | 91% |  |  | 92% |  |  |  |  |  |

Table 4 is the summary results of the proposed method compared with RCE1 Exp2.

Table 4 Summary Results by the proposal 1 compared with RCE1 Exp2

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 2.3% | 1.7% | 1.9% | 2.2% | 1.4% | 1.9% | 1.6% | 0.9% | 1.4% |
| YCbCr 4:4:4 | 0.2% | 0.9% | 1.5% | 0.3% | 0.8% | 1.4% | 0.4% | 0.6% | 1.0% |
| Overall | 1.3% | 1.3% | 1.7% | 1.3% | 1.1% | 1.6% | 1.0% | 0.8% | 1.2% |
|  | 1.3% | 1.3% | 1.8% | 1.3% | 1.1% | 1.7% | 1.1% | 0.7% | 1.2% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 1.4% | 0.9% | 1.3% | 1.1% | 0.6% | 1.1% |  |  |  |
| YCbCr 4:4:4 | 0.1% | 0.9% | 1.1% | 0.1% | 0.8% | 1.0% |  |  |  |
| Overall | 0.8% | 0.9% | 1.2% | 0.6% | 0.7% | 1.1% |  |  |  |
|  | 0.8% | 1.3% | 1.2% | 0.7% | 1.0% | 1.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | 0.4% | 0.3% | 0.4% | 0.3% | 0.2% | 0.5% |  |  |  |
| YCbCr 4:4:4 | 0.0% | 0.4% | 0.0% | 0.0% | 0.2% | 0.1% |  |  |  |
| Overall | 0.2% | 0.3% | 0.2% | 0.2% | 0.2% | 0.3% |  |  |  |
|  | 0.2% | 0.1% | 0.3% | 0.2% | 0.3% | 0.3% |  |  |  |
| Enc Time[%] | 100% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |  |  |  |

# Conclusion

This contribution proposes the inter colour component residual coefficient prediction. Compared to HEVC Range Extension 4.1, significant BD-rate gain for both RGB and YUV sequences are observed. Compared to RCE1 Experiment 2, less dependency in decoder pipeline is provided by moving forward operation point, while the BD-rate losses are very small.

# References

- W. Pu, W.-S. Kim, J. Chen, K. Rapaka, L. Guo, J. Sole, and M. Karczewicz, “Non RCE1: Inter color component residual prediction,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0266, August 2013.

- D. Flynn, K. Sharman, and C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1006, April 2013.

- W. Pu, W.-S. Kim, J. Sole, and M. Karczewicz, “RCE1: Descriptions and Results for Experiments 1, 2, 3, and 4,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-O0202, November 2013.

- W.-S. Kim and T. Nguyen, “HEVC Range Extensions Core Experiment 1 (RCE1): Inter-component decorrelation methods,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1121, August 2013.

# Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).