| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0264 |
| --- | --- |

| Title: | RCE1: cross-check results of test 2 in inter color component residual prediction (JCTVC-O0202) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15, Ohara, Fujimino-shi, Saitama, JAPAN | Tel: Email: | +81 49 278 7411 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI R&D Laboratories, Inc.) |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results of RCE1 test 2, which is described in JCTVC-O0202 for inter color-component residual prediction.

# Introduction

An adaptive inter color component residual prediction scheme is proposed in JCTVC-N0266. The scaling parameter  is calculated at the encoder side and signaled. This prediction is performed both for intra- and inter-coded blocks. For each TU, if the luma component’s cbf is zero,  is not signaled and no prediction is performed. Otherwise,  is signaled for each chroma component, respectively.

In the experiment 2 of RCE1, at the encoder side, when calculating the , the luma residual before scaling and transformation is used rather than the reconstructed luma residual signals.

This contribution reports the cross-check results of this adaptive inter color component residual prediction scheme.

The software version was HM 12.0-RExt-4.1. The reference was generated by using the common test condition of range extensions used in RCE1 [3]. Provided software was compiled on 64-bit Windows 7 environment. The experiments were performed on the cluster PCs.

Error: Reference source not found, 2, and 3 show the cross-check results.

Table 1 Cross-check results of RCE1 Test 2 under AI condition

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -18.1% | -16.6% | -17.3% | -14.2% | -13.5% | -13.8% | -10.8% | -10.4% | -10.5% |
| YCbCr 4:4:4 | -1.4% | -6.6% | -6.9% | -1.8% | -4.4% | -6.2% | -1.9% | -3.2% | -4.4% |
| Overall | -10.3% | -11.9% | -12.4% | -8.4% | -9.3% | -10.3% | -6.7% | -7.1% | -7.7% |
|  | -10.3% | -12.0% | -12.5% | -8.4% | -9.4% | -10.4% | -6.7% | -7.2% | -7.3% |
| Enc Time[%] | 119% |  |  | 121% |  |  | 121% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |

Table 2 Cross-check results of RCE1 Test 2 under RA condition

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -14.1% | -11.7% | -13.5% | -11.1% | -8.6% | -10.5% |
| YCbCr 4:4:4 | -0.5% | -8.2% | -7.2% | -0.9% | -6.0% | -7.5% |
| Overall | -7.8% | -10.1% | -10.6% | -6.3% | -7.4% | -9.1% |
|  | -7.7% | -10.7% | -10.4% | -6.5% | -9.5% | -9.4% |
| Enc Time[%] | 113% |  |  | 116% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3 Cross-check results of RCE1 Test 2 under LDB condition

|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -12.6% | -8.7% | -10.1% | -9.8% | -6.5% | -7.9% |
| YCbCr 4:4:4 | -0.2% | -5.8% | -5.4% | -0.6% | -3.9% | -6.0% |
| Overall | -6.8% | -7.3% | -7.9% | -5.5% | -5.3% | -7.0% |
|  | -6.7% | -6.5% | -7.6% | -5.6% | -6.0% | -7.0% |
| Enc Time[%] | 112% |  |  | 116% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

We had verified results reported in JCTVC-O0202. The results of the cross-check matched with proponent’s report. The encoding / decoding runtime also matched closely to those reported by the proponent.

# References

- W. Pu, W.-S. Kim, J. Chen, K. Rapaka, L. Guo, J. Sole, and M. Karczewicz, “Non RCE1: Inter color component residual prediction,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0266, August 2013.

- W.-S. Kim and T. Nguyen, “HEVC Range Extensions Core Experiment 1 (RCE1): Inter-component decorrelation methods,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1121, August 2013.

- D. Flynn, K. Sharman, and C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1006, April 2013.