| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 October–1 Nov 2013 | Document: JCTVC-O0267 MPEG document: m31059 |
| --- | --- |

| Title: | Proposed updates to HEVC verification test plan draft 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | TK Tan | Tel: Email: | +65 9799 3861 tktan@m-sph.com |
| Source: |  |  |  |

_____________________________

# Abstract

This document contains proposed updates to the JCTVC-N1011 document.  Test sequences and encoding bitrates for use in the verification test plan are proposed for discussions and consideration at the JCTVC meeting in Geneva.

# Introduction

This document contains the draft plan for the video verification test to be conducted to verify the coding performance of the HEVC Main and Main 10 profiles. A formal subjective evaluation will be conducted comparing the HEVC Main and Main 10 profiles to the AVC High and High 10 profiles, respectively. A range of video resolutions from 480p to 4K will be tested.

# Schedule

Test Sequences Identified: T

Encoding of sequences completed: T+3 weeks

Subjective evaluation starts: T+4 weeks

Subjective evaluation completed: T+6 weeks

Subjective evaluation results available: T+8 weeks.

# Test conditions

The following test conditions will be used for the HEVC verification test.

- Number of sequences and video resolutions:

  - 4 for 1080p and 4K

  - 5 for 480p and 720p

- Bitsreams

  - Generated with HM 11.0 for HEVC bitstreams

  - Generated with JM 18.5 for AVC bitstreams

  - In addition to a. and b., other HEVC and/or AVC bitstreams generated with encoders that are optimized for subjective quality may be tested if available.

- Encoding parameters

  - Fixed QP.

    - 4 bit rate points per sequences covering the whole MOS range as much as possible

  - Bit depth

    - 8 bits for 480p, 720p and 1080p

    - 8 and 10 bits for 4K

  - Coding structure depending on the nature of the sequence.

    - Random access, RA (Storage/Streaming)

      - Intra refresh at approximately 1 second intervals.

      - Picture reordering allowed.

    - Low delay, LD (Video conferencing)

      - No Intra refresh.

      - Without picture reordering.

  - Other settings as in the configuration files

    - cfg/encoder_randomaccess_main.cfg, encoder_randomaccess_main10.cfg or encoder_lowdelay_main.cfg for HM

    - bin/HM-like/encoder_JM_RA_B_HE.cfg or bin/HM-like/encoder_JM_LB_HE.cfg configurations for JM18.5

# Test Sequences

The following test sequences are under consideration for the subjective test.

Table 1: Selected test sequences and properties

| Sequence | Source | Width x Height | Frame rate | Bit depth | Length (frames) | RA / LD | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BT709Birthday | Technicolor | 3840x2160 | 50 | 10 | 500 | RA | Encoded and played as 50 fps |
|  |  | 3840x2160 |  | 10 |  |  |  |
| digestL07 | NICT | 3840x2160 | 24 | 8 | 240 | RA |  |
| traffic | Plannet, Inc | 4096x2048 | 30 | 8 | 300 | RA | ? |
| JohnnyLobby | Vidyo | 1920x1080 | 60 | 8 | 600 | LD |  |
| Railway2 | NTIA | 1920x1080 | 30 | 8 | 300 | RA |  |
| SVT15 | SVT | 1920x1080 | 50 | 8 | 500 | RA |  |
| ScoreNarrow | NTIA | 1920x1080 | 30 | 8 | 300 | RA |  |
| UnderBoat1 | NTIA | 1920x1080 | 30 | 8 | 300 | RA |  |
| ThreePeople | Vidyo | 1280x720 | 60 | 8 | 600 | LD |  |
|  |  | 1280x720 |  | 8 |  | RA |  |
| BT709Parakeets | Technicolor | 1280x720 | 50 | 8 | 500 | RA |  |
| SVT01a | SVT | 1280x720 | 50 | 8 | 500 | RA |  |
| SVT04a | SVT | 1280x720 | 50 | 8 | 500 | RA |  |
| Anemone | NTIA | 832x480 | 30 | 8 | 300 | RA |  |
| BT709BirthdayFlash | Technicolor | 832x480 | 50 | 8 | 500 | RA |  |
|  |  | 832x480 |  | 8 |  |  |  |
|  |  | 832x480 |  | 8 |  |  |  |
|  |  | 832x480 |  | 8 |  |  |  |

Note: It is desirable that at least one sequence with film grain and other types of noise (e.g. compression noise after a sequences goes through MPEG-2 and/or AVC compression) that is present in some real world sequences.

# Encoding Results

The following table shows the JM18.5 and HM11.0 encoding results on the sequences shown in Table 1. The QP parameters were selected such that the bitrate of the HM11.0 bitstreams are approximately half of the bitrate of the corresponding JM18.5 bitstreams.  The range of the QP values were also selected so that the subjective quality of the encoded sequence span as large a range of the MOS range as possible.

Table 2: JM18.5 and HM11.0 encoding results

|  |  | JM18.5 |  | HM11.0 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | QPISlice | kbps (a) | QPISlice | kbps (b) | Bitrate Difference (a - b)/a |
| 4K | BT709Birthday |  |  |  |  |  |
|  |  | 0 | 0 | 0 | 0 | #DIV/0! |
|  |  | 0 | 0 | 0 | 0 | #DIV/0! |
|  |  | 0 | 0 | 0 | 0 | #DIV/0! |
|  |  | 0 | 0 | 0 | 0 | #DIV/0! |
|  | digestL07 | 27 | 10266 | 32 | 4929 | 52% |
|  |  | 32 | 6206 | 36 | 3190 | 49% |
|  |  | 36 | 4353 | 40 | 2138 | 51% |
|  |  | 41 | 2832 | 44 | 1277 | 55% |
|  | traffic | 27 | 13309 | 31 | 6206 | 53% |
|  |  | 32 | 6583 | 36 | 3136 | 52% |
|  |  | 37 | 3618 | 40 | 1844 | 49% |
|  |  | 42 | 2090 | 44 | 1057 | 49% |
| 1080p | JohnnyLobby | 23 | 2761 | 24 | 1477 | 46% |
|  | (low delay) | 27 | 895 | 28 | 444 | 50% |
|  |  | 31 | 468 | 32 | 227 | 51% |
|  |  | 35 | 298 | 36 | 138 | 54% |
|  | Railway2 | 28 | 4421 | 32 | 2056 | 53% |
|  |  | 32 | 2419 | 36 | 1102 | 54% |
|  |  | 36 | 1408 | 39 | 686 | 51% |
|  |  | 39 | 948 | 42 | 422 | 56% |
|  | SVT15 | 28 | 6805 | 31 | 3547 | 48% |
|  |  | 32 | 3767 | 35 | 1899 | 50% |
|  |  | 36 | 2214 | 39 | 1027 | 54% |
|  |  | 41 | 1109 | 43 | 546 | 51% |
|  | ScoreNarrow | 25 | 2918 | 28 | 1348 | 54% |
|  |  | 29 | 1861 | 31 | 911 | 51% |
|  |  | 33 | 1242 | 34 | 627 | 50% |
|  |  | 37 | 897 | 37 | 443 | 51% |
|  | UnderBoat1 | 24 | 4407 | 27 | 1909 | 57% |
|  |  | 29 | 2026 | 31 | 991 | 51% |
|  |  | 33 | 1196 | 35 | 553 | 54% |
|  |  | 37 | 729 | 39 | 325 | 55% |
| 720p | ThreePeople | 25 | 1414 | 28 | 647 | 54% |
|  | (low delay) | 29 | 739 | 32 | 345 | 53% |
|  |  | 33 | 433 | 36 | 200 | 54% |
|  |  | 38 | 240 | 40 | 117 | 51% |
|  | BT709Parakeets | 26 | 1151 | 30 | 552 | 52% |
|  |  | 30 | 709 | 34 | 333 | 53% |
|  |  | 33 | 499 | 37 | 231 | 54% |
|  |  | 37 | 332 | 40 | 161 | 52% |
|  | SVT01a | 27 | 1271 | 31 | 594 | 53% |
|  |  | 31 | 733 | 35 | 336 | 54% |
|  |  | 35 | 435 | 38 | 215 | 51% |
|  |  | 39 | 283 | 41 | 132 | 53% |
|  | SVT04a | 28 | 4178 | 32 | 2153 | 48% |
|  |  | 31 | 2665 | 35 | 1362 | 49% |
|  |  | 34 | 1699 | 38 | 849 | 50% |
|  |  | 37 | 1072 | 41 | 504 | 53% |
| 480p | Anemone | 25 | 990 | 29 | 478 | 52% |
|  |  | 29 | 581 | 33 | 271 | 53% |
|  |  | 33 | 353 | 37 | 164 | 54% |
|  |  | 38 | 202 | 41 | 99 | 51% |
|  | BT709BirthdayFlash | 29 | 1515 | 33 | 773 | 49% |
|  |  | 32 | 1003 | 37 | 498 | 50% |
|  |  | 35 | 679 | 41 | 314 | 54% |
|  |  | 39 | 399 | 44 | 205 | 49% |

### Annex A

### Description of testing environment and methodology

The test procedure foreseen for the formal subjective evaluation will consider two main requirements:

- to be as much as possible reliable and effective in verifying the performance in terms of subjective quality (and therefore adhering the existing recommendations);

- to take into account the evolution of technology and laboratory set-up oriented to the adoption of FPD (Flat Panel Display) and video server as video recording and playing equipment.

Therefore, one of the test methods described in [1] are planned to be used, applying some modification to them, in relation to the kind of display, the video recording and play-back equipment.

## A.1 Test method

The test method adopted for this evaluation is DCR (Degradation Category Rating) [1].

## A.1.1 DSIS

This test method is commonly adopted when the material to be evaluated shows a range of visual quality that well distributes across all quality scales.

This method will be used under the schema of evaluation of the quality (and not of the impairment); for this reason a quality rating scale made of 11 levels will be adopted, ranging from "0" (lowest quality) to "10" (highest quality). The test will be held in three different laboratories located in countries speaking different languages: This implies that it is better not to use categorical adjectives (e.g. excellent good fair etc.) to avoid any bias due to a possible different interpretation by naive subjects speaking different languages.

All the video material used for these tests will consist of video clips of 10 seconds duration.

The structure of the Basic Test Cell (BTC) of DSIS method is made by two consecutive presentations of the video clip under test; at first the original version of the video clip is displayed, immediately afterwards the coded version of the video clip is presented; then a message displays for 5 seconds asking the viewers to vote (see Figure 1)

Figure 1 - DSIS  BTC

The presentation of the video clips will be preceded by a mid-grey screen displaying for one second.

## A.2 How to express the visual quality opinion with DSIS

The viewers will be asked to express their vote putting a mark on a scoring sheet.

The scoring sheet for a DSIS test is made of a section for each BTC; each section is made of a column of 11 vertically arranged boxes, associated to a number from 0 to 10 (see Figure 2).

The viewers have to put a check mark on one of the 11 boxes; checking the box "10" the subject will express an opinion of "best" quality, while checking the box "0" the subject will express an opinion of the "worst” quality.

The vote has to be written when the message "Vote N" appears on the screen. The number "N" is a numerical progressive indication on the screen aiming to help the viewing subjects to use the appropriate column of the scoring sheet.

Figure 2 -Example of DSIS test method scoring sheet

## A.4 Training and stabilization phase

The outcome of a test is highly dependent on a proper training of the test subjects.

For this purpose, each subject has to be trained by means of a short practice (training) session.

The video material used for the training session must be different from those of the test, but the impairments introduced by the coding have to be as much as possible similar to those in the test.

The stabilization phase uses the test material of a test session; three BTCs, containing one sample of best quality, one of the worst quality and one of medium quality, are duplicated at the beginning of the test session. By this way, the test subjects have an immediate impression of the quality range they are expected to evaluate during that session.

The scores of the stabilization phase are discarded. Consistency of the behaviour of the subjects will be checked inserting in the session a BTC in which original is compared to original.

## A.5 The laboratory set-up

The laboratory for a subjective assessment is planned to be set up following [1], except for the selection of the display and the video play-out server.

For 4K video clips,

<to be completed>

For other resolutions, High quality LCD monitors (or TV set) are used, having a diagonal size equal or higher of 40” and capable to accept resolution equal to 1920 x 1080. When using TV sets all the local colour and contrast features must be disabled (where applicable).

Play-out of 1080p, 720p and 480p video clips is done at the native resolution using the central area of the screen; the remaining part of the screen is set to a mid grey level (128 in 0-255 range).

The video play server, or the PC, used to play video has to be able to support the display of 4K, 1080p, 720p and 480p video formats, at 24, 30, 50 and 60 frames per second, without any limitation, or without introducing any additional temporal or visual degradation.

## A.5.1 Viewing distance

The viewing distance varies according to the physical dimensions of the active part of the video; this will lead to a viewing distance varying from 2H to 4H, where H is equal to the height of the active part of the screen.

The number of subjects seating in front of the monitor is a function of the monitor size and of the selected viewing distance.

## A.5.2 	Viewing environment.

The test laboratory has to be carefully protected from any external visual or audio pollution.

Internal general light has to be low (just enough to allow the viewing subjects to fill out the scoring sheets) and a uniform light has to be placed behind the monitor; this light must have an intensity as specified in Recommendation ITU-R BT.500. No light source has to be directed to the screen or create reflections; ceiling, floor and walls of the laboratory have to be made of non-reflecting material (e.g. carpet or velvet) and should have a colour tuned as close as possible to D65.

## A.6 	Example of a test schedule for a day.

This is an example of the schedule of the planned test activity for one day.

A time slot of one hour is dedicated every morning and afternoon to welcome, to screen and to train the viewers.

After the subjects’ screening for visual acuity and colour blindness, the subjects will be grouped in testing groups.

In the following example, four groups of subjects are created, according to the laboratory set-up and to the time constraints.

|  | DAY 1 |  | DAY 2 |  |
| --- | --- | --- | --- | --- |
| 9:00 – 10:00 | Screening / training |  | Screening / training |  |
| 10:00 -10:40 | G1-S1 |  | G1-S1 |  |
| 10:40 -11:20 |  | G2-S1 |  | G2-S1 |
| 11:20 -12:00 | G1-S2 |  | G1-S2 |  |
| 12:00 -12:40 |  | G2-S2 |  | G2-S2 |
| 13:00 – 14:00 | Screening / training |  | Screening / training |  |
| 14:00 -14:40 | G3-S3 |  | G3-S3 |  |
| 14:40 -15:20 |  | G4-S3 |  | G4-S3 |
| 15:20 -16:00 | G3-S4 |  | G3-S4 |  |
| 16:00 -16:40 |  | G4-S4 |  | G4-S4 |

## A.7 Overall test effort and subjects’ involvement

The duration of the test will depend on the number of submissions; in any case the each viewing session will not stand for more than 25 minutes and the same viewing subject will not participated to the test run for more than six hours in total. The same subject may not be enrolled for two consecutive days. Young humans subjects, equally distributed in gender, are hired, selecting them for an age from 18 to 30 and, highly preferably among University students of scientific faculties. Viewing subjects are compensated for their participation to the testing activities (compensation may be done in money or services).

## A.8 Statistical analysis and presentation of the results

The data collected from the score sheets, filled out by the viewing subjects, will be stored in an Excel spreadsheet.

Five spread-sheets will be prepared: four containing the results for 4K, 1080p, 720p and 480p (Main profile) and one for 4K (Main 10 profile).

For each coding condition the Mean Opinion Score (MOS) and associated Confidence Interval (CI) values will be given in the spreadsheets.

The MOS and CI values will be used to draw graphs. The Graphs will be drawn grouping the results for each video test sequence. No graph grouping results from different video sequences will be considered.

References:

- International Telecommunication Union Standardization Sector; Recommendation ITU-T P.910

The viewing rooms of the three laboratories could be different according to the test material and/or to the design of the laboratory. Large displays (e.g. monitor equal or wider than 50’’) will allow to seat three (or more) subjects at the same time; a laboratory set-up, in which three wide monitors are available, will allow the creation of wider groups of viewers (three or more).