| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0269 |
| --- | --- |

| Title: | AhG8/AhG22: Using qp=0, 4, 8, 12 for visually lossless coding experiments and results from SC coding by packed pixel Pseudo-2D-matching integrated with HM12.0RE4.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tao Lin, Xianyi Chen, Peijun Zhang, Liping Zhao | Tel: Email: | 86-21-5103-0961 lintao@tongji.edu.cn |
| Source: | Tongji University |  |  |

_____________________________

# Abstract

This document proposes to use qp=0, 4, 8, 12 in visually lossless screen content coding experiments. This document also reports the coding results of 30 screen content test sequences by both HM12RE4.0 and HM12RE4.0+packed pixel Pseudo-2D-matching technique of JCTVC-L0303. The coding results report that the PSNR using qp=0, 4, 8, 12 is between 50dB to 999.99dB.

# Introduction

Currently, JCTVC has lossless coding experiments which bypass quantization and lossy experiments which use qp=12, 17, 22, 27, 32, 37. Neither is originally designed for visually lossless coding experiments. For visually lossless coding experiments, a different set of qp needs to be selected to achieve coding quality between traditional lossless coding experiments and lossy coding experiments. This document proposes to use qp=0, 4, 8, 12 in visually lossless screen content coding experiments. Using the proposed set of qp, this document also reports the coding results of 30 screen content test sequences using both HM12RE4.0 and packed pixel Pseudo-2D-matching technique integrated with HM12RE4.0.

# Proposed Method

In visually lossless coding experiments, to achieve coding quality between traditional lossless coding experiments and lossy coding experiments, the most natural and reasonable way is probably to select qp between 0 and the qp used in traditional lossy coding experiments, that is, to select qp=0, 4, 8, 12.

To understand the consequences of selecting qp=0, 4, 8, 12, we encode 30 screen content test sequences using both HM12RE4.0 and a modified version of HM12RE4.0 that integrates the packed pixel Pseudo-2D-matching technique of JCTVC-L0303.

# Experimental Results

See ppt and xls files.

# Conclusion

The proposed test method can achieve experimental coding quality between traditional lossless coding experiments and lossy coding experiments. We propose to adopt the set of qp for visually lossless coding experiments.

# Patent rights declaration(s)

Tongji University does not have any current or pending patent rights relating to the test method described in this contribution.