| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0271r1 |
| --- | --- |

| Title: | SHVC: On direct_dependency_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

In SHVC specification, layer sets can be defined by the syntax element “layer_id_included_flag[i][j]” in VPS. Direct dependencies between layers can be defined by “direct_dependency_flag[i][j]” in VPS_extension(), and indirect dependencies between layers can be inferred from the values of this syntax element. If there exists direct or indirect dependency between layerId A and B (A > B), it means that layer A cannot be decoded unless layer B is decoded. Then if a certain layer set contains layer A, it should also contain layer B. However there is no constraint in the semantics of direct_dependency_flag[i][j].

This contribution proposes to add constraint on the semantics of direct_dependency_flag[i][j] associated with layer sets defined by layer_id_included_flag[i][j].

# Introduction / Problem Statement / …

In SHVC specification, layer sets can be defined by the syntax element “layer_id_included_flag[i][j]” in VPS as shown in Table 1.1.

Table 1.1

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| vps_max_layer_id | u(6) |
| vps_num_layer_sets_minus1 | ue(v) |
| for( i = 1; i <= vps_num_layer_sets_minus1; i++ ) |  |
| for( j = 0; j <= vps_max_layer_id; j++ ) |  |
| layer_id_included_flag[ i ][ j ] | u(1) |
| ... |  |
| } |  |

Direct dependencies between layers can be defined by by “direct_dependency_flag[i][j]” in VPS_extension(). With this syntax element, dependency[i][j], which is direct or indirect dependency between layerId i and j, can be inferred.

Table 1.2

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| … |  |
| } |  |

If there exists dependency between layerId A and B (A > B), it is necessary to decode layer B for decoding of layer A. Therefore if a certain layer set include layer A, it should also contain layer B.

However there is no constraint in the semantics of direct_dependency_flag under the current SHVC WD.

# Proposal

To solve the problem stated above, it is proposed to add constraint in the semantics of direct_dependency_flag[i][j] as follows:

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with index j may be a direct reference layer for the layer with index i. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

…

It is a requirement of bitstream conformance that if the value of dependency[CurrLayerId][RefLayerId] is equal to 1 and a layer set defined by layer_id_included_flag[i][j] contains the current layer with CurrLayerId, it shall also contain the ref layer with RefLayerId inferred by the syntax element direct_dependency_flag[i][i].

# Conclusion

In SHVC specification, layer sets can be defined by the syntax element “layer_id_included_flag[i][j]” in VPS. Direct dependencies between layers can be defined by “direct_dependency_flag[i][j]” in VPS_extension(), and indirect dependencies between layers can be inferred from the values of this syntax element. There is no constraint between these two syntax elements.

If there exists direct or indirect dependency between layerId A and B (A > B), it means that layer A cannot be decoded unless layer B is decoded. Then if a certain layer set contains layer A, it should also contain layer B.

It is proposed by this contribution to add constraint on the semantics of direct_dependency_flag[i][j] associated with layer sets defined by layer_id_included_flag[i][j]. It is recommended that the suggested modification be adopted into SHVC WD.

# References

[1] “Scalable High Efficiency Video Coding Draft 3”, JCTVC-N1008, J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, July 2013

[2] “Editors' proposed corrections to HEVC version 1”, JCTVC-M0432, B. Bross, G. J. Sullivan, Y.-K. Wang, April 2013

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).