| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0272r1 |
| --- | --- |

| Title: | SHVC: Upsampling with shorter-tap filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato Sony City Osaki 2-10-1 Osaki Shinagawa Tokyo 141-8610 Japan | Tel: Email: | +81 50 3750 2741 Kazushi.Sato@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

Upsampling of the reconstructed reference layer picture is needed for spatial scalability. Under the current SHVC specification same filters as the ones for MC interpolation are applied also for upsampling both of luma and chroma components. At the 14th JCTVC meeting in Vienna, it is proposed by JCTVC-N0265 to apply shorter-tap filters for complexity reduction.

This document proposes to apply shorter-tap filters for upsampling with higher temporal layers. It is also proposed to add a syntax element that specifies from which temporal layer shorter-tap filter is applied for upsampling.

Simulation results show that by applying shorter-tap filter just for the 1st highest temporal layer loss in coding efficiency is observed by 0.0%, 0.0%, and 0.0% for Y,U and V component with the RA_2x case, and 0.0%, 0.0%, and 0.1% for Y,U and V component with the RA_1.5x case. By applying shorter-tap filter for the 1st and 2nd highest temporal layers loss in coding efficiency is observed by 0.1%, 0.0%, and 0.0% for Y,U and V component with the RA_2x case, and 0.2%, 0.1%, and 0.2% for Y,U and V component with the RA_1.5x case.

The proposed method provides a trade-off between coding efficiency and complexity to the encoder manufactures. It is recommended that the proposed method be adopted into SHVC WD.

# Introduction / Problem Statement / …

Upsampling of the reconstructed reference layer picture is needed for spatial scalability. Under the current SHVC specification same filter as the one for MC interpolation is applied also for upsampling. At the 14th JCTVC meeting in Vienna, it is proposed by JCTVC-N0265 to apply shorter-tap filters for complexity reduction.

HEVC supports temporal scalability with hierarchical B-picture structure. With pictures in lower temporal hierarchy time distance between the current and the reference pictures are long, and with single-layer coding more PUs are coded as intra rather than inter. With scalable coding more PUs are predicted from the reference layers, and in spatial scalability, image quality of the upsampled base-layer pictures do affect coding efficiency in the enhancement layers.

On the contrary with pictures in higher temporal hierarchy time distance between the current and the reference pictures are short, and more PUs are coded as inter with single-layer coding. With such pictures inter-layer prediction is not so helpful to improve coding efficiency as for pictures in lower temporal hierarchy.

# Proposed Method

To solve the problem stated above, it is proposed that shorter-tap filters for upsampling be applied for higher temporal layers. In the example shown in Fig 2.1, shorter-tap filters are applied for the highest temporal layer. It is also proposed to add a syntax element in vps_extension() that specifies from which temporal layer shorter-tap filter is applied for upsampling.

Fig 2.1

# Experiment

## Simulation Condition

SHM-3.0.1 is used as anchor. On top of the anchor software the proposed method has been implemented by the author.

As for the shorter-tap filters 4tap FIR used for chroma MC interpolation is applied for luma, and 2tap FIR (linear interpolation) is applied for chroma.

2 tests have been conducted as:

- Experiment -1: Shorter-tap upsampling filter is applied just for the 1st highest temporal layer

- Experiment -2: Shorter-tap upsampling filter is applied for the 1st and the 2nd highest temporal layers

Class A&B sequences are tested with RA_2x and RA_1.5x conditions.

## Simulation Result

Summary of the results of Experiment -1 and -2 are shown in Table 3.2.1 and 3.2.2 respectively. Complete results are shown in JCTVC-O0272_Exp1.xls and JCTVC-O0272_Exp2.xls respectively.

As can be seen by applying shorter-tap filter just for the 1st highest temporal layer loss in coding efficiency is observed by 0.0%, 0.0%, and 0.0% for Y,U and V component with the RA_2x case, and 0.0%, 0.0%, and 0.1% for Y,U and V component with the RA_1.5x case. By applying shorter-tap filter for the 1st and 2nd highest temporal layers loss in coding efficiency is observed by 0.1%, 0.0%, and 0.0% for Y,U and V component with the RA_2x case, and 0.2%, 0.1%, and 0.2% for Y,U and V component with the RA_1.5x case.

The author would like to thank Sharp for crosschecking [4].

Table 3.2.1

Table 3.2.2

# Proposed Change in WD text

| vps_extension( ) { | Descriptor |
| --- | --- |
| … | u(1) |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| for( i = 0; i < vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| all_ref_layers_active_flag | u(1) |
| for( i = 0; i < vps_max_layers_minus1; i++ ) |  |
| max_tid_il_upsampling_with_longer_filter_plus1[ i ] | u(3) |
| … | u(10) |
| } |  |

max_tid_il_upsampling_with_longer_filter_plus1 minus1 specifies the max tid where upsampling with longer filters are applied. Longer filters mean that 8/7-tap FIR for luma and 4-tap FIR for chroma specified for fractional pixel extraction process for inter prediction. Above the tid specified by this syntax element, 4-tap FIR originally for chroma fractional pixel extraction for inter prediction is applied for upsampling of luma, and linear interpolation is applied for upsampling of chroma. The value shall be smaller or equal to vps_max_sub_layers_minus1 plus1. When the value of max_tid_ref_present_flag is equal to 1, the value shall be smaller or equal to max_tid_il_ref_pics_plus1[ i ].

- Conclusion

This contribution proposes to add option that enables shorter-tap filters for up-sampling be applied for higher temporal layers. Simulation results show that by applying shorter-tap filter just for the 1st highest temporal layer loss in coding efficiency is observed by 0.0%, 0.0%, and 0.0% for Y,U and V component with the RA_2x case, and 0.0%, 0.0%, and 0.1% for Y,U and V component with the RA_1.5x case. By applying shorter-tap filter for the 1st and 2nd highest temporal layers loss in coding efficiency is observed by 0.1%, 0.0%, and 0.0% for Y,U and V component with the RA_2x case, and 0.2%, 0.1%, and 0.2% for Y,U and V component with the RA_1.5x case.

The proposed method provides a trade-off between coding efficiency and complexity to the encoder manufactures. It is recommended that the proposed method be adopted into SHVC WD.

# References

[1] “Scalable High Efficiency Video Coding Draft 3”, JCTVC-N1008, J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, July 2013

[2] “Common SHM test conditions and software reference configurations”, JCTVC-N1009, X. Li, J. Boyce, P. Onno, Y. Ye, July 2013

[3] “AHG13: Complexity reduction of the up-sampling process”, JCTVC-N0265, K. Andersson, R. Sjöberg (Ericsson), July 2013

[4] “Cross check of JCTVC-O0272”, JCTVC-O0287, JCTVC-O0287, T. Yamamoto (Sharp), October 2013

# Patent rights declaration(s)

Sony Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).