| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0274 |
| --- | --- |

| Title: | Cross-check report for JCTVC-O0215 on phase alignment of up-sampling process in SHVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Dong, Yan Ye 9710 Scranton Rd, Ste 250, San Diego, CA 92121, USA | Tel: Email: | +1 858.210.4809 jie.dong@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This document is the cross-check report on Qualcomm’s proposal JCTVC-O0215 on phase alignment of upsampling process in SHVC. For spatial scalability, JCTVC-O0215 proposes to add a flag in VPS to indicate whether the sampling grids of the base layer (BL) and enhancement layer (EL) videos have non-zero phase shift or not. According to the value of this flag, the upsampling process is adjusted such that the sampling grids of the EL video and the upsampled BL video are aligned and a relatively accurate inter-layer prediction can be achieved. The implementations and experimental results provided by the proponent are confirmed in this document.

# Introduction

JCTVC-O0215 proposes to add a flag in VPS to indicate whether the sampling grids of the base layer (BL) and enhancement layer (EL) videos have non-zero phase shift or not. The flag equal to 0 indicates that the sampling grids of the BL and EL videos have zero phase shift, as shown in Fig. 1(a); the flag equal to 1 indicates that the sampling grids of the BL and EL videos have non-zero phase shift, which is assumed to be central position aligned, as shown in Fig. 1(b). According to the value of this flag, the upsampling process is adjusted such that the sampling grids of the EL video and the upsampled BL video are aligned and a relatively accurate inter-layer prediction can be achieved.

(a) Zero position aligned                                               (b) Central position aligned

Fig. 1 Sample location of reference layer and enhancement layer

We checked the implementation of the algorithm and the R-D performance. We can confirm the implementation in software matches the description in the document, and the performance results match what are provided by the proponent.

# Experimental Results

The cross-check experiments were conducted on 64-bit Linux cluster under SHVC CTC, except that the BL and EL videos coded are central position aligned, i.e., the flag equal to 1. The BL sequences are generated from the CTC EL sequences by the downsampler in [1]. As shown below, the results are exactly the same as what are provided by the proponent. Note that we also provide the results on the configuration of low delay P (LDP), which is optional and not included in JCTVC-O0215. Table 2 in JCTVC-O0215 in not cross-checked.

Table 1 BD-rate of using JCTVC-O0215

|  | AI HEVC 2x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -13.8% | -15.9% | -16.5% |  |  |  |
| Class B | -7.6% | -8.8% | -8.7% | -8.3% | -10.2% | -10.2% |
| Overall (Test vs Ref) | -9.4% | -10.9% | -10.9% | -8.3% | -10.2% | -10.2% |
| Overall (Test vs single layer) | 13.2% | 15.1% | 14.7% | 11.0% | 9.9% | 9.5% |
| Overall (Ref vs single layer) | 25.1% | 29.3% | 28.9% | 21.2% | 22.5% | 22.0% |
| EL only (Test vs Ref) | -17.0% | -18.4% | -18.5% | -22.7% | -24.6% | -24.7% |
| Enc Time[%] | 100.6% |  |  | 101.2% |  |  |
| Dec Time[%] | 96.0% |  |  | 97.1% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -10.0% | -5.4% | -6.8% |  |  |  |
| Class B | -5.2% | -3.5% | -2.8% | -6.0% | -5.1% | -4.6% |
| Overall (Test vs Ref) | -6.6% | -4.1% | -3.9% | -6.0% | -5.1% | -4.6% |
| Overall (Test vs single layer) | 19.4% | 33.5% | 32.1% | 16.5% | 29.1% | 29.2% |
| Overall (Ref vs single layer) | 28.1% | 39.2% | 37.7% | 24.1% | 36.1% | 35.5% |
| EL only (Test vs Ref) | -12.1% | -9.6% | -9.4% | -15.9% | -14.9% | -14.4% |
| Enc Time[%] | 102.8% |  |  | 104.3% |  |  |
| Dec Time[%] | 103.4% |  |  | 104.2% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -7.5% | -2.7% | -3.8% |  |  |  |
| Class B | -3.8% | -2.7% | -2.1% | -5.9% | -4.9% | -4.4% |
| Overall (Test vs Ref) | -4.8% | -2.7% | -2.6% | -5.9% | -4.9% | -4.4% |
| Overall (Test vs single layer) | 28.8% | 39.3% | 40.3% | 25.1% | 33.0% | 35.6% |
| Overall (Ref vs single layer) | 35.6% | 43.2% | 44.2% | 33.0% | 39.9% | 41.9% |
| EL only (Test vs Ref) | -8.8% | -6.7% | -6.6% | -15.0% | -14.1% | -13.6% |
| Enc Time[%] | 103.6% |  |  | 104.7% |  |  |
| Dec Time[%] | 103.4% |  |  | 103.8% |  |  |
| BL Match | Matched |  |  | Matched |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -8.8% | -3.1% | -4.3% |  |  |  |
| Class B | -4.6% | -3.2% | -2.5% | -6.6% | -5.2% | -4.7% |
| Overall (Test vs Ref) | -5.8% | -3.2% | -3.1% | -6.6% | -5.2% | -4.7% |
| Overall (Test vs single layer) | 27.0% | 38.3% | 39.6% | 23.1% | 32.6% | 35.5% |
| Overall (Ref vs single layer) | 35.1% | 42.9% | 44.2% | 31.9% | 39.8% | 42.2% |
| EL only (Test vs Ref) | -10.1% | -7.6% | -7.5% | -16.4% | -15.0% | -14.5% |
| Enc Time[%] | 100.6% |  |  | 104.8% |  |  |
| Dec Time[%] | 103.7% |  |  | 103.3% |  |  |
| BL Match | Matched |  |  | Matched |  |  |

# Reference

- J. Dong, Y. He, Y. Ye, "Downsampling filter for anchor generation for scalable extensions of HEVC," m23485, 99th MPEG meeting, San Jose, USA, Feb. 1-10, 2012.