| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0281 |
| --- | --- |

| Title: | SCE1: Verification of Test 1.2 results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina 416, Maetan 3-dong, Yeongtong-gu, Suwon, Korea, 443-742 | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution verifies performance of re-sampling filter set proposed by Arris. Proposed re-sampling filters set changes filters which are already is specification text and so change SHVC performance under common test condition. There is BD-rate performance drop 0,4% in ×1,5 associated with this change. In ×2 test performance is also different from SHM3.0 (but average difference is 0,0%). There is no gain in terms of Luma BD-rate (0,0% in average) and small Chroma gain (0,2%) compare to default SHM3.0 filter set in the test which exploits all 16 filters needed for arbitrary spatial scalability support.

# Introduction

SHVC specification draft text mentions re-sampling filters for 16 fractional positions (both for Luma and Chroma). But only re-sampling filters needed for spatial ratios ×2 and ×1,5 support are defined in specification draft text.

For arbitrary spatial ratio support missed filters need to be defined. Taking into account mirror symmetry of interpolation filter coefficients for phases α and 1-α it total 8 Luma and 8 Chroma filters need to be defined. 3 Luma and 7 Chroma filters are already specified.

Verified contribution

- adds missed filters needed for arbitrary spatial ratio support;

- modifies existing in draft text filters (and so changes HEVC performance under ctc).

Both Luma and Chroma re-sampling filters sets are modified.

# The summary of test results

Tests were performed according to SCE1 description. In both 3 tests (for ratios ×2, ×1.5 and specific ration ~×1.75, which exploits all 16 phases) there is performance difference compare to the anchor.. All intra, random access, low-delay-B and low-delay-P tests were performed. Results we obtained perfectly match with proponent’s data. Performance is summarized in the Table 1 below.

Table 1. Performance test results summary (SHVC down-sampler is used).

|  | AI HEVC 2x |  |  | AI HEVC ~1.75x |  |  | AI HEVC 1.5x |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | -0,1% | -0,3% | -0,3% |  |  |  |
| Class B | 0,0% | 0,0% | 0,0% | -0,1% | -0,2% | -0,2% | 0,5% | 0,3% | 0,3% |
| Overall (Test vs Ref) | 0,0% | 0,0% | 0,0% | -0,1% | -0,2% | -0,2% | 0,5% | 0,3% | 0,3% |
| Overall (Test vs single layer) | 12,8% | 14,8% | 14,5% | 13,9% | 15,0% | 14,5% | 11,0% | 10,1% | 9,6% |
| Overall (Ref vs single layer) | 12,8% | 14,9% | 14,6% | 14,0% | 15,3% | 14,8% | 10,5% | 9,8% | 9,3% |
| EL only (Test vs Ref) | 0,0% | 0,0% | 0,0% | -0,2% | -0,4% | -0,4% | 1,3% | 1,1% | 1,1% |
| Enc Time[%] | 100,0% |  |  | 97,6% |  |  | 100,1% |  |  |
| Dec Time[%] | 96,9% |  |  | 94,1% |  |  | 96,0% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | RA HEVC 2x |  |  | RA HEVC ~1.75x |  |  | RA HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,1% | 0,0% | 0,0% | -0,3% | -0,2% |  |  |  |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,1% | 0,4% | 0,1% | 0,1% |
| Overall (Test vs Ref) | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,1% | 0,4% | 0,1% | 0,1% |
| Overall (Test vs single layer) | 19,0% | 33,1% | 31,8% | 19,5% | 33,1% | 31,8% | 16,6% | 29,0% | 29,3% |
| Overall (Ref vs single layer) | 19,0% | 33,1% | 31,8% | 19,5% | 33,2% | 32,1% | 16,2% | 28,9% | 29,2% |
| EL only (Test vs Ref) | 0,0% | 0,0% | 0,0% | -0,1% | -0,2% | -0,2% | 0,8% | 0,6% | 0,5% |
| Enc Time[%] | 98,1% |  |  | 96,7% |  |  | 98,2% |  |  |
| Dec Time[%] | 94,1% |  |  | 92,1% |  |  | 93,4% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
|  | LD-B HEVC 2x |  |  | LD-B HEVC ~1.75x |  |  | LD-B HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,2% |  |  |  |
| Class B | 0,0% | 0,0% | -0,1% | 0,0% | -0,1% | -0,2% | 0,3% | 0,1% | 0,0% |
| Overall (Test vs Ref) | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% | -0,2% | 0,3% | 0,1% | 0,0% |
| Overall (Test vs single layer) | 28,3% | 38,9% | 39,7% | 28,5% | 38,5% | 39,6% | 25,1% | 33,2% | 36,0% |
| Overall (Ref vs single layer) | 28,3% | 38,9% | 39,7% | 28,5% | 38,6% | 39,9% | 24,8% | 33,2% | 36,0% |
| EL only (Test vs Ref) | 0,0% | 0,0% | 0,0% | -0,1% | -0,2% | -0,3% | 0,7% | 0,4% | 0,4% |
| Enc Time[%] | 99,0% |  |  | 97,0% |  |  | 98,1% |  |  |
| Dec Time[%] | 93,5% |  |  | 91,8% |  |  | 90,5% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| Optional Tests |  |  |  |  |  |  |  |  |  |
|  | LD-P HEVC 2x |  |  | LD-P HEVC ~1.75x |  |  | LD-P HEVC 1.5x |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0,0% | 0,0% | -0,1% | 0,0% | -0,3% | -0,2% |  |  |  |
| Class B | 0,0% | 0,0% | -0,1% | 0,0% | 0,0% | -0,1% | 0,3% | 0,1% | 0,1% |
| Overall (Test vs Ref) | 0,0% | 0,0% | -0,1% | 0,0% | -0,1% | -0,1% | 0,3% | 0,1% | 0,1% |
| Overall (Test vs single layer) | 26,5% | 37,9% | 38,9% | 26,6% | 37,8% | 39,3% | 23,2% | 32,9% | 35,8% |
| Overall (Ref vs single layer) | 26,5% | 37,9% | 38,9% | 26,6% | 38,0% | 39,5% | 22,8% | 32,8% | 35,6% |
| EL only (Test vs Ref) | 0,0% | 0,0% | -0,1% | -0,1% | -0,2% | -0,2% | 0,7% | 0,4% | 0,4% |
| Enc Time[%] | 99,2% |  |  | 96,1% |  |  | 98,7% |  |  |
| Dec Time[%] | 92,7% |  |  | 92,2% |  |  | 91,6% |  |  |
| BL Match | Matched |  |  | Matched |  |  | Matched |  |  |
| AVG | 0,0% | 0,0% |  | 0,0% | -0,2% |  | 0,4% | 0,1% |  |

In current specification draft text re-sampling filter with efficient phase shift 1/3 is specified to be used instead phase-shift 5/16. This was done in order to achieve compression if spatial ratio is ×1.5 (this is important use case for 720p base layer and 1080p enhancement layer). In verified contribution re-sampling with efficient phase shift 5/16 is used instead. Due to this there is 0,4% (in average) Luma BD-rate performance degradation in ×1.5 test. One might expect that specific adjustment of 5/16 filter in current specification draft text might harm performance if spatial ratio is not ×1.5. But no gain was shown by verified contribution in the test where all 16 re-sampling filters are involved (~×1.75 ratio).

# Conclusion

Samsung confirms test results reported in SCE1 for re-sampling filters set proposed by Arris. Due to 0,4% average Luma BD-rate drop in ×1.5 test and no gain in test for ratios ×2 and ~×1.75 filters set from verified contribution is not recommended for consideration.