| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0282 |
| --- | --- |

| Title: | SCE3: Verification of performance and complexity assessment for sharpening inter-layer filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina 416, Maetan 3-dong, Yeongtong-gu, Suwon, Korea, 443-742 | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution verifies performance and complexity assessment test for of sharpening inter-layer filter presented by Huawei in SCE3. Average Luma BD-rate gain 1,1%, 0,7% and 0,4% is confirmed in spatial scalability ×2, ×1.5 and SNR scalability test correspondently. Measured encoding and decoding time compare to anchor is 104% and 116% correspondently.

# Introduction

Verified algorithm was presented for the first time at JCTVC 14th meeting [1]. The goal of verified method is to minimize the distortion between reconstructed (and up-sampled if needed) base layer signal and original enhancement layer signal and so improve the quality of inter-layer texture prediction. Algorithm consists of following steps:

- Vertical gradient map calculation

  - 2 samples block extension in vertical direction

  - 3 “adds” per sample

- Horizontal gradient map calculation

  - 2 samples block extension in horizontal direction

  - 3 “adds” per sample

- Blurring filtering

  - 4 samples block extension in both vertical and horizontal direction

  - 2D separable 5 taps filter with fixed coefficients

- Gradient magnitude calculation

()

  - 3 “adds”, 5 “abs” operations, 1 multiplication, clipping with “non power of 2” value

- Displacement vector evaluation

  - 1 sample block extension in both vertical and horizontal direction

  - 2 “adds”, 2 multiplication

- Steps mentioned above are applied for Luma only. These steps are cascaded (output of one step is the input for the next), so total block extension is 7 samples (additionally to 7 sanples extension for re-sampling).

- Bi-linear interpolation in fractional position given by displacement vector

  - The range of displacement vector claimed by proponent is -2…2 (in samples). It means that block extension for this process is 4 samples (3 samples block extension is needed for Chroma re-sampling).

  - This step is applied for Chroma and Luma

If this algorithm is implemented assuming block-based inter-layer processing then additionally to 7 samples extension of Luma block (for re-sampling) 7 more samples extension is needed for the steps described below.

There is simple calculation for the memory access per one Luma sample prediction. The memory access during inter-layer texture prediction is changed as follows (estimated for the worst case which is 4x8 Luma block):

- SHM3.0 inter-layer processing: ((4+7)x(8+7))/( 4x8) = 5,2 samples need to be accessed per one Luma sample prediction

- Proposed inter-layer processing: ((4+14)x(8+14))/( 4x8) = 12,4 samples need to be accessed per one Luma sample prediction

Which is already higher than the worst HEVC case (bi-directional MC prediction in 8x8 block):

- HEVC bi-pred: ((8+17)x(8+7))/( 8x8) = 7,0 samples need to be accessed per one Luma sample prediction

Multiple steps processing introduces delay in inter-layer texture prediction.

# The summary of test results

Test were performed according to SCE3 description for both ×2 and ×1,5 spatial scalability and SNR scalability tests. All intra, random access, low-delay-B and low-delay-P tests were performed. Results we obtained perfectly match with proponent’s data. Performance is summarized in the Table 1 below.

Table 1. Performance test results summary.

| AI HEVC 2x; AI HEVC 1.5x. Y; U; V; Y; U; V. Class A; -3,1%; -4,3%; -3,8%. Class B; -1,2%; -2,0%; -2,1%; -1,0%; -2,1%; -2,2%. Overall (Test vs Ref); -1,8%; -2,7%; -2,6%; -1,0%; -2,1%; -2,2%. Enc Time[%]; 111,6%; 108,9%. Dec Time[%]; 115,8%; 113,9%. RA HEVC 2x; RA HEVC 1.5x; RA HEVC SNR. Y; U; V; Y; U; V; Y; U; V. Class A; -2,6%; -4,1%; -2,8%; -0,9%; -2,2%; -1,4%. Class B; -0,7%; -1,8%; -1,8%; -0,8%; -2,5%; -2,7%; -0,3%; -1,5%; -1,5%. Overall (Test vs Ref); -1,2%; -2,4%; -2,1%; -0,8%; -2,5%; -2,7%; -0,5%; -1,7%; -1,5%. Enc Time[%]; 109,5%; 106,4%; 103,4%. Dec Time[%]; 119,3%; 113,6%; 114,4%. LD-B HEVC 2x; LD-B HEVC 1.5x; LD-B HEVC SNR. Y; U; V; Y; U; V; Y; U; V. Class A; -2,1%; -3,6%; -2,1%; -0,9%; -2,1%; -1,4%. Class B; -0,4%; -1,4%; -1,3%; -0,6%; -2,3%; -2,5%; -0,3%; -1,3%; -1,4%. Overall (Test vs Ref); -0,9%; -2,0%; -1,5%; -0,6%; -2,3%; -2,5%; -0,5%; -1,5%; -1,4%. Enc Time[%]; 106,6%; 103,1%; 99,0%. Dec Time[%]; 117,9%; 112,4%; 113,7%. Optional Tests. LD-P HEVC 2x; LD-P HEVC 1.5x; LD-P HEVC SNR. Y; U; V; Y; U; V; Y; U; V. Class A; -1,7%; -3,2%; -2,0%; -0,4%; -1,5%; -1,1%. Class B; -0,3%; -1,1%; -1,2%; -0,4%; -2,0%; -2,1%; -0,2%; -1,2%; -1,2%. Overall (Test vs Ref); -0,7%; -1,7%; -1,4%; -0,4%; -2,0%; -2,1%; -0,2%; -1,3%; -1,2%. Enc Time[%]; 100,5%; 98,7%; 95,8%. Dec Time[%]; 120,6%; 113,3%; 117,3%. Luma; Chroma; Luma; Chroma; Luma; Chroma. Avg.; -1,1%; -2,1%; -0,7%; -2,3%; -0,4%; -1,4%. Enc; 104%. Dec; 116%. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Our analysis for the worst case memory access shows that with verified algorithm it increases by 23% (145%-->168% compare to single layer) in ×1.5 test and by 46% (200%-->246% compare to single layer) in SNR test .

# Conclusion

Samsung confirms test results reported in SCE3 for sharpening inter-layer filter. Average Luma BD-rate gain 1,1%, 0,7% and 0,4% is confirmed in spatial scalability ×2, ×1.5 and SNR scalability test correspondently.

# References

[1]M. Sychev, V. Anisimovskiy, S. Ikonin, “Inter-layer prediction modes based on base layer sharpness filter”, JCTVC-N0070, Vienna, Austria, July 2013.