| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0283 |
| --- | --- |

| Title: | RCE2: Verification of test C1 on Rice parameter initialization |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, S.Lee, J.H.Min 416, Maetan 3-dong, Yeongtong-gu, Suwon, Korea, 443-742 | Tel: Email: | +82 10 3026 1305 elena_a.alshina@samsung.com sunil.lee@samsung.com jh643.min@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Abstract

This contribution verifies performance of two algorithms presented by Qualcomm in RCE2. Both methods modify Rice parameter initialization depending on the statistics from previously decoded blocks. Verified tool provides BD-rate gain mainly for screen content. In lossy test (AhG8 content) average gain for screen content is 3.4% (method A) / 3.7% (method B) in intra and 2.3% (method A) / 2.5% (method B) in motion compensation test scenarios. Performance impact for some parts verified algorithm is also analysed in this report.

# Introduction

Materials for verification were provided by proponent in time. Code, test results and clear algorithm description for test C1 in RCE2 [1] were carefully prepared and distributed to RCE2 members before dead-line.

The basic idea is to up-date Rice parameter once per 4×4 group of transform coefficients, based on statistics collected. Statistics is collected for regular transform blocks and transform-skip blocks separately. If seperate_stats_flag is enabled then additionally Luma and Chroma are distinguished during statistics collection. Block type is derived as follows (for both methods “A” and “B”):

sbType=(seperate_stats_flag? 2*isLuma: 0)+ (isTransformSkip);

The statistics up-date is simpler in method “A”:

  - statCoeff[sbType ] += (uiLevel == statCoeff[ sbType ]) ? 0 : ( ( uiLevel<statCoeff[ sbType ] ) ? -1 : 1

compare to method “B’:

  - if (uiLevel>=3*(1<<(statCoeff [blType]/4)))

  - statCoeff [blType]++;

  - else if ((2*uiLevel)<(1<<(statCoeff [blType]/4)))statCoeff [blType]--;

The Rice parameter up-date is simpler for “B”:

  - cRiceParam = Clip3( minRicePara, maxRicePara, statCoeff [blType]/4)

compare to method “A’:

  - cRiceParam= statCoeff[sbType]>16? ((statCoeff[sbType]+1)>>2): (4+(statCoeff[sbType]+40)>>6)

  - cRiceParam = Clip3( minRicePara, maxRicePara, cRiceParam)

  - Where minRicePara = 2 if bitdepth>=12, and minRicePara = 0 otherwise.

# Brief summary of test results

Verified test results are summarised in this section. Results for the simplest modification of Rice parameter up-date from RCE2 test A [2] (from the same company) is presented for comparison. For the performance evaluation of just Maximum Rice parameter value increment in RCE2&3 anchor results published in [3] are used.

Table 1. The summary of lossy test (AhG8 content)

|  |  |  | BD-rate, all Intra |  |  | BD-rate, MC tests |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Method | Max Rice Param | Separate stat flag | Y | U | V | Y | U | V |
| A | 7 | adaptive | -3,4% | -3,4% | -3,5% | -2,3% | -2,3% | -2,3% |
| B | 7 | adaptive | -3,7% | -3,8% | -3,8% | -2,5% | -2,5% | -2,5% |
| A | 4 | adaptive | -3,4% | -3,3% | -3,4% | -2,4% | -2,4% | -2,4% |
| B | 4 | adaptive | -3,6% | -3,6% | -3,7% | -2,5% | -2,5% | -2,5% |
| A | 7 | 0 | -3,2% | -3,2% | -3,3% | -2,1% | -2,2% | -2,2% |
| B | 7 | 0 | -3,5% | -3,5% | -3,5% | -2,2% | -2,2% | -2,3% |
| RCE2 A | 4 | NA | -1,3% | -1,3% | -1,3% | -0,7% | -0,8% | -0,7% |

Table 2. The summary of lossless test (AhG8 content)

| Method | Max Rice Param | Separate stat flag | I | RA | LD |
| --- | --- | --- | --- | --- | --- |
| A | 7 | adaptive | 8,9% | 5,6% | 4,9% |
| B | 7 | adaptive | 9,3% | 5,8% | 5,1% |
| A | 4 | adaptive | 6,3% | 3,9% | 3,4% |
| B | 4 | adaptive | 6,5% | 4,0% | 3,6% |
| A | 7 | 0 | 8,6% | 5,5% | 4,8% |
| B | 7 | 0 | 9,0% | 5,7% | 5,1% |
| RCE2 A | 4 | NA | 3,1% | 1,9% | 1,6% |

Table 3. The summary of AhG 18 tests (High Precision Transform disabled)

| Method | Max Rice Par | 12 bits |  |  | 14 bits |  |  | 16 bits |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| A | 16 | -7,1% | -6,8% | -6,8% | -18,7% | -18,4% | -18,4% | -28,4% | -28,2% | -28,2% |
| B | 16 | -7,1% | -6,8% | -6,8% | -18,7% | -18,4% | -18,3% | -28,6% | -28,5% | -28,5% |
| RCE2&3 | 9 | -1,9% | -2,0% | -2,0% | -8,4% | -8,3% | -8,3% | -14,1% | -14,0% | -14,1% |
| RCE2&3 | 16 | -1,9% | -2,0% | -2,0% | -8,4% | -8,3% | -8,3% | -14,2% | -14,1% | -14,1% |
| RCE2 A | 9 | -1,8% | -1,8% | -1,8% | -10,4% | -10,7% | -10,7% | -23,6% | -23,6% | -23,7% |

Table 4. The summary of AhG 18 tests (High Precision Transform enabled).

| Method | Max Rice Par | 12 bits |  |  | 14 bits |  |  | 16 bits |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V | Y | U | V |
| A | 16 | -6,9% | -7,0% | -7,0% | -18,3% | -18,1% | -18,1% | -27,9% | -27,6% | -27,6% |
| B | 16 | -6,9% | -7,1% | -7,1% | -18,2% | -18,1% | -18,1% | -27,9% | -27,7% | -27,7% |
| RCE2&3 | 9 | -2,1% | -2,0% | -2,0% | -8,8% | -8,5% | -8,5% | -14,5% | -14,2% | -14,2% |
| RCE2&3 | 16 | -2,1% | -2,0% | -2,0% | -8,8% | -8,6% | -8,6% | -14,7% | -14,4% | -14,4% |

Following conclusions can be drawn from results reported above:

- For AhG8 content

  - In lossy test gain from verified tools is ~2,5%(MC) …3,5%(all Intra).

  - In lossless test gain from verified tools is ~5%(MC) …9%(all Intra).

  - The performance difference between verified methods A and B is 0,2…0,4% (method “B” performs better).

  - More than 1/3 of the gain reported from Rice parameter up-date can be achieved in RCE2 method A without statistics collection.

  - The performance impact of separate_stat_flag adaptation is 0,2% only.

- For AhG18 content

  - About ½ of the gain reported for 14 and 16 bits content comes from just simple modification of the maximum Rice parameter value.

  - For 12 bits content 2% from 7% gain reported comes from just simple modification of the maximum Rice parameter value.

  - It seems that maximum Rice parameter value 9 is enough to achieve mentioned gain (16 might be not needed)

# Additionally it should be noticed that in all tests slice is equal to the picture which is not necessarily true in actual applications (especially for screen content coding). So impressive gain demonstrated from Rice parameter up-date based on statistics expected to be less impressive in multiple slices per picture configuration.

# Separate statistics flag

# In verified algorithm the seperate_stats_flag was introduced. This flag is encoded in PPS. Coefficients statistics for Rice Parameter up-date is collected for regular transform and skip-transform blocks separately. If seperate_stats_flag is enabled then additionally Chroma and Luma are distinguished during statistics collection (it means that the buffer for statistics collection is doubled if this flag is 1). Encoder decision about this flag value is not clear. In the s/w provided for verification it was set differently for YUV and RGB content.

# The performance variation depending on seperate_stats_flag value is analysed in this section. In both tests listed below the seperate_stats_flag is false for anchor but true for modification. It is easy to notice that performance variation is quite small compare to whole gain provided by verified algorithm. Results for this test are very similar for both variations of verified algorithm (method “A’” and method “B”).

## Table 5. Performance impact of seperate_stats_flag (method “A”, loss-less test).

|  | AI Main |  | RA Main |  | LB Main |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Total) | Bit-rate saving (Average) |
| Class F | 0,3% | 0,4% | 0,0% | 0,2% | 0,0% | 0,1% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| SC RGB 444 | -0,2% | -0,3% | 0,0% | -0,1% | 0,0% | -0,1% |
| Animation RGB 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| SC YUV 444 | 0,7% | 0,7% | 0,2% | 0,3% | 0,2% | 0,4% |
| Animation YUV 444 | 0,1% | 0,1% | 0,0% | 0,0% | 0,0% | 0,0% |
| RangeExt | 0,4% | 0,4% | 0,1% | 0,1% | 0,1% | 0,1% |
| SC GBR 444 Optional | -0,2% | -0,1% | -0,2% | -0,1% | -0,2% | -1,2% |
| SC YUV 444 Optional | 0,7% | 0,8% | 0,2% | 0,8% | 0,0% | 0,2% |
| Enc Time[%] | 100% |  | 100% |  | 100% |  |
| Dec Time[%] | 102% |  | 104% |  | 104% |  |

## Variant A. Lossy.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0,1% | -0,1% | -0,2% | -0,1% | -0,2% | -0,3% | -0,3% | -0,4% | -0,4% |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| SC RGB 444 | 0,1% | 0,1% | 0,1% | 0,1% | 0,1% | 0,1% | 0,1% | 0,1% | 0,1% |
| Animation RGB 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| SC YUV 444 | -0,2% | -0,2% | -0,2% | -0,5% | -0,5% | -0,5% | -0,8% | -0,9% | -0,8% |
| Animation YUV 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| RangeExt | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| SC(444) GBR | 0,0% | 0,0% | 0,0% | 0,1% | 0,1% | 0,0% | 0,3% | 0,2% | 0,1% |
| SC(444) YUV | -0,3% | -0,2% | -0,3% | -0,5% | -0,5% | -0,5% | -0,7% | -0,7% | -0,7% |
| Enc Time[%] | 100% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0,0% | -0,2% | -0,1% | -0,1% | -0,2% | -0,2% |  |  |  |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |  |  |  |
| SC RGB 444 | 0,0% | 0,1% | 0,1% | 0,2% | 0,2% | 0,2% |  |  |  |
| Animation RGB 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |  |  |  |
| SC YUV 444 | -0,1% | -0,1% | 0,0% | -0,3% | -0,3% | -0,2% |  |  |  |
| Animation YUV 444 | 0,0% | 0,1% | 0,1% | 0,0% | 0,1% | 0,1% |  |  |  |
| RangeExt | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |  |  |  |
| SC(444) GBR | 0,0% | 0,0% | -0,1% | 0,0% | -0,1% | -0,2% |  |  |  |
| SC(444) YUV | -0,3% | -0,4% | -0,2% | -0,9% | -1,0% | -0,9% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0,2% | -0,3% | -0,2% | -0,2% | -0,3% | -0,2% |  |  |  |
| Class B | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | -0,1% |  |  |  |
| SC RGB 444 | 0,2% | 0,1% | 0,1% | 0,1% | 0,1% | 0,1% |  |  |  |
| Animation RGB 444 | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |  |  |  |
| SC YUV 444 | -0,2% | -0,1% | -0,3% | -0,1% | 0,0% | -0,2% |  |  |  |
| Animation YUV 444 | -0,1% | 0,0% | -0,1% | -0,1% | 0,0% | -0,1% |  |  |  |
| RangeExt | 0,0% | -0,1% | -0,1% | 0,0% | -0,1% | -0,1% |  |  |  |
| SC(444) GBR | -0,6% | -0,5% | -0,4% | -1,0% | -0,9% | -0,8% |  |  |  |
| SC(444) YUV | 0,0% | -0,3% | -0,3% | -0,3% | -0,4% | -0,4% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |  |  |  |

# Conclusion

Samsung confirms test results of RCE2 test C1 presented by Qualcomm for both variants A and B. Based on results of verified test and other experiments on Rice parameter modification following conclusion can be drown:

- Impressive performance improvement

  - ~2,5%(MC) …3,5%(all Intra) in lossy test,

  - ~5%(MC) …9%(all Intra) in lossless test

was demonstrated mainly for screen content.

- Increment of maximum Rice parameter value seems reasonable and useful for high bit-depth content

- At least 1/3 of the gain from verified tool can be achieved w/o statistics collection (RCE2 test A).

- Performance impact of statists based Rice parameter modification is expected to be lower in multiple-slice per picture configuration (which is actual use case but was not evaluated in RCE2).

- Performance impact of seperate_stats_flag coding is 0,2% only in average.

# References

[1] M. Karczewicz, L. Guo, J. Sole, R. Joshi (Qualcomm) “RCE2: Results of Test C1 on Rice Parameter Initialization”, JCTVC-O0219, Geneva, Switzerland, Oct. 2013

[2] J. Sole, R. Joshi, M. Karczewicz (Qualcomm) “RCE2 Test A1: Simplified update of the coefficient level Rice parameter”, JCTVC-O0206, Geneva, Switzerland, Oct. 2013

[3] S. Lee, J. Min, E. Alshina, C. Kim (Samsung) “Non-RCE2 and AHG18: Increase in the maximum value of Rice parameter for high bit-depth support”, JCTVC-O0129, Geneva, Switzerland, Oct. 2013